Input paper:	     ENG1-6.6.1

Input paper for the following Committee(s): 	check as appropriate		Purpose of paper:

□  ARM	X  ENG		□  PAP					□   Input

□  ENAV	□  VTS							X   Information

Agenda item			6.6

Technical Domain / Task Number	TD#2

Author(s) / Submitter(s)			Dr. Jong-Uk Kim /Korea Association of Aids to Navigation

# Developments in AtoN Simulation Systems

# - Monitoring development of simulation techniques

# for AtoN simulation in Korea(1) -

Aids to navigation(AtoN) is a important facility to give a navigation information to a ship. The previous distribution planning of AtoN was carried out by using a expert's experience and a marine map. Recently the size of ship, the vessel traffic and the complexity of harbour area are increasing. So the need for a scientific design technique of distribution planning of AtoN is increasing. The AtoN simulator provides the simulation circumstance which includes the topographical and environmental characteristics of a primary harbour and the characteristics of a navigating ship and maritime traffic. The expert of AtoN distribution planning can design a safer and more efficient distribution planning of AtoN using the simulator. To get the goal the AtoN simulator must have further function than a ship handling simulator.

Korea has developed a AtoN simulator system including the integrated system design, the construction of AtoN database, the operational software and the implementation of the integrated systems for AtoN simulation. Configuration of operation SW in the system is as follows:

- IOS SW

-  Simulation Scenario Management

-  Simulation Control

- AtoN Management SW(AtoN Manager)

-  AtoN Database Management(Properties, Placement Scenario)

-  AtoN Placement(Real-time Control of Properties and Placement)

-  AtoN Database Print and Import/Export

-  Quantification of AtoN Visibility

- Motion Solver SW

-  Motion Solver for Ship and AtoN

- RADAR SW

-  Radar Beacon Support

- ECDIS SW

-  Marine Weather Station and AtoN AIS Support

- 3D Graphic SW

-  Visualization of Terrain/Harbor, Ocean, Ship, AtoN, Environment, etc.

- Sound SW

-  Fog Signal Support

Fig. 1. Configuration of operation SW of AtoN simulator system in Korea

The main operating program and data for all sub-systems developed by KRISO(Korea Research Institute of Ships & Ocean Engineering) and KAAN(Korea Association of Aids to Navigation). Especially, AtoN Manager SW for operating to AtoN simulation was developed including AtoN database management(Properties, Placement Scenario), AtoN placement(real-time control of properties and placement), AtoN database print and import/export  and quantification of AtoN visibility .

AtoN is artificial facilities to help to sail safely by using light, shape, color, sound and radio. Especially, because AtoN provides a lot of visual information, AtoN simulator must be developed to reflect this point. AtoN Simulator has to visualize the maritime environment and AtoN realistically. This system was performed the design and implementation of a visualization system that is a sub-system of the AtoN simulator. This system reflects a variety of visualization factors related to AtoN. Also the AtoN experts can make decision and evaluate visual effects of the AtoN.

Fig. 2. Configuration of operation SW for visualization

This system is necessary to visualize the picture was carried through the 3D modeling of the AtoN such as Manned Lighthouse, Unmanned Lighthouse, Beacon, Leading Light, Projector Light, Direction Light, Light Staff, Light Buoy, Traffic Signal Light, Unlighted Beacon and Bridge Daymark etc.

| Manned Lighthouse | Unmanned Lighthouse |
| --- | --- |
| Beacon | Light Staff |
| LANBY | Spa-buoy |
| Buoy | Bridge Daymark |

Fig. 3. 3D modeling pictures of AtoN

The Visual System provides daylight, dusk and night scenes in true perspective and true colour. It shows terrain features, waterways, AtoN such as lighthouses, navigation marks and buoys etc.

AtoN Manager SW was developed as a virtual/real AtoN management for AtoN simulation.

The AtoN data are the difficulties in managing the conversion to electronic data because it is complicated large data including various properties of AtoN systems. Also, operation of  AtoN simulation system is needed for effective database management of AtoN properties. Therefore this system was designed database structural of AtoN properties based on the analysis of AtoN properties for Implementation of  visual  simulation.

Fig. 4 Schema of AtoN database in AtoN Manager SW

Fig. 5 Schema of Manned lighthouse table of AtoN database in AtoN Manager SW

Fig. 6 Picture of operation of AtoN Manager for AtoN simulation

Fig. 7 Picture of visual Implementation for AtoN simulator

Future Tasks

In future tasks, it is suggested that the AtoN simulator would be an item of an AtoN Accountability toolbox for work programme (2014-2018) and the Ministry of Ocean and Fisheries(MOF) of Korea/Korea Association of Aids to Navigation(Jong-Uk Kim) will provide information about simulation techniques(2) for AtoN simulation, the possible availability of the simulator to the IALA WWA and development of performance measures based on visibility for effective placement of AtoN to the next meeting. Also we will propose new guideline on the operation techniques for AtoN simulation.