| From: VTS Committee | VTS18/output/01 |
| --- | --- |
| To: AIS Committee - AIS14 | Date: 14-03-2003 |

Liaison Note

Draft – Frequently Asked Questions (FAQ) on AIS to assist in broadening awareness and understanding of the capabilities of AIS by the maritime community

- Introduction

The VTS Committee gave consideration to VTS18/4/9 Liaison Note Draft – Frequently Asked Questions (FAQ) on AIS to assist in broadening awareness and understanding of the capabilities of AIS by the maritime community, as requested by the AIS Committee.

- Proposal

The VTS Committee puts forward the following suggested text amendments and requests that this list of FAQs be added to the AIS page on the new IALA web site as a matter of priority as an interim measure until the list can be further considered by the AIS Committee.

- Action Requested

The AIS Committee is asked to review the amendments made.

The IALA Secretariat is asked to forward the FAQ list to AIS13 for further review.

AIS Frequently asked Questions & Answers

- Will AIS replace Radar?

No.  There always will be ships without AIS onboard.  Radar detects targets independent of the target’s onboard-equipment.

- How does the AIS supplement the Radar information?

In addition to Radar, AIS has the following benefits:

- Automatic vessel identification

- Improved vessel tracking (no track swap)

- Wider geographical coverage, although in some circumstances e.g. mountainous areas, this may only be achieved with the provision of repeater stations

- Greater positional accuracy, dependent on the position input sensor (see question 11)

- Provide information in radar shadow area

- Real time manoeuvring data

- What are some of the limitations of AIS?

AIS is like all other broadcast devices and is subject to electrical interference.  In addition, it has limited range (VHF), is dependant on the quality of the input data and cannot provide unlimited data.

[ask for more input from AIS Committee]

- How does the master know that his AIS unit is working properly?

AIS provides:

- a built-in integrity test (BIIT) running continuously or at appropriate intervals;;

- monitoring of the availability of the data

- an error detection mechanism of the transmitted data; and

- error checking of the received data.

If no sensor is installed or if the sensor (e.g. the gyro) fails to provide data, the AIS automatically transmits the "not available" data value. However, the integrity check cannot validate the accuracy of the data received by the AIS. The AIS requires that an alarm output (relay) be connected to an audible alarm device or the ship’s alarm system, if available. Alternatively, the BIIT alarm system may use the alarm messages’ output on the Presentation Interface (PI), provided its alarm system is AIS compatible.

- How does the master know that the information received from another AIS unit is accurate?

[ask for input from AIS Committee]

- Can I send a letter to my agent via AIS?

No. AIS is not intended for common messaging on the two designated AIS frequencies. AIS is mainly for monitoring other ships and allows only the transmission of safety related messages on AIS designated frequencies.

- When is a vessel permitted to sail without operational AIS equipment?

If the vessel comes under the SOLAS Regulations, SOLAS Ch.V Reg.16, the vessel is allowed to proceed to a port where repairs can be carried out.

However, if the vessel does not fall under SOLAS Regulations, then it depends on the regulations of the national maritime administrations (port state control).

- Why do AIS messages not include the rudder angle?

Rudder angle information does not give an accurate indication of the actual path of the vessel.

- How do I display AIS?

The stand-alone minimum keyboard display (MKD) is the only approved method of display.  Type approved AIS-capable radars do not exist yet.  Manufacturers are the best source of information on this issue.

Why does a vessel with ECDIS or ECS need to carry a minimum keyboard display (MKD)?

Because type-approved ECDIS/ECS with AIS capabilities do not exist yet. The requirement for Class A AIS is to be equipped with MKD.  However, some vessels display AIS on optional bridge PCs.

Will non-SOLAS vessels be required to carry a Minimum Keyboard Display (MKD) as an AIS display device?

Regulations for non-SOLAS vessels are a matter for Administrations.

Is the MKD intended as a navigation device?

No.

How will AIS be displayed on an onboard radar?

The display of AIS on Radar is not yet approved.  This issue is subject to ongoing development.

- Can I connect my GPS to the AIS?

The GPS connected to the AIS must fulfil the IMO performance standard.  It means that the output from the GPS must be in WGS 84 and that there is no dead reckoning of the position.

- Can I connect my gyro to the AIS?

The gyro connect to the AIS must fulfil the XXX performance standard.  Without heading information the presentation of the ship shape will not be available on the display of receiving stations.

[AIS Committee to elaborate on the response]

- What is the accuracy of information provided?

The accuracy of nautical information, as position, speed etc., depends on the accuracy of the sensors used.  There is an indication of the position accuracy transmitted by AIS if GPS or DGPS is used.  Further data accuracy, as voyage related data, depends on the accuracy with which this data is typed in and the frequency of update. Masters must bear in mind that the information they receive is typed in by a third party and therefore must consider this when assessing its accuracy.

- How will AIS contribute to the prevention of collisions?

AIS should not be used in isolation as an anti- collision device. It should be used in conjunction with all means available to assist the mariner. It should be borne in mind that there will always be other vessels not equipped with AIS.

- Is there an alarm to indicate that the AIS unit is not powered on?

This depends on the onboard installation.

- Under what circumstances is it allowed to switch off the AIS?

Where the master believes it is necessary for the safety or security of the crew and/or the vessel and/or the cargo, or while alongside.

[AIS to confirm wording – check to ensure this is in line with outcome of Diplomatic Conference]

- Is AIS information stored somewhere?

Ashore - this is a matter for national administrations.

Onboard - this depends on the other equipment installed.

- For what purposes could the stored or recorded data be used?

The data could be used for a variety of purposes, such as:

- accident investigation,

- traffic monitoring,

- planning and

- optimising statistics.

- Are naval ships required to fit AIS?

No.  They are outside the SOLAS Convention, but Administrations may dictate naval vessel use of AIS.  Other categories of vessels may also be exempt.

- What are the differences between AIS Class A and AIS Class B?

SOLAS vessels have a statutory obligation to carry Class A AIS equipment.

Class B AIS equipment is under development and is intended to be simpler, have fewer features, and be a less expensive version for the use on non-SOLAS vessels.

- Should the non-SOLAS maritime community wait for the development of Class B, particularly if it will cost less?

Class B is intended primarily for recreational craft.  The carriage requirement for non-SOLAS vessels may be decided by national authorities.

- Is there a standard datum for GPS, AIS and charts?

No.

AIS co-ordinates are in WGS-84, the co-ordinate system used by GPS signals. However, the on board GNSS receiver may be referenced to another datum, depending upon the navigational chart in use.

- What methods are available to prevent misuse of AIS data?

This is a matter for National Administrations to manage.

[AIS committee to elaborate on the response]

- How is AIS derived data controlled?

This is a matter for Administrations.

[AIS committee to elaborate on the response]

- Are AIS messages limited to safety information?

Yes.  ITU-R recommendation M.1371-1 limits AIS transmission on AIS designated channels to maritime safety related messages.

[AIS committee to elaborate on the response]

- Is there a limit on the scope and content of AIS messages?

Yes.  Some messages are defined, others have limited capacity for text content and are limited to safety related messages.

- Will AIS messaging grow to include commercial usage?

No, currently commercial messages cannot be transmitted on designated, safety related frequencies, e.g. AIS1 and AIS2.

[AIS committee to elaborate on the response – please distinguish between messages]

- How do we detect/prevent transmission of false messages?

This is a matter for national administrations to manage

[ask for more input from AIS Committee]

- Should AIS data be integrated on the same display as other sensors information in a VTS?

Ideally, yes. All sensor information should be integrated on a single display.

- Can I get DGNSS corrections over the AIS link?

Yes, if the Administration decides to do so, the message structure and technical approach are defined.

- How is meteorological/hydrological information displayed?

There are prototype ways to display this information. A message structure is being proposed to IMO.

- Is there any calibration (regular/frequent/interval/initial) of the AIS equipment required?

No.

[AIS committee to elaborate on the response]

- Is there any set up (regular/frequent/interval/initial) of the AIS equipment required?

Yes.  Static and voyage-related data must be entered and should be checked and updated at appropriate intervals.

- Is AIS mandatory for inland waterways?

No.  This is a matter for Administrations.

- Should I use Type Approved equipment?

Ships required to carry Class A AIS must use type approved equipment. .

- Is there any training on the use of AIS available?

The manufacturers may well give initial training on their equipment.