VTS 18 / 4 / 6

DRAFT
Display of AIS Information at a VTS Centre [Integration and Display of Information at a VTS Centre?]

# Introduction

Chapter V of the 1974 SOLAS Convention (as amended) requires mandatory carriage of Automatic Identification Systems (AIS) equipment on all vessels constructed on or after 01 July 2002.  Implementation for other types and sizes of SOALS Convention vessels is continuing and will be completed not later than 31 December 2004 (note, as amended at the December 2002 Diplomatic Conference [ insert full name for conference]

SOALS Chapter V, Regulation 19, section 2.4.5 states that:

“AIS Shall:

.1	provide automatically to appropriate equipped shore stations, other ships and aircraft information, including ship’s identity, type, position, course, speed, navigational status and other safety-related information;

.2	receive automatically such information from similarly fitted ships;

.3	monitor and track ships; and

.4	exchange data with shore-based facilities.”

In addition, the IMO Performance Standards for AIS state:

1.2 The AIS should improve the safety of navigation by assisting in the efficient navigation of ships, protection of the environment and operation of Vessel Traffic Services (VTS), by satisfying the following functional requirements:

.1 in a ship-to-ship mode for collision avoidance;

.2	as a means for littoral States to obtain information about a ship and its cargo; and

.3	as a VTS tool, i.e. ship-to-shore (traffic management)

IALA Recommendation A-123 on the Considerations relating to the provision of shore based AIS notes that the provision of shore based AIS will be necessary to attain the full benefit of the 1974 SOLAS Convention (as amended).  National Members and other appropriate authorities should consider the provision of AIS shore infrastructure in existing or planned VTS systems so that the full benefit of the system can be realised in terms of navigation safety and protection of the environment.

# Background

AIS is an autonomous and continuous broadcast system, operating in the VHF maritime mobile band.  […..]

# Benefits

[include as appropriate – the benefit of adhering to the information contained in the document]

# Operations

Note:  The VTS operator should continue to communicate via voice VHF.  The importance of verbal communication should not be underestimated.  This is important for the VTS operator to:

- Assess vessel’ communicative ability; and

- Establish a direct communication link which would be needed in critical situations.

Caution

Not all ships carry AIS

The Vessel Traffic Service (VTS) Operator should always be aware that ships and, in particular, leisure craft, fishing boats warships, and some coastal shore stations including other VTS centres, might not be fitted with AIS.

The VTS Operator should always be aware that AIS fitted on ships as a mandatory carriage requirement, might, under certain circumstances, be switched off on the master’s professional judgement.

Modified from: IMO resolution A.917 (22) AIS

It is recommended that IALA look at IMO Resolution A.917 (22) AIS as a guide to a similar document drafted specifically for VTS AIS systems.

AIS, by itself, may not provide a reliable picture of the actual traffic image as the majority of shipping may be non-compulsory, or have disabled their AIS system.  In developing a traffic image and maintaining situational awareness, the VTS operator must consider the limitations of AIS when used without other navigational aids.

Communications in a VTS system may be substantially reduced to participating vessels using AIS.  Once identified, such vessels may be exempt from further communications with a VTS, for example, passing established “reporting points”, “lost target” or “target swaps”.  This may provide a relatively “Silent” VTS.  However, voice communications between the bridge team and the VTS providing an “information service”,  “traffic organization service” or a “navigational assistance service” will continue as recommended by the IALA guidelines.

Space allocations for AIS data should be reserved for ship to ship or ship to VTS whenever possible. Transmission of data to specific vessels from a VTS using AIS should be restricted to short, safety messages requiring acknowledgement.

## AIS Symbols

A VTS Integrated display using Radar and/or AIS should have the ability to default to a pre-set standard using Radar and AIS information as new targets are acquired. The VTS operator should be able to select AIS or Radar targets separately, or select a display using no symbology other than the radar target in areas of extreme congestion.

A radar or radar/AIS symbol should interpolate the data, showing the relative size of different vessels.

VTS operators  need to know by what equipment this information is coming from (AIS, RADAR, RDF)

## On-screen information

On screen information should never interfere with vessel targets displayed.

With the amount of AIS information that may be available, there is a risk of information overload which could add to the workload of the VTS operator.  The VTS authority should have the ability to determine what information is important to the type of service offered by a particular VTS.  The VTS operator(s) should have the option to determine what information is important to provide this service during times of varying degrees of workload.

An example to minimize the potential for information overload would be to move mouse cursor over a target, with the information concerning that target displayed.  The information will be displayed as long as the cursor rests over a particular target.  To keep this information on screen, click once. To remove it, click again on any target.

Information may be missed by VTS due to specific ‘on screen’ information being filtered out as vessels update or add new information without advising VTS by other means.

## Equipment

AIS information required by a VTS would be determined by the competent authority.  VTS operators, however, may require less information to maintain proper traffic management.  The AIS software must be sufficiently flexible to allow VTS operators to select whatever information is needed for their particular sectors or operational console.

As an interim measure, a stand alone AIS system may be necessary until integration of the AIS can be programmed into the existing VTS system.  To permit a step by step installation, while VTS personnel use both RADAR and AIS systems, the stand alone system must be adjacent to the VTS operator’s existing display.

In case of power or equipment failures the VTS system should have a built in redundancy to maintain a display of AIS targets should RADAR be lost.

## Third Party Liability

It is understood that AIS information received or rebroadcast through VTS systems is automatic in nature and not normally edited by VTS personal.

[Who is responsible?]

- When passing information from vessels via VTS to other users, when information is misleading or incorrect.  For example, a vessel loaded with gasoline, claiming dangerous cargo of kerosene in all contact with a VTS

- re-broadcasting incorrect AIS information using VTS systems.

[It was pointed out that regardless of the information transmitted by voice, data or any other means, the ultimate responsibility to accept and act upon this advice, or not, rests with the Master of the vessel.

Note:  See item 34), 38), 40.1), 40.2) 41) and 42) IMO Resolution A.917 (22) AIS

## [Human Factors

A VTS operator must be aware that direct voice communications with AIS equipped vessels may be decreased or non-existent in some cases.  This “silent” system will not remove the responsibility of advising all vessels of conflicting traffic as it occurs.  It will be vital that the training institutes instruct VTS personal of this limitation.

Though it is possible VTS workload may be reduced as a result of AIS, it must be stressed that advisories will continue to be required for all non-AIS equipped vessels.   This reduction may be experienced at some VTS Centres as the Operator will not have to advise AIS fitted vessels of who is ahead or astern of them, for example.  This information will be available to all AIS equipped vessels who could be in direct contact with each other as the need arises.

The inclusion of AIS as an operational tool may necessitate a major change in how existing VTS personnel operate.  This may be particularly apparent when moving from actively participating in traffic advisory’s to that of passively observing AIS equipped vessels communicating directly with each other without VTS intervention.  The VTS operator, however, must be alert to the fact that although this “silent” VTS system has developed as a result of AIS, the obligation of a VTS, to advise of traffic within their area of responsibility has not been relinquished. A “silent” VTS could become a risk, without a skilled VTS operator able to provide pertinent traffic information to all mariners within his AOR.

This being the case, all vessels, AIS equipped or not, must continue to be advised of pertinent traffic.  It must be stressed that AIS is primarily a tracking system and not a primary method for message delivery.  It will be important for competent authorities to ensure a step by step implementation of AIS at VTS Centres is conducted to allow bridge teams and VTS Operators sufficient time to adapt safely to the efficient use of AIS as an operational tool.

Using AIS as an operational tool, may be a problem as VTS operators looking at the entire VTS image may assume vessels see the same thing.  In reality, however, vessels may see much less due the limitations of their specific AIS installation.  This situation may require additional VTS voice advisories for any traffic close to his position.  It is important that VTS personnel understand that what the ship sees is not necessarily what the VTS sees.  Ships may have a very narrow view of the overall traffic image.  VTS centres without radar coverage may also have an incomplete view of an overall traffic image.

An inherent “human” problem may be experienced when a VTS begins passively observing traffic.  This can become more stressful by the lack of direct involvement by the VTS Operator using voice communications.  As the workload for bridge teams may decrease, the stress experienced by VTS operators may increase due to their lack of direct communications with traffic within their AOR.   An important advisory may be missed due to this problem of attention deficit.

A VTS operator may give navigational assistance using RADAR information at the moment, however, in certain areas, a competent authority may decide to use AIS information instead.  Changes to existing operating procedures could be easily amended to reflect these changes and made available to local users of the system.]

Comment:  Regardless of electronic aids to navigation, it is the master’s responsibility to maintain a good lookout for proper navigation of his vessel.

## Skills

The topic of ‘skills’ was discussed at length with the conclusion that IALA v103 and associated model courses sufficiently cover this area at present.