From

Marten Koopmans

Date

16 August 2004

Telephone

+31 70 351 1562

Enclosure(s)

1

Subject

To

Adres

VTS Manual – Chapter 4

Dear VTS Committee colleagues,

At the last meeting of the VTS Committee I was tasked with the coordination of the IALA VTS Manual sub-group drafting Chapters 4, 6 and 7. Under the revised work plan for the 2008 edition of this Manual the draft of Chapter 4 has to be finalised first for circulation at the next VTS 21 session in September.

Please find attached my first draft for Chapter 4 for review and possible finalisation by the VTS Committee at this coming session.

Due to time constraints and my own busy work programme I was only able to write a first draft now and was therefore unable to discuss this draft with the sub-group in time for the next meeting for which I offer my apologies.

I assume this draft will be discussed and amended first in (a drafting sub-group of) WG1, but I would appreciate suggestions for improvement beforehand, e.g. at the initial plenary session of VTS21.

The Chapter is relatively short as I purposely limited this Chapter to only describing VTS functions without addressing related aspects, which will be addresses in other Chapters.

Yours sincerely,

Marten Koopmans

CHAPTER 4

The Functions of VTS

# 4.1.  Introduction

As stated in the IMO Guidelines on VTS (IMO Resolution A.857(20)) the purpose of VTS is:

“to improve the safety and efficiency of navigation, safety of life at sea and the protection of the marine environment and/or the adjacent shore area, worksites and offshore installations from possible adverse effects of maritime traffic”

To fulfil this overall purpose VTS has to carry out several functions, some of which have emerged recently. The most important functions are related to ensuring or improving:

- Safety of the vessel traffic

- Efficiency of the vessel traffic

- Protection of the environment

- Protection of the adjacent communities and infrastructure

- Efficiency of related activities

- Security in the maritime domain

These will be discussed in more detail in the following paragraphs.

4.2.  Safety of the vessel traffic

Incidents involving vessels can lead to loss of life, injuries and damage. VTS contributes to the prevention or reduction of such incidents resulting from vessel traffic. It also assists in minimising the consequences from this type or other types of incidents.

VTS can:

- prevent incidents from developing

- prevent incidents from developing into accidents

- prevent accidents from developing into disasters

- cure the consequences of incidents, accidents and disasters

Unlike other aids to navigation VTS has the capability interact and influence the decision making process on board the vessel. VTS can detect the development of close quarter situations and vessels standing into danger and can then alert such vessels accordingly and in some cases instructing them to take certain avoiding action. As approximately 80-90% of maritime accidents can be attributed to human error the improvement that can be gained through the involvement of and interaction with other humans through the VTS as an additional safeguard can easily be seen.

Although the safety of vessel traffic, in particular the reduction of loss of life of seafarers, should be the primary reason for implementing VTS the other mentioned VTS-functions can often be a more persuasive arguments as the effect of VTS on the expected (or even actual) number or size of vessel traffic accidents and casualties is difficult to determine.

The best way, but unfortunately not the easiest way, to be able to assess the effect of VTS on safety of vessel traffic is by determining the risk reduction, which is or can be achieved by VTS. Fortunately methods have been developed for this. These methods will be further discussed in Chapters 6 and 7 of this Manual.

Consideration, implementation and even operation of VTS is in essence a risk management activity, trying to reach an acceptable risk level at acceptable costs and efforts. In the actual VTS-operations, e.g. determining whether a vessel passage through the VTS-area is safe enough, the above-mentioned methods can also be useful, but this still remains a mostly skill- and experience-based activity.

More work needs to be done to develop clear operational procedures, which are properly based on a scientific risk-analysis approach and which can then be applied consistently.

If an accident has unavoidably occurred VTS can be used to assist and support calamity abatement operations. In the context of safety of vessel traffic safety these are in particular Search and Rescue (SAR), fire fighting and salvage operations. However for the other mentioned functions other activities, such as pollution combating and rebuilding/restoring, can be also supported. In some VTSs such operations are even carried out under the supervision of the VTS itself.

4.3.  Efficiency of the vessel traffic

VTS can improve the efficiency of vessel traffic in two ways, i.e. through:

- minimizing accidents

- maximizing the capacity of the infrastructure

Prevention of an accident directly leads to an improvement, i.e. preventing a deterioration, of the efficiency of vessel traffic. An accident causes delays for the vessels involved and for vessels in the vicinity. With serious accidents there could be hold-ups and rerouting of traffic for a lengthy time.

Infrastructure (waterways, locks, ports etc.) has a certain maximum capacity, both in the size of and the number of the vessels that the infrastructure can accommodate. When this maximum capacity is being reached a VTS can safely increase this maximum capacity, e.g. by enabling:

- larger vessels to use the infrastructure (e.g. larger draft, width, length, height)

- longer use of the infrastructure (e.g. tidal windows, continued operation under detrimental conditions)

- more use of the infrastructure (e.g. higher traffic density, higher speed)

The resulting improvement for the vessels concerned in carrying capacity and reduction in delays increases the efficiency of these vessels. At the same time this increases the capacity of the infrastructure, which may eliminate, delays or reduce the need for costly investments in the expansion of this infrastructure.

These benefits are more directly noticeable to parties concerned and are easier to determine then the benefits of VTS for safety of vessel traffic.

  - Protection of the environment

In many societies, communities and areas the protection of the environment is nowadays considered of the highest importance. Pollution can also cause substantial economic damage to activities, in particular those activities dependant on a clean environment, such as tourism, recreation and fishery. Generally spills, in particular oil spills, by tankers are the biggest concern, but accidental bunker spills by other vessels and spills of chemical cargoes into the water, but also accidental emissions of polluting gasses into the air can also cause environmental pollution.

Although there are no known VTSs implemented solely for protection of the environment it is often a substantial driving force for the need for VTS. It has resulted in VTSs being implemented in areas with low traffic volumes, densities and other traffic complications where therefore solely the need for safe vessel traffic did not sufficiently justify VTS, in particular in areas where relatively high quantities of polluting cargoes are transported and/or where areas that are considered to be environmentally sensitive are nearby.

Apart from preventative actions to avoid incidents occurring and curative actions to avoid incidents, when they have occurred, developing into accidents or even disasters, VTS can also support and contribute to the identification of sources of illegal pollution. With the information available in the VTS on vessel movements in the VTS area sources of pollution in or nearby the VTS area can be better and more easily identified and proven. It should also be noted that the very fact that a vessel knows that it is being monitored by VTS helps to deter from intentionally discharging any pollutants.

As environmentally sensitive areas are often outside port areas at sea it might sometimes be desirable to have VTS extend into international waters or straits for the protection of these sea areas. It should be realised that VTS-participation by vessels is only voluntary and cannot be made mandatory in those waters until IMO has made this possible. Although there are no concrete proposals for this at IMO the expectation is this will be discussed and possibly so decided within IMO in a few years.

As with the safety of vessel traffic it is not easy to assess the effect of VTS on protection of the environment as not only the impact of VTS on the size and number of accidents is difficult to determine, but also the impact of VTS on the reduction of the pollution which could possibly result from such accidents. This requires a proper and thorough risk analysis, which needs availability and access to a lot of data on traffic, circumstances and environmental sensitivity.

4.5.  Protection of the adjacent communities and infrastructure

In particular in ports, narrow straits and inland waterways vessels sail in close proximity to populated areas and associated infrastructure. In ports there is also the industrial activities with its associated infrastructure. A vessel traffic accident can cause loss of life, injuries and serious damage there. Generally accidents involving spills or emissions of hazardous chemical fluids or gasses are the biggest concern, but deaths, injuries and damages can be caused by vessels sailing into shopping centres, restaurants, housing blocks etc. on the waterfront, as has happened in the past. Also the additional impact of a chain reaction in oil or chemical plants on the waterfront initiated by an accident with a vessel needs to be considered.

Again a VTS can prevent such accidents occurring and developing into disasters. VTSs are also being used as calamity abatement centres by the policy, fire brigades, ambulance services etc. in case of such calamity happening on the waterfront as much of the necessary coordinating facilities and information are available there.

As with the safety of vessel traffic and protection of the environment it is not easy to assess the effect of VTS on protection of the adjacent communities and infrastructure as not only the impact of VTS on the size and number of accidents is difficult to determine, but also the impact of VTS on the reduction of the safety of the adjacent communities and infrastructure which could possibly result from such accidents. This requires a proper and thorough risk analysis, which needs availability and access to a lot of data on traffic, circumstances and sensitivity of the adjacent communities and infrastructure.

4.6.  Efficiency of related activities

In ports there are many activities related to shipping, such as:

- pilotage

- towage

- line handling

- bunkering

- chandelling

- repairs

- inspections

- cargo transfer

- cargo treatment

- cargo processing

- cargo (onward) transport

All these services need early, correct and speedy information on real time and expected vessel positions, movements, destinations with ETAs etc. This enables those services to offer better services, whilst at the same time to better plan and utilise their resources, which will reduce the cost base and thus their fees for these improved services.

Every self-respecting port therefore sees improvement in information gathering and dissemination as a means to offer a better port service to the shipping community and so to obtain a competitive advantage over other ports.

VTS has a significant amount of this desired information. In this respect the contribution that can be offered by supplying this information to businesses involved in cargo transfer and onward transport (by road, rail, inland waters and sea) is gaining importance. It improves the optimalisation of the overall transport of logistical chain of intermodal transport from producer to consumer. The transfer of information concerning cargo position, movement, destination and ETA is part of the interconnectivity within this chain, which is essential to improve intermodal transport. When the cargo is still on board the information concerning the whereabouts and intentions of involved vessel is in part an acceptable substitute for the desired cargo information.

Making this information available in the VTS available to others offers direct benefits to the port and transport community. This could be a big additional driving force for implementing VTS, in particular for port authorities trying to improve the competitive position of their port.

However special attention needs to be paid to which information can be made available. There are legal restrictions and societal sensitivities with regard to protection of privacy and commercially secret or valuable information. Recently a concern of misuse of this information by terrorists and vandals has been added to this.

  - Security in the maritime domain

As a result of 9/11 security has become a main concern and priority, very much also in the maritime community. Together with aviation maritime transport is now one of the frontrunners in improving security of transport as those two forms of transport can enable terrorist to do the gravest damage to society.

Protection against terrorist action in the maritime domain requires, among others, a complete image of vessel traffic in areas of concern with information on the intentions and cargoes of those vessels as well as vigilant monitoring of this vessel traffic. This could also be of use as safeguards against smuggling of goods and illegal immigrants by sea.

VTS has an image of the vessel traffic in the VTS area and partly of the adjacent areas. VTS has also VTS-operators monitoring this traffic real-time and skilled to do so. However VTS does not have the facilities to see all traffic, in particular small craft. VTS-operators are not specifically trained to identify potential security threats nor are they qualified and equipped to deal with those security threats.

VTS can contribute to improving security. This could be an additional driving force for implementing VTS. However without additional powers, facilities and training VTS can only offer some monitoring and coordination facilities to the security authorities.