Guideline

G1117

VHF Data Exchange System (VDES) Overview

Preliminary Draft Revision of Edition 2.0

March 16th, 2022, JWG IALA eNAV WG3/IEC TC80/WG15

Revisions to this IALA document are to be noted in the table prior to the issue of a revised document.

| Date | Details | Approval |
| --- | --- | --- |
| December 2016 | 1st issue | Council 63 |
| December 2017 | Edition 2 Updated to reflect technical developments in VDES and to reflect transmission aspects (addressed / broadcast) | Council 65 |

Revision by  working group

| Date | Details |  |
| --- | --- | --- |
| 28 February 2022- 3 March 2022 | JWG IALA/IEC Intersessional workshop, multiple authors; the work of all contributions at the workshop is included in this version (also from Pieter Winter, but as edits of Stefan Pielmeier due to technical reasons, see ANNEX). | N/A |
| March 3rd-16th | Homework according to action list from JWG in above line | eNAV29 |

List of Tables

Table 1	Maritime Service, IMO update 1 - e-Navigation Strategic Implementation Plan	7

Table 2	Considerations for VDES Implementation	14

Table 3	Comparison of Digital Communications Technologies	16

Table 4	Potential VDES Uses cross-referenced to IMO SIP MS update 1	17

Table 5	Frequencies allocated to VDES	30

Table 6	VDE link layer messages	34

List of Figures

Figure 1	Implementation of VDES	5

Figure 2	VDES functions and frequency use – full system	7

Figure 3	Concept for VDES	9

Figure 4	Concept for VDES with Martime Connectivity Platform [WorkInProgress]	10

Figure 5	VDES implementation decision matrix	13

Figure 6	Increasing data bandwidth in VDES	30

Figure 7	VDES ship station logical description	32

Figure 8	VDES shore station logical description	32

Figure 9	VDES into the Common Shore Side Architecture	33

Figure 10	Ship control station	34

# INTRODUCTION

## Purpose of the document

This Guideline introduces the VHF Data Exchange System (VDES) at an overview level. While it does not include technical details, it refers to documents providing the necessary details for implementation.

This document is intended to assist in the understanding, development and promotion of VDES.

## Background

<<Add revision of appendix 18 in 2019>>

AIS is well recognized and accepted as an important tool for safety of navigation and is a carriage requirement for SOLAS vessels (Class-A). With increasing demand for maritime VHF data communications, AIS has become heavily used for maritime safety, maritime situational awareness and port security.  As a result, overloading of AIS 1 and AIS 2 created a need for additional AIS channels. Using the VHF marine band (International Radio Regulations Appendix 18) AIS can broadcast data to vessels in the vicinity of the AIS unit. AIS can also transmit an addressed message.

International Telecommunications Union (ITU) has recognised the efficiency and the necessity for digital communications at sea. ITU therefore has produced technical standards and revised the VHF marine band (Radio Regulations Appendix 18) to designate channels for data transmission.  It is recognized that both analogue voice communications and digital communications will share the band. The VDES, as envisioned by IALA and presented to ITU, addresses the identified need to protect AIS along with essential digital communications contributions for e-Navigation and GMDSS Modernization.

The VHF marine band (Radio Regulations Appendix 18) was initially used for transmission of voice communications on 25 kHz channels. The ITU introduced the first marine data transmission system, DSC (Digital Selective Calling) to help ensure that calling and distress communications attempts were successful. VHF DSC transmits data at 1.2 kbps, slow by modern data standards, but very robust.  At the request of the IMO to improve safety of navigation, ITU introduced another VHF data transmission system, AIS, which provides navigation and identification data for ships, shore stations, aids to navigation and search and rescue devices at 9.6 kbps.

ITU introduced a standard, with options for 25 kHz, 50 kHz and 100 kHz channels at data rates up to 307.2 kbps in order to improve spectrum efficiency in 2012.  Both voice and data communications coexist in the VHF marine band.  The developments in maritime radio technology, including the introduction of software defined radios (SDR) coupled with enhanced capabilities for digital data exchange over existing VHF marine band spectrum resulted in the development of the VHF Data Exchange System (VDES).  VDES builds on the experience gained through the development of AIS, and provides the capability to communicate to

- a specific vessel (addressed) ,

- all units in the vicinity (broadcast),

- a group of vessels (addressed, and

- a fleet of vessels (addressed).

Consequential to WRC 19, Recommendation ITU-R M.2092-1 was published in February 2022.

2020

2021

2018

2019

2016

2017

2022

AIS + VDE Terrestrial Initial Operational Capability

VDES Terrestrial Initial Operational Capability

VDES Full Operational Capability

AIS

AIS 1
AIS 2

VDE-SAT uplink/downlink

VDE-TER

VDE-TER + ASM (incl. uplink)

VDE-TER; VDE-SAT + ASM

VDE-TER

ASM 1
ASM 2

International availability of VDES spectrum (as ITU Radio Regulations App.18)

The introduction of VDES is expected to happen through following phases:

(2016) AIS exists as defined by ITU.R M.1371-5 on the AIS frequencies, and Coastal Stations use the ASM and VDE frequencies for Voice VHF.

(2017-2018) Post WRC-15 - AIS+ASM: Regionally, where there is an urgent need for offloading the AIS VDL from significant ASM traffic, it is recommended to allow the introduction of 4-channel AIS + ASM devices. These devices may receive and transmit ASM on the ASM1 and ASM2 frequencies, but shall discontinue their transmit capability, using the existing GMSK modulation after January 1st 2019 unless a software upgrade enables them to participate in the modulation and access scheme agreed for the ASM frequencies. Note that the ASM frequencies will need to be shared with the VHF voice service from Coast Stations in many areas during this time frame.

(2019) at the first of January 2019 the VDES VHF channels should only be used digital according to ITU. This led to operational problems and IMO postponed (MSC.1/Circ.1460/Rev.2 ) the use until the 1st of January 2024. ITU decided at WRC-19 that the VDE frequencies are also available for VDE-SAT.

(2019-2020) Post WRC-19 operational capability established on global basis. Note that Administrations may need to coordinate their internal use of the frequencies in the transition.

(2021) When a satellite service is developed, full operational capability of the VDES including the Satellite component can be achieved.

(2022) ITU has published ITU-R M.2092-1 incorporating changes to add VDES Satellite functionality and updates consequential to feasibility tests.

(2024) All the VDES VHF channels shall only be used for the VDES (MSC.1/Circ.1460/Rev.2) globally

(2030) Administrations have until 2030 (footnote w) of Appendix 18 of the RR) to completely vacate the VDES VHF channels from voice communications.

## Overview

The VHF Data Exchange System (VDES) is seen as an effective and efficient use of radio spectrum, building on the capabilities of AIS and addressing the increasing requirements for data through the system.  New techniques providing 32 times the raw data rates than those used for AIS are a core element of VDES. Furthermore, the VDES network protocol is optimized for data communication so that each VDES message is transmitted with a high confidence of reception. VDES increases the capability for digital data exchange in a manner similar to AIS, which includes provision of data to vessels in a geographic area (broadcast), to a specific vessel or a group of vessels in a geographic area (addressed) or to a fleet of vessels (addressed).

In this document, when communications from ship to shore are referenced, this includes ship to satellite to shore and shore to satellite to ship.  It is noted that, following WRC-19, the full satellite capability of VDES is available. IMO MSC.1/Circ.1460/Rev.2 advises that the operational use of ASM and VDE frequencies is globally possible from the 1st of January 2024 .

For simplicity, this document uses the term ship station, to refer to stations that are mobile and are not control stations. In contrast, control stations are stations coordinating the traffic of ship stations. Control stations can be coastal control stations, or satellite control stations, but follow --- largely and for the sake of this document --- the same basic concepts.

# GENERAL DESCRIPTION

The VDES should improve the safety of life at sea, the safety and efficiency of navigation, and the protection of marine environment and enhance maritime safety and security.  These goals will be achieved through efficient and effective use of maritime radiocommunications, incorporating the following functional requirements:

- As a means of AIS.

As a means of radiocommunications equipment through exchange of digital data between ship and ship, ship and shore including satellite, via AIS, Application Specific Messages (ASM) and VHF Data Exchange (VDE).

As a means of applications external to the VDES equipment itself.  These applications use AIS, ASM or VDE separately or combined.

## System concept

The VDES concept was originally proposed to address emerging indications of overload of the VHF Data Link (VDL) of AIS and simultaneously enable a wider seamless data exchange for e-navigation, potentially supporting the modernization of GMDSS.  VDES could support the increasing communications requirements identified through the development of e-Navigation, as documented in the e-Navigation Strategic Implementation Plan (SIP) IMO MSC.1/Circ.1595.

The purpose of e-Navigation is to enhance berth-to-berth navigation and related services for safety and security at sea and protection of the marine environment.  e-Navigation seeks to enhance maritime safety through simplification and harmonization of information.  In addition, e-Navigation seeks to facilitate and increase efficiency of maritime trade and transport by improved information exchange.

The VDES system concept recognizes the parallel work being carried out related to Maritime Service in the context of e-Navigation formerly known as Maritime Service Portifolio (MSP).  Where applicable, these MS are referenced  in this document as user requirements.

Table 1 identifies the Maritime Services in the context of e-Navigation  as defined by IMO e-navigation strategic implementation plan (SIP) and their updated E-Navigation Strategy Implementation Plan - Update 1 (IMO MSC.1/Circ.1595).

| Service No | Service | Domain coordinating body | Identified responsible service provider |
| --- | --- | --- | --- |
| MS 1 | VTS Information Service (INS); | IALA | VTS Authority |
| MS 2 | Navigation Assistance Service (NAS) | IALA | VTS Authority |
| MS 3 | Traffic Organization Service (TOS) | IALA | VTS Authority |
| MS 4 | Local Port Service (LPS) | IHMA | Local Port/Harbour Authority |
| MS 5 | Maritime Safety Information (MSI) service | IHO | National Competent Authority |
| MS 6 | Pilotage service | IMPA | Pilotage Authority/Pilot Organization |
| MS 7 | Tugs service | TBD | Tug Authority |
| MS 8 | Vessel Shore Reporting | TBD | National Competent Authority and appointed service providers |
| MS 9 | Telemedical Maritime Assistance Service (TMAS) | TBD | National Health Organization/dedicated health Organization |
| MS 10 | Maritime Assistance Service (MAS) | TBD | Coastal/Port Authority/Organization |
| MS 11 | Nautical Chart Service | IHO | National Hydrographic Authority/ Organization |
| MS 12 | Nautical Publications Service | IHO | National Hydrographic Authority/ Organization |
| MS 13 | Ice Navigation Service | WMO | National Competent Authority/Organization |
| MS 14 | Meteorological Information Service | WMO | National Meteorological Authority/Public Institutions |
| MS 15 | Real-time hydrographic and environmental information Services | IHO | National Hydrographic and Meteorological Authorities |
| MS 16 | Search and Rescue Service | TBD | SAR Authorities |

Maritime Services, IMO update 1 - e-Navigation Strategic Implementation Plan

The system concept, including VDES functions and frequency usage is illustrated pictorially in Figure 2 (full system).

VDES functions and frequency use – full system

## Concept of operations

The key concept of operation of the VDES includes:

- The VDES provides a capability of data exchange between ships and shore users by terrestrial or satellite link.

- The operation of VDES is subject to control stations but it may operate autonomously without a shore infrastructure.

- Data exchange from the ship may occur automatically or manually.

- Data exchange uses the designated VHF channel(s).

- Transmission and reception of the data occurs with the minimum involvement of ship’s personnel.

- The VDES includes existing AIS applications.

- The VDES includes existing ASM.

- VDES additional capabilities include support of the VHF Data Exchange (VDE).

- The VDES related applications should support language independent communications (e.g. through the use of a digital data dictionaries).

- The VDES implements data integrity monitoring at the VDES link level (e.g. check sum).

- The VDES related applications address cyber security (e.g. authentication, key management and, if required, encryption).

- The VDES has a high level of availability.

- The VDES supports machine-to-machine communications (for example, interfaces with external equipment providing applications related to VDES).

- The VDES related applications enable clear comprehension of the information sent / received through the VDES.

The concept of operations is identified in Figure 3.

Concept for VDES

### Architecture using Message Content Identifier

In order to use VDE terrestrial or satellite messages, and being able to identify the content, following methods exist:

- Identification of the content format and encoding by use of the VDES Source Station ID, i.e. “MSI Denmark Service S-100” uses a a-priory known encoding announced by the service authority,

- Identification of the content format by using a 16-bit application identifier  as defined by AIS for all VDE binary message content to clearly identify its format and encoding.

- b)c)	Identification of the content format using a 40 bit VDE Application Identifier as described below in section 2.2.1.1.

Method (c) is the preferred method because:

- is the most flexible and makes it possible by one VDES Station to use multiple formats from the same Source Station ID.

- allows ships directly to exchange content of different formats and encodings, clearly identified by the VDE Application Identifier.

- allows the transport of existing ASM formats and encodings, as defined by IMO and national authorities.

Therefore, this document assumes, that VDE-TER and VDE-SAT binary messages will use the 40 bit VDE Application Identifier in all applications. It should be noted that binary messages transmitted on the ASM channels will continue to use the 16 bit application identifier as defined by AIS.

#### VDE Application Identifier Description

In order to allow identification of the data formats by the applications, equipment supporting VDE-TER and VDE-SAT shall support the VDE Application Identifier (VAI) preceding all encapsulated multi sentence messages at PI input or PI output.

VAI consists of:

- 16 bit Registry Holder Identifier;

- 0 for test;

- 1-9 for international applications;

- 10-65565: other;

- 16 bit VDE Function Identifier VFI, administered by the Registry Holder;

- 8 bit function version number identifying the format of the application data; starting with 0, and incremented up to 255; thereafter counting starts again with 0;

- application data.

The VAI builds a unique identification for the format of the encapsulated data content.

The Registry Holder is organizing the use of the 16 bit VDE Function identifier.

#### Definition of function messages

Each unique combination of VDE Application Identifier (VAI) and application data forms a functional message. The coding and decoding of the application data of a VDE functional message is based on an algorithm identified by the VAI value.

The International VAI (IVAI) should be maintained and published by the international authority registry owner responsible for defining International Function Messages (IFM), as for AIS.

Non-international or regional VAI (RVAI) should be maintained and published by the registry owner.

Table 4 identifies IVAI designed to provide support for mandatory VDE system applications that are defined in this document.

Table 4 – International VDES Application Identifiers (IVAI)

| VDE Application identifier VAI (decimal) |  |  | Description |
| --- | --- | --- | --- |
| Registry Holder Identifier |  | VFI |  |
| 001 |  | 00 | IVFM 0 = Text telegram 6-bit ASCII |
| 001 |  | 01 | IVFM 1 = Signed Text telegram UTF-8 |
| 001 |  | 02 | IVFM 2 = Encrypted Text telegram UTF-8 |
| 001 |  | 03 | IVFM 3 = Capability interrogation |
| 001 |  | 04 | IVFM 4 = Capability interrogation reply |
| 001 |  | 05 | IVFM 5 = Application acknowledgement to an addressed message |
| 001 |  | 06 | IVFM 6 = Virtual AtoN |
| 001 |  | 07 | IVFM 7 = ASM message |
| 001 |  | 08 | IVFM 8 = Maritime Messaging System (MMS) message |
| 001 |  | 09 | IVFM 9 = VDE-SAT Orbit Data |
| 001 |  | 10 | IVFM 10 = VDE-SAT Position Retransmit |
| 001 |  | 11 | IVFM 11 = VDE Position Message |
| 001 |  | 12 to 1023 | Reserved for future system applications |
| 001 |  | 1024 to 65565 | Reserved for international operational applications |

For the IVFM Definitions, see Annex B.

### Security Options

Authentication of messages is an important feature which VDES can offer. Authentication ensures the authenticity of the delivered data. Authentication is not part of AIS, but the use of ASM and VDE can provide this feature. Examples of use cases that would benefit from authentication are: Position reports (from AIS), safety related ASMs, repetitive ASM as well as single ASM, AtoN messages.

A broadcast authentication protocol may be the TESLA (Timed Efficient Stream Loss-tolerant Authentication) [editor: add reference] protocol that uses a combination of asymmetric (public key-based) and symmetric (secret key) cryptography to minimise protocol overhead to meet the VDE-TER capabilities.

### Platforms for the provision of Maritime Services

#### Example: Architecture using the Maritime Connectivity Platform

VDES can be used as data transport medium in the maritime domain to support many applications.

For applications that require the above listed key features of

  - language independent and machine-to-machine communications,

  - cyber security, and

  - international service discovery,

the concept of the maritime connectivity platform [editor: add reference to https://maritimeconnectivity.net/ ] contributes with major key building blocks

  - Maritime Identity Register (MIR),

  - Maritime Service Registry (MSR), and

  - Maritime Messaging Service (MMS).

The Maritime Identity Register MIR is a decentral register of maritime digital communication participants and provides authenticity between individual participating parties in the maritime digital communication domain. Trust is provided by issuing unique digital certificates to users based on their individual maritime resource name (MRN) [editor: add reference to IALA MRN specification] according to accepted vetting procedures by certified organizations. Participants can use the MIR to verify maritime service or other individual participants’ signatures for authentication, to fetch public certificates needed for encryption and to update revocation lists to invalidate cached certificates that were reported outdated or stolen. Participants listed in the MIR are secured by a vetting procedure, ensuring their authenticity. Each participant in the maritime digital communication domain is using a list of trusted MIR servers. The content of the MIRs is cacheable on board to allow ad hoc secure communications without requiring direct connectivity to a MIR in real time.

The Maritime Service Registry MSR is a decentral register listing vetted and quality checked services with their browsable and searchable key parameters such as:

- Service MRN,

- Service Name,

- Service Topic,

- Service geographical area,

- Digital format used.

The Maritime Messaging Service MMS is a concept comprised of components that allow exchange of digital binary “messages”, i.e. arbitrary digital data, organized in a predefined known way to be used by digital applications. The MMS provides a non-synchronous data exchange, that can be realized over changing, intermittent and heterogenous digital connectivity. The MMS supports both secure IP based transport through internet connectivity and through VDE-TER and VDE-SAT connections, and thus provides a solution to fulfil the requirements to securely transport trusted services and private communication between all maritime participants who at least have VDES or IP connectivity sometimes.

The concept of operations of MMS with VDES is identified in [Figure 4].

Shore services are registered in an MIR and MSR. The ship installation holds a cached version of these, shown in [Figure 4] as MIR* and MSR*. Synchronization of these ship copies of MIR and MSR should happen through MMS when broadband connectivity is available. These MIR and MSR copies on board of the ship make it possible for ship applications to discover services and provide all security features of the MMS according to the principles of MCP.

MMS with VDES, System Overview

Shore MCP/MMS infrastructure

Ship Communications

Ship MMS Applications

Ship MMS Proxy

VDES

IP connection

(VSAT, Iridium, 3G,

cable, etc.)

ECDIS Ed

.

4

Legacy

Systems

Active Route

Route negotiation

/

Text Chat

Standardized route

exchange format

(

IEC

61174

Ed

.

4

)

Communication Router with VTS

(

select AIS

/

VDES

/

VSAT

)

Route Optimization

/

Flow Management

Handling VDES messages

(

e

.

g

.

MSP

,

etc

)

App

App

App

App

App

App

App

App

App

App

VTS Shore Center

AppServ

App

Other Shore Service

Instances

App

AppServ

LAN

MMS Agent

VDE

AIS

ASM

IEC 61162-450

VDE-SAT

network

SMMP (Secure Maritime Messaging Protocol)

VDE-TER

network

MMS

Agent

MMS

Agent

MMS

Router

Network

IEC 61162-1

MIR

MSR

service registration

identity registration

Modern ECDIS

MIR*

MSR*

VDES

#### The Maritime Messaging Service Architecture in short

In the MMS, all communication between ship applications and to shore services happens through a shipside MMS Agent and an MMS router network using two Protocols:

        - The Secure Maritime Messaging Protocol SMMP, and

        - The Maritime Messaging Transport Protocol MMTP.

The basic MMS communication architecture is shown in [Figure 5] and the following chapters give a short introduction to the components of the MMS [later we can refer to documentation at MCP, but that is not yet officially available].

[Caption Figure 5]: The MMS communication architecture

#### Secure Maritime Messaging Protocol

The SMMP connects applications on ship and shore securely with each other providing

- segmentation and reassembly of messages larger than the MMTP (and VDES) limit,

- streaming-like connectivity,

- integrity (no alternation of the message),

- confidentiality (encryption),

- authenticity (origin of message), and

- non-repudiation (message received by receiver).

#### Maritime Messaging Transport Protocol

For the transport of MMS messages between MMS Agents and through the MMS Router Network, the MMTP provides

- authentication of an MMS Agent to an MMS Router,

- the subscription to messages of a specific topic,

- the subscription to messages to a specific MRN,

- the transport of subscribed messages from the MMS Router Network to the MMS Agent, and

- the transport of direct messages from the MMS Agents to the MMS Router Network.

#### Maritime Message Service Agent

An MMS Agent provides message broking functionality to its local environment including

- local transport of MMS messages between ship side applications,

- store and forward of MMS messages between the MMS Router Network and the local applications on ship,

- subscription to the MMS Router Network provided topics on behalf of the local applications, and

- distribution of topic messages received from the Router Network to the local applications.

The store and forward functionality provided by the Agents towards the MMS Router Network makes it possible to use the Agent while it has no connectivity to the Router Network. As soon as the Agent detects connectivity, it then may use connectivity to forward messages to other ships directly or through the Router Network by use of different communication technologies available to it.

An MMS Agent may define routing priorities to allow certain types of messages through certain connectivity types to allow cost control, e.g. to avoid using expensive satellite connectivity for huge chart updates, but at the same time to allow safety critical messages to pass through all kinds of communication.

#### Router Network

The MMS Router Network consists of one or multiple MMS Routers.

An MMS Router handles MMS message transport for a set of subscribed MMS Agents. Subscriptions are either for a specific topic or a specific MRN. An MMS Router queues messages that an MMS Agent has subscribed to until they are fetched by that MMS Agent. An MMS Router deletes stored subscriptions and queued messages after a timeout occurs.

Note that MMTP does not give a delivery guarantee, but SMMP does. So SMMP will handle retransmissions in case this is requested for applications that require this.

The MMS Router Network handles the transfer of stored subscriptions and queued messages between the MMS Routers in case an MMS Agent roams from one Router to another.

The MMS Router Network supports the lookup of a MMS Router by request from an MMS Agent, according to its current connectivity situation.

#### Using Maritime Messaging Service with VDES

In order to utilize VDES transport with VDES, the shipside MMS Agent does have a VDES connector that transports MMTP over VDES though a VDES network. At the edge of that VDES network, a gateway contains another MMS Agent function to connect to the MMS Router Network on behalf of the ship’s MMS Agent, and perform store and forward functions as connectivity through that VDES Network comes and goes.

[Caption Figure 6]: MMS with VDES as transport

As [Figure 6] shows, the VDES Network is bridging the shipside Agent, extending its reach into the access to the MMS Router Network.

This is facilitated by

- identifying the MMS VDE binary message content in any VDES message by a leading 16-bit ASM identifier to indicate the following content is encoded by the MMTP protocol,

- the VDES Network informing the ship Agents about the VDES Destination ID in use for the Agent that has connectivity to the MMS Router Network; ship Agents address all MMS traffic that uses a VDES Network to this VDES Destination ID, which then will unpack the MMTP protocol and transport it to the MMS Router Network,

- acknowledging that a VDES Network may perform MMS Routing internally if the MMTP destination is known to the VDES Network, i.e. on board of a VDES Satellite seeing the destination ship right now in its range,

- the optional application of MMTP header compression in the VDES transport to reduce the overhead of kown service MRNs, i.e. MSI service provider can be compressed to a VDES Station ID; this can be achieved by assigning VDES Station ID’s outside the MMSI range to MRNs that are frequently used in a VDES Network, and informing all Agents about these in a broadcast.

The resulting protocol stacks are shown in [Figure 7]

[Caption Figure 7]: The MMS protocol stack using VDES transport; the ‘?’ indicates that the transport inside the VDES Network is not relevant for MMS to work; the ‘*’ on the ship side indicates that multiple options exist, HTTP being one of them (like in SECOM [editor: add ref. to SECOM IEC standard]).

#### Status of Maritime Messaging Service

The MMS is currently under development at the MCC in 2022, with the plan to be standardized formally by RTCM during 2023 to be published for the deployment of VDES with MMS in start of 2024.

European and Baltic demonstration projects [editor add reference to the MARIOT project] will demonstrate early MMS versions during 2023.

## operational characteristics

The VDES operates according to ITU-R M.2092, which includes the following operational characteristics:

- The system gives its highest priority to the automatic identification system (AIS) position reporting and safety related information.

The system installation is capable of receiving and processing the digital messages and interrogating calls.

The system installation operates continuously while under way, moored or at anchor.

The system, for the terrestrial links, uses the appropriate time-division multiple access (TDMA) techniques, access schemes and data transmission methods in a synchronized manner.

The system is capable of various modes of operation, including the autonomous, assigned and polled modes.

The system prioritizes applications and adapts parameters of the transmission (robustness or capacity) while minimizing system complexity.

## System Overview

The VDES includes:

- Antenna(s), capable of transmitting and receiving data through terrestrial and satellite link.

An AIS as set out in resolution MSC.74(69) ANNEX3.

A multi-function data communication and timing process that is interoperable with AIS, ASM and VDE.

A multi-function transmitter, capable of operating on the designated AIS, ASM and VDE frequencies.

Multi-function receivers, capable of operating on the designated AIS, ASM and VDE frequencies.

A means to automatically input data from other sources.

A means to automatically output data to other devices.

A means of ensuring the integrity of the data.

A means to automatically or manually update the device software as needed.

Functionality of a built-in test equipment (BITE).

GNSS receiver to support AIS and to possibly serve as a secondary source of PNT.

## Assumptions and dependencies

The applications related to the VDES address the following assumptions and dependencies:

- VDES operates within the existing AIS environment.

VDES respects and supports requirements for GMDSS communications, including SAR, urgency, and safety related messages.

VDES applications are uniquely identified.

The VDES related applications operate in a manner that ensures there is no unnecessary repetition of messaging.

VDES contains AIS and may extend its capabilities.

While AIS is it used primarily for position reports, it is assumed that VDE will be used for other data communications. For ASM, AIS channels may still be used, but in high traffic areas, ASM channels are preferred.

## Services that use VDES

Following aspects may be of help when deciding on implementation of VDES as a transport for maritime services.

VDES shore station operations should consider:

AIS system load;

services to be provided;

existing equipment (for existing sites);

antenna requirements, including interaction with other services/systems;

power requirements;

installation costs (which may include mobile station, tower access);

operational costs;

coordination with other administrations may be required;

Provision of VDES shore infrastructure  may be managed through:

a regular scheduled lifecycle process for existing AIS shore stations;

through an upgrade of existing AIS shore stations; or

through the implementation of VDES at a new location where there is no existing AIS shore station.

Table 2 provides further information on considerations for VDES implementation.

| Consideration | Detail | AIS recommended | VDES recommended |
| --- | --- | --- | --- |
| AIS system load | AIS system load (see measurement technique in note 1) | <50% load | >50% load |
| Application Specific Messages | Application Specific Messages (ASM) are delayed beyond that acceptable to the users (see note 2) | Service Latency expected | Service Latency not expected |
| Advanced data transfer services | If the Advanced system data transfer features are required as per the Maritime Service Portfolios | Not required to accommodate | May be accommodated |

Considerations for VDES Implementation

Note 1: System load is determined by measuring the number of vessels in the coverage area of an AIS base station (a single slot map) and calculating the sum of the slots consumed in the coverage area based on the expected reporting rates of the vessels in the coverage area (Class A, Class B, AtoN and AIS base station) and the number of Addressed Binary Messages (ABM) and Broadcast Binary Messages (BBM) of all types.

Note 2: Service latency is defined as the delivery of the total service from the initiation of the delivery of the service component to the completion of the delivery of that service component e.g. a single maritime chart update.

The services offered by a VDES system will allow for priority with essential services / safety related services having the highest priority and non-essential / commercial service having the lowest priority.

The transfer of data using VDES should consider that the available VDES data transfer capacity is shared by all users within the coverage range of a VDES base station.

A technical overview of VDES is provided in Annex A.

Other means could be considered but take into account specific characteristics of these communication system

- Near real time small messages

- No streaming or large files

- Doesn’t need shore infrastructure

- No need for subscription at provider

- Support for PNT consistency system near shore (R-mode)

# POTENTIAL USES OF VDES

To assist in identifying possible options for use of VDES a number of potential scenarios have been developed. These are presented to provide context for development and implementation of digital communications, including VDES. The potential uses of VDES presented here are not intended to be an exhaustive list.  It is expected that VDES uses will evolve as the system is implemented. The use cases are cross referenced to Maritime Service in the context of e-Navigation as noted in Table 4. Other potential use cases of VDES are presented in [Table 5].

| Potential uses of VDES | MS in the context of e-Navigation Reference |
| --- | --- |
| SAR Communications | MS 9 - Telemedical Maritime Assistance Service (TMAS) MS 16 - Search and Rescue (SAR) Service |
| Maritime Safety Information | MS 5 - Maritime Safety Information (MSI) service MS 13 - Ice navigation service MS 14 - Meteorological information service MS 15 - Real-time hydrographic and environmental information services |
| Ship Reporting | MS 8 - Vessel shore reporting MS 15 - Real-time hydrographic and environmental information services |
| Vessel Traffic Services | MS 1 - VTS Information Service (INS) MS 2 - Navigation Assistance Service (NAS) MS 3 - Traffic Organization Service (TOS); MS 4 - Local Port Service (LPS) MS 6 - Pilotage service MS 7 - Tugs service |
| Charts and Publications | MS 11 - Nautical chart service MS 12 - Nautical publications service MS 15 - Real-time hydrographic and environmental information services MS 17 – AtoN service (including PNT and SBAS) |
| Route Exchange | MS 1 - VTS Information Service (INS) MS 2 - Navigation Assistance Service (NAS) MS 3 - Traffic Organization Service (TOS); MS 4 - Local Port Service (LPS) MS 5 - Maritime Safety Information (MSI) service MS 6 - Pilotage service MS 7 - Tugs service MS 8 - Vessel shore reporting MS 10 - Maritime Assistance Service (MAS) MS 11 - Nautical chart service MS 12 - Nautical publications service MS 13 - Ice navigation service MS 14 - Meteorological information service MS 15 - Real-time hydrographic and environmental information services MS 16 - Search and Rescue (SAR) Service |
| Logistics | MS 7 - Tugs service |

Potential VDES Uses cross-referenced to IMO SIP MS update 1

| Potential uses of VDES | New services that may utilize VDES |
| --- | --- |
| Loss of GNSS (PNT) | R-Mode (provides backup PNT) |
| Message forwarding | Message forwarding to prevent shadowing or for redundancy |
| AtoN | Adding VDES capability (including ASM on ASM channels) to AtoN |
| Enforcement of fishing territories |  |

Other potential VDES Uses

VDES is intended to support the following use cases by terrestrial and satellite communication. The VDES extend the capabilities of AIS.

## IALA test beds

IALA has provide a list of test beds that have been performed by administrations (refer to: https://www.iala-aism.org/technical/e-nav-testbeds/active-testbeds/).

## Integrity check / authentication

VDES comprises of AIS, ASM, VDE-TER and VDE-SAT with different data rate capabilities. Therefore, AIS and ASM have no or very limited capabilities to authenticate the data for the receiver. The authentication of the AIS and ASM messages allows the necessary trust in the shared messages to make safe decisions. VDES could extend messages from AIS, ASM and VDE to provide additional authentication information e.g. by the TESLA protocol.

### Scenario – Authentication for position reports

Positioning reports are broadcasted without authentication by ships and AtoNs through AIS. Without authentication there is no way for the recipient to verify the content of the positioning report. VDE may complement the continuous broadcast stream of AIS positioning reports to authenticate the content through e.g. by the TESLA protocol.

### Scenario – Fishing vessels certification

Independent authentication of the location of a fishing vessel with time will be used for the fishery certification to prevent IUU (Illegal, Unreported and Unregulated) fisheries. Consistency between R-mode and GPS location will be an endorsement.

For example, fishing vessels may provide the information of fish catch (fish name/type, amount) together with the AIS information (ship ID, location, date). This information is sent to port of fish unloading and also fishing operation control authority/center. The authority may send a certificate of fish catch to the vessel. Fishing vessels that do not send information of fish catch, may be suspected for IUU fishing.

## MASS

MASS (Maritime Autonomous Surface Ships) may navigate by its own AI system. However, it is necessary to monitor and observe the movement of, and if necessary to control to, MASS from remote (land-based) operation/control facility. VDES provides a reliable ship-to-shore, shore-to ship and ship-to-ship communications using low data-rate exchange for MASS navigation safety purposes, in particular, to prevent incidents such as grounding, collision with other ships, adverse consequences of bad weather etc. VDES can also be provide ship-to shore data communication of situation/status/condition of MASS. It is also expected an establishment of VDES communication between/among AIs of MASSs for prevention of collision of these MASSs. The format of data will be defined/specified by other technical standards (ISO, IEC). Regulatory framework for MASS will be established by IMO.

## Positioning, Navigation and TIming (PNT)

### Scenario - R-mode

### Scenario - GNSS Resiliency, precision  and integrity

GNSS receiver has progressively acquired increasing relevance as primary means of PNT information, integrated with other functions as ECDIS, GMDSS and AIS. However GNSS and other radionavigation and data services are vulnerable to intentional and unintentional interferences and common failure mode.

To increase the level of PNT resilient nowadays there are available different alternatives like the use of GNSS multi-constellation and multi-band receivers [Multi system GNSS receiver IMO Resolution MSC.401(95)], the use of augmentation systems (SBAS/GBAS)[compliant with Operational Requirements IMO Resolution A.1046(27)], the use of RAIM algorithms for maritime, the provision of redundancy of the electronic position fixing system (EPFS), the use of new GNSS services (e.g. Galileo High accuracy service – HAS - and Galileo Open Service Navigation Message Authentication – OS-NMA) among others. VDES and R-Mode will also contribute to minimize these sources of vulnerabilities on-board.

In that sense SBAS together with RAIM provides increased accuracy and integrity associated to PNT solution, complementing DGNSS services and providing additional capabilities (e.g. extended regional coverage or and the evolution to include dual frequency multi-constellation - DFMC).

SBAS is aligned with IALA guideline G1152 (“SBAS Maritime Service”). SBAS maritime receiver will be type approved equipment and compliant with the future new standard IEC 61108-7 that sets up the performance requirements and method of testing of these equipment.

SBAS corrections can be used as a source of differential corrections that can be retransmitted following the methodology and rules fixed in the IALA guideline G1129 for marine Aids to Navigation (AtoN) service providers which explain how to implement and use SBAS as a data source for DGNSS. These SBAS corrections can be retransmitted using MF IALA beacons and AIS stations.

SBAS information is re-transmitted in MF IALA beacons and AIS leveraging the RTCM format in place to inject the differential corrections.

SBAS information could also be transmitted in RTCA native format in the new VDES, especially in areas where there is not coverage or a proper reception of SBAS signal in space (SiS). Note that the current RTCA format allows the implementation of the corrections, without any change, with independency of the location of the VDES transmitter including Satellite VDES.

SBAS information and corrections can be also re-transmitted using VDES architecture and functions, enhancing safety of navigation, in accordance to the current transmission rules through AIS message application.

Moreover, SBAS information will be able to be transported through VDES using a new VDES binary message in RTCM format (if is applicable). This VDES message will allow to indicate that one differential corrected position is used on-board based upon SBAS corrections (set differential flag). In addition, the VDES message will also provide a GNSS health report to clarify the current status of this PNT source.

SBAS information can be provided to external equipment via the PI, using a -450 interface (a new PI message may also be required).

### Scenario - Sharing Almanac DGNSS

DGPS stations, eLORAN, AIS message 17 stations, RACON’s….

## SAR communications

SAR Communications are defined in existing documentation (ref SOLAS IV, SAR 79, IAMSAR Manual, NAVTEX manual and SafetyNet manual).

VDES is a technology that supplements AIS communications, and as such may be used for data communication of Maritime Safety Information (MSI) and supplementary distress communications.  VDES supports both addressed (unicast and multicast) and broadcast communications to support SAR response.

When available, the VDES satellite component (VDE-SAT) may be an effective means to extend the VDES to areas outside of coastal VHF coverage.  The VDES-SAT may deliver information in a broadcast, multicast or unicast mode to a broad area, addressing many ships using only minimal radio spectrum resources.  The VDE-SAT will provide a communication channel that is complementary to GMDSS and the terrestrial components of the VDES system (i.e. coordinated with terrestrial VDES, application specific messages (ASM) and AIS functionalities and their supporting systems).

As a communications medium, VDES may be used to relay distress alerts and locating signals (i.e. SARTs).  VDES has also potential to supplement other GMDSS functional requirements which require further development through the GMDSS review process.

In this use case the mix of current communications and developing communications techniques can enhance and improve the sharing of information in prosecution of a SAR incident.  This would include text in free form / standard formats; transfer of waypoints / route information for display on on-board equipment; transfer of GIS information / search patterns; images; etc.

The VDES can be used in SAR planning, execution and decision making.

### Scenario - Distress Communications – Distress Relay

Once a distress alert is initiated and the information has been forwarded to a Rescue Coordination Centre (RCC) through the established GMDSS process, the RCC forwards information of the incident to vessels in the area.  The forwarding of information, using existing formats, could be provided by various communications means, including VDES.  The forwarding of data over a digital communications system such as VDES could facilitate the integration and display of information on external systems on-board (for example, Radar, ECDIS). Information could then be passed to the RCC and other vessels in the area including course to intercept, ETA on-scene, on-scene conditions, sharing of a common operating picture, etc.

### Scenario - SAR Operations – initiate search / response

SAR Mission Co-ordinator (SMC) develops response to SAR using resources, search plan, etc.  Information to prosecute SAR operation is transmitted to the On Scene Commander (OSC) and SAR response units (SRU) (for example information on resources, plan, waypoints for search pattern, SRU responsibilities, etc.).  This could be done using VDES, providing a common operating picture and information using standard templates and formats.

### Scenario - SAR Operations – information exchange

During a SAR mission, the OSC and SRU provide regular updates on the search / response to the SMC.  In addition, the OSC and SRU share information between each other to facilitate the response.  The VDES could be used to exchange information on the SAR plan, SAR execution and other pertinent information to facilitate SAR operations.  Information could be automatically integrated with, and portrayed on, external systems both ashore and on-board, including the RCC GIS, decision planning and support systems.

Using information from other systems, such as vessel route, information provided could be tailored to be relevant for the vessel based on its route.

In addition, the OSC and SRU could share information between each other to facilitate the response.

### Scenario - Tele-medical

A tele-medical happens when a person is injured or sick on-board a vessel or platform and there is a need to communicate with a doctor ashore for medical assistance and prognosis.  The conversation with doctor could be by voice, with transfer of images / photos / indication from medical equipment on patient’s condition.  VDES could be used to transfer advice, images or other information.  Where there are language difficulties, VDES could assist with machine to machine communications and/or language independent communication.  Information exchange could be integrated with, and portrayed on, external systems on-board or ashore (medical facility).

### Scenario – Medical Evacuation (MEDEVAC)

A MEDEVAC may be necessary to evacuate a severely injured or sick person.  VDES may be used to exchange pertinent medical information from the ship to the SRU and destination medical facility.  The SAR Mission Co-ordinator (SMC) develops the response to prosecute the MEDEVAC and could use VDES to provide the plan to the ship and responding unit.  Information on the status of the patient could be transferred during the MEDEVAC from medical equipment on the SRU.

### Scenario - Initial Distress Position Sharing (IDPS)

The Galileo Search and Rescue Service (SAR/Galileo), designed in order to support the COSPAS-SARSAT (C/S) International Satellite System for Search and Rescue (MEOSAR) Programme, introduces the SAR/Galileo Return Link Service (RLS) [editor: add reference to https://www.gsc-europa.eu/sites/default/files/sites/all/files/Galileo-SAR-SDD.pdf]. Once an emergency beacon (EPIRB, PLB,…) RLS-enabled is activated, its signal and alarm message is forwarded via the Galileo satellites in view to the corresponding Mission Control Center (MCC), and following the established operational chain, an acknowledgment receipt is sent to the beacon, the return link message (RLM), which indicates that the emergency signal has been received and the help is coming.

This RLM message includes the 15 Hexadecimal ID of the 406 MHz Beacon in distress, with 19 bits indicating its coarse position [editor: add reference to Specification for COSPAS-SARSAT 406 MHz Distress Beacons C/S T.001 - issue 4 (June 2021)]. This information is used to build the NMEA sentence, understandable by any compatible equipment compliant with IEC standard [editor: add reference to IEC 61162-1 and NMEA 0183).

Any Galileo-enabled receiver implementing the Galileo OS SIS ICD is able to read and decode the RLM and provide the standardised NMEA sentence, including the beacon ID and the gross position of the located beacon (c.a. 30 nm). VDES equipment (either shoreside or shipside) with a GNSS receiver implementing Galileo OS SIS ICD can receive and decode the RLM, including the beacon ID and its position, and then put this information available to the VDES equipment itself so that it can be able to retransmit the information about a vessel/mariner in distress in the specific location specified in the RLM. This information can be sent to any vessel in the area with VDES onboard equipment (even if they are not using a Galileo-enabled receiver) and (potentially displayed in the vessel ECDIS) thus allow them to help, when possible, in the assistance of the mariners in distress after contacting the corresponding search and rescue service for confirmation of last known position (if possible[1]).

Search and Rescue services can use this mechanism as an initial communication to vessels in the area. This is linked to 3.4.1.

- The above proposed service can be used as a secondary supplement to GMDSS.

[1] Note that according to regulation V/33 of SOLAS convention, the master of a ship at sea which is in a position to be able to provide assistance, on receiving information from any source that persons are in distress at sea, is bound to proceed with all speed to their assistance, if possible informing them or the search and rescue service that the ship is doing so. This obligation to provide assistance applies regardless of the nationality or status of such persons or the circumstances in which they are found. If the ship receiving the distress alert is unable or, in the special circumstances of the case, considers it unreasonable or unnecessary to proceed to their assistance, the master must enter in the log-book the reason for failing to proceed to the assistance of the persons in distress, taking into account the recommendation of the Organization to inform the appropriate search and rescue service accordingly.

## Safety Related Information

Information regarding safety of navigation and protection of the environment can be transmitted through the VDES.  Safety related information could use the broadcast aspect of VDES.

This includes Maritime Safety Information (MSI) as defined in IMO SOLAS V, regulation 4 (navigational warnings), SOLAS V, regulation 5 (meteorological services and warnings) SOLAS V, regulation 9 (hydrographic services) and SOLAS V, regulation 31 (Danger Messages).  Other references include MSC.1/Circ.1287 rev1; MSC.1/Circ.1288; (additional reference COMSAR Cir.15) the IMO Worldwide Radionavigation System IMO resolution A.706(17) (as amended) sets out the Worldwide Radionavigation System (WWRNS).

The floating navigation aids may provide near real-time information on meteorological and hydrographical information such as tide problems, changes in the seabed, water depths and sea states may be provided.

In this use case information shall be transmitted in standardised formats that can take advantage of the VDES capabilities (for example - IHO S-124 formats).

The requirement includes the ability to send information to a predetermined area (i.e. NAVAREA and METAREA) or an area of particular interest defined by administration.

### Scenario - Meteorological Services and Warnings / Navigational Warnings

It is important to have up to date information on the weather that can be expected along a ship’s planned route.  VDES could be used to facilitate information exchange relating to the route of the vessel, integrated with, and portrayed on external systems on-board.

### Scenario - Ice maps

Information on sea ice conditions around a vessel is important to help ensure safe passage at sea. Knowledge of areas with sea ice along a ship’s planned route allows ships to find the most efficient route at an early stage. Together with prognoses for expected ice movements, ice charts allow mariners to plan ahead and significantly reduce the risk of vessels becoming ice locked.  VDES could be used to provide this information, which could then be integrated with, and portrayed on external on-board  systems. Ships may also provide information from observations back to the service provider to update the ice maps.  In addition, information on the latest version of ice maps may be provided from one ship station to another ship station.

### Scenario - Notices to Mariners

Notices to mariners are a means to disseminate navigational safety information (as part of maritime safety information).  SOLAS V, Regulation 9 (Hydrographic Services) notes that administrations should undertake to arrange the dissemination and update of all nautical information necessary for safe navigation (for example. predictive and real-time tides and currents).  VDES could be used to provide this information, and changes to information, with respect to the waterway.

### Scenario - GNSS Augmentation

SOLAS Chapter V, Regulation 19 notes that all ships, irrespective of size shall have a receiver for a global navigation satellite system or a terrestrial radio navigation system, or other means, suitable for use at all times throughout the intended voyage to establish and update the ship's position by automatic means.  IMO Resolutions A.915(22) and A.953(23) provide the requirements for Maritime Radio navigation Systems.  Distribution and/or retransmission of GNSS augmentation information via VDES could allow GNSS users to get timing, integrity data and improved position accuracy.

### Scenario - Crowd sourced information

Information from users or ship systems may enhance and/or validate meteorological hydrological and hydrographic information that is made available to other vessels in the area and authorities.  VDES could be used to facilitate crowd sourced information.

## Ship Reporting

Ship reporting can include mandatory and voluntary reports required for a number of purposes by vessels to various shore authorities.  Information on ship reporting is provided in IMO SOLAS V, regulation 11 (ship reporting systems), 19-1 (LRIT), regulation 31 (danger messages), regulation 32 (information required in danger messages), MARPOL and SAR Convention, Chapter 5.  Additional information on ship reporting is contained in Resolution A.851(20) and FAL.5/Circ.36.

Information forwarded through VDES may transfer data for integration into national and/or regional systems could be sent by VDES to a Maritime Single Window Platform (MSW). (e.g. SafeSeaNet, VTS).  Information may also be sent to the ship agent or owner or a service provider.  Ship reporting could use the addressed (unicast and multicast) aspect of VDES.

### Scenario - Submit updated information

As the voyage continues, updated information will be provided.  This can include updated estimated time of arrival; change in condition of the vessel; change in route of the vessel.  This is a user defined report, which could be based on a set template for ‘updated information’ or free-text report.  VDES could facilitate exchange of information.  The vessel may be interrogated for information on request, based on its route, operating area or position.

### Scenario - Submit arrival notice

A notice of arrival report is based on known content and could be set in a template form.  The aspects of the template report, such as information on the ship particulars, would be pre-populated.  Where possible, additional information related to the voyage, such as destination, ETA destination, last port(s) could be populated from other systems that contain such information.  Other information that may be provided include ISPS reports; ship crew information; information specifically required by the shore authority.  This may be submitted using data populated automatically from other systems or may require manual input by the mariner.  VDES could facilitate exchange of information.

### Scenario - Provide initial report to shore (prior to departure)

Prior to departure specific, standard information is required.  This may include information required for clearance to depart.  Reports could be pre-populated from available information where possible.  Specific information may be required to be entered manually.

### Scenario - Secure ship reporting

There can be times when it is necessary for secure ship reporting, including times when the vessel may disable  AIS transmissions.  Using VDES, information could be forwarded through a secure communications link.

### Scenario - Danger Message

The Master of a vessel is required to report dangerous conditions (SOLAS V, regulation 31 and 32) as well as any irregular conditions referring general marine domain awareness, such as: dangerous ice, derelicts, dangers to navigation, tropical storm, severe weather, ice accretion.  VDES could facilitate the provision of this information to both shore authorities and other vessels in the area.  Information exchange may be integrated with, and portrayed on, external systems on-board.

### Scenario – Reporting 3D position

Currently vessels report automatically their absolute horizontal position (i.e. latitude and longitude) via AIS. They also report their current draught, which gives the vertical position related to the surface of the sea. Horizontal position information is obtained (usually) via GNSS while draught is reported manually by the crew.

The vertical component of GNSS provided position is less accurate than the horizontal component, but with current GNSS technologies with multifrequency receivers supported by augmentation information it is possible to achieve submeter accuracies also for absolute vertical position. Thus, if the location of the GNSS antenna related to the keel and to the highest point of the vessel is known, it would be possible to monitor and report the absolute and accurate 3D position of the vessel in real time.

In cases, where the seabed and/or any above water obstruction (e.g. bridge) has been charted/mapped using a stable reference level (geodetic datum), which can be related to GNSS reference level, it is possible to monitor the safe distance to seabed (i.e. Under Keel Clearance) (http://www.famosproject.eu/activities/future-navigation/) or to the bridge structures using GNSS position. This information could assist the vessel but also VTS when monitoring the vessel's safe journey in the VTS area.

The current AIS messages do not allow the automatic reporting of vertical component of the GNSS position, but this might in the future be something that would be useful information also for shore authorities. In addition to the 3D position, report could include additional sensor information related to the vessel's movements caused by wind and waves if such information is available (i.e. pitch, roll and heave in vertical direction and sway, surge and yaw in horizontal plane). IMO MSC.1/Circular.1575 defines requirements on data output of PNT, establishing several grades, and where the most complete one (Grade IV) considers the ship's hull (3D).

It is suggested to include a field in the VDES message indicating if the 3D position is authenticated, following the recommendation on the Guidelines on cyber security onboard ships, where it is stated that cyber incidents can arise as the result of loss of or manipulation of external sensor data, critical for the operation of a ship. This includes but is not limited to Global Navigation Satellite Systems (GNSS).

[Real-time monitoring of under keel clearance (three dimensional position would be nice – reporting requirement)

Where does the information come from competent waterway authority or ship? What about responsibility?

In some cases data is needed about the depth of a fairway combining the position of the antenna with SBAS accuracy and the depth (echo sounder) of the ship and the difference between the antenna and the echo sounder. (Height of the antenna to sea bottom)

Bridge clearance is depending on the height of the ship and the water level at the moment the ship is planned to cross the bridge or at the dock with cranes.

- Sounder depth

- Height of antenna

- Loaded-unloaded (and effect on ship – draught will change)

- Tide problems

- Difference between antenna position and sounder

- Drawing?

- Structure above receiving antenna

- Resolution A.915(22) Adopted on 29 November 2001 (Agenda item 9) REVISED MARITIME POLICY AND REQUIREMENTS FOR A FUTURE GLOBAL NAVIGATION SATELLITE SYSTEM (GNSS)

- May need additional measurement of water level from floating AtoN.]

### Scenario – Ship Weather observation report from ship

Ships may participate in the provision of weather observations, as noted in MSC.1/Circ.1293.  This is a voluntary observing ship (VOS) scheme with information provided to the World Meteorological Organisation.  VDES could facilitate this reporting, with information provided directly from on-board sensors.

VDES  may improve the reliability of the reception via  the global satellite capabilities of VDE-SAT for weather information provided by IMO SN.1/Circ.289 DAC 1 FI 21.

- The Circ.-289 weather messages can be transported via VDE-SAT using VDES messages.

The WMO Weather observation report from ship message is intended for ships which have been recruited by national meteorological services to undertake weather observations at sea in accordance with the provisions of SOLAS chapter V, regulation 5, and the World Meteorological Organization's Voluntary Observing Ship (VOS) Scheme. Because national meteorological services are the intended primary users of this message it has been developed to reflect the coding principles prescribed by WMO in its Binary Universal Form for the Representation of meteorological data (BUFR), and as contained in Part B of WMO Publication No.306, (Manual Codes, Volume I.2). The parameters coded in this message are therefore not fully compatible with the recommendations set out in ITU M.1371.

## Vessel Traffic Services

Vessel Traffic Services is included in SOLAS Chapter V, Regulation 12, with further information in IMO Resolution A.1158(32) Guidelines for Vessel Traffic Services (NCSR7/23 Annex 5).

IMO Resolution A.1158(32) states that:

“The purpose of VTS is to contribute to safety of life at sea, improve the safety and efficiency of navigation and support the protection of the environment within the VTS area by mitigating the development of unsafe situations through:

- providing timely and relevant information on factors that may influence ship movements and assist onboard decision-making. This may include:.

  - position, identity, intention and movements of ships; .

  - maritime safety information;

  - limitations of ships in the VTS area that may impose restrictions on the navigation of other ships (e.g. manoeuvrability), or any other potential hindrances;

  - other information such as reporting formalities and International Ship and Port Facility Security Code (ISPS Code) details; and

  - support for, and cooperation with, allied services;

- monitoring and managing ship traffic to ensure the safety and efficiency of ship movements. This may include:.

  - planning ship movements in advance;

  - organizing ships under way;

  - organizing space allocation;

  - establishing a system of traffic clearances;

  - establishing a system of voyage or passage plans;

  - providing route advice; and

  - ensuring compliance with and enforcement of regulatory provisions for which they are empowered,

- Responding to developing unsafe situations, which may include:

  - a ship unsure of its route or position;

  - a ship deviating from the route;

  - a ship requiring guidance to an anchoring position;

  - a ship that has defects or deficiencies, such as navigation or manoeuvring equipment failure;

  - severe meteorological conditions (e.g. low visibility, strong winds);

  - a ship at risk of grounding or collision; and

  - emergency response or support for emergency services.

To achieve their purpose, VTS should provide information or issue advice, warnings and instructions, as deemed necessary”.

The IALA Guideline G1089 “Provision of Vessel Traffic Services (VTS)” has the purpose to provide guidance for the provision of VTS to participating ships in a harmonized manner, in accordance with IMO Resolution A.1158(32) and IALA standards.

[This is where we stopped work during JWG June 3, 2022]

Information required by the VTS can be both standardised (supported by templates) or specific to a situation.  VTS could use both the addressed  and broadcast aspects of VDES. This would improve the efficiency of the port operations and would expedite the ships equipped with VDES for those ports also equipped with VDES.

VTS involves maintaining a vessel traffic image, and relies on vessel tracking from sensors such as radar, AIS, CCTV, other VTS centres.  The vessel traffic image may be supplement with crowd-sourced information from vessels data (sensor data from ships provided to the shore to expand the traffic image range).

VTS also requires interaction with traffic to respond to developing traffic situations.

VTS relies on the ability to provide essential and timely information; monitor the actions of vessels in the VTS area, including monitoring routes and changes in route; interacting with other VTS centres in the region; interact with other port agencies (allied services).

Ports may also provide a specific Local Port Service (LPS) where it is deemed through a risk assessment that a VTS is not required.

### Scenario - Waterway Monitoring

VTS provides monitoring and other services.  VDES may be used to monitor vessels and autonomously provide information to these vessels based on predetermined parameters as defined by the shore authority.  In addition, VDES may enable sharing of information on synthetic VTS targets from the VTS to vessels transiting the VTS area.  Information exchange may be integrated with, and portrayed on, external systems ashore and on-board.

### Scenario – VTS Timely and Relevant Information Service

VTS Timely and Relevant Information Service is provided by broadcasting information at fixed times and intervals or when deemed necessary by the VTS, or at the request of a vessel. .  The information provided may include safety information as previously defined.  Additional information could include specific limitations for navigation in the VTS area (for example manoeuvrability limitations; draft restrictions; channel closures; diving operations).  Information exchange may be integrated with, and portrayed on, external systems ashore and on-board. For example, timely information exchange to achieve mutually coordinated shipping will reduce potential risk. Computer-based automated notification between ships is useful. Further, as noted in section 3.10 below, the efficiency of the port operation would be improved by considering the scheduling of intermodal transportation services to expedite the delivery of the ships’ cargoes and the reloading of the ships with new cargoes.

### Scenario - VTS Responding Service

The VTS Responding Service is described by IMO as ‘a service to prevent the development of dangerous maritime traffic situations and to provide for the safe and efficient movement of vessel traffic within the VTS area’.  The purpose of the VTS Responding Service is to prevent hazardous situations from developing and to ensure safe and efficient navigation through the VTS area.  VDES could be used to exchange this information (e.g. a suggested route).  Information exchange may be integrated with, and portrayed on, external systems ashore and on-board.

### Scenario - VTS Monitoring and Management Service

The VTS Monitoring and Management Service is described by IMO as ‘a service to assist on-board navigational decision-making and to monitor its effects’.  VTS Monitoring and Management Service may be provided on request by a vessel in circumstances such as equipment failure or navigational unfamiliarity.  VDES could be used in the exchange of information during the provision of VTS Monitoring and Management Service.  Information exchange may be integrated with, and portrayed on, external systems ashore and on-board.

## Charts and Publications

IMO SOLAS Chapter V, Regulation 27 (nautical charts and nautical publications) notes that charts and publications necessary for the intended voyage shall be adequate and up to date.  Provision of information on charts and publications could use the addressed and broadcast aspects of VDES.

The aim of nautical chart and publication services are to safeguard navigation at sea by providing information such as nature and form of the coast, water depth, tides table, obstructions and other dangers to navigation, location and type of aids to navigation.

The nautical chart and publication services ensure the official distribution, update and licensing of electronic charts and publications to vessels and other users.

Nautical publications include list of lights, sailing directions, tide and current tables, etc.

There may be a requirement for a ‘user pay’ aspect for some services (i.e. ENC updates).

### Scenario - Updates linked to a ships’ route

An example of this would be a vessel proceeding to a specific location.  At the time of sailing, the vessel had all required charts and publications for the voyage.  These charts and publications were the most up to date at the time of sailing, however some information may have changed during the voyage.  Updated information could be provided through VDES as the vessel as it continues on its voyage, providing information based on the route of the vessel, and limiting the updates to only that information that has changed.  Information exchange may be integrated with, and portrayed on, external systems ashore and on-board.

### Scenario - charts and nautical publications dynamic up-to-dateness

- ship would report revision of existing charts and nautical publications

- shore would provide reference to relevant updates.

## Route Exchange

The development of e-navigation has highlighted the opportunity to make effective use of digital data exchange to support safe and efficient vessel movements.

Route exchange could enhance safety by providing early indication of deviation from the reported voyage plan which may be due to fatigue, weather conditions, or condition of the vessel (possible malfunction).

The use of route exchange could assist with fleet management, whereby information on routes can be exchanged with shore personnel as well as other vessels in the fleet.

In addition, the use of route exchange could assist with route and speed optimisation based on weather and currents; just in time arrival and traffic congestion.

Route exchange could take advantage of the addressed (unicast and multicast) and broadcast aspects of VDES.

Details for route exchange are specified by IEC 61174 edition 4 annex S.

### Scenario - Ship to Ship

This scenario is outside of VTS coverage areas, noting that within a VTS area, route exchange would be through the VTS.  Ship to ship exchange could be sequential i.e. allowing a route to be exchanged between stations via message forwarding (multiple hops i.e. stations in between). Ship to ship route exchange would assist vessels on a transit by predicting when interactions may occur.  VDES could assist in the exchange of digital data to facilitate ship to ship route exchange in order to allow exchange of intentions. Route exchange is essential for the safe passage of MASS.

When a ship meets another ship in close distance, it is necessary to communicate promptly to know and clarify each other the intention of the operation of the ships to avoid a collision. VDES provides direct ship-ship communication when meeting on sea. In addition, automatic translation/interpretation is introduced in this system, the communication can be done among different languages. (Existing system takes longer time to establish such communication: firstly obtain ships call sign by AIS; secondly call the ship by VHF. Such voice communication shall be done the common languages; this is many times difficult for local fisherman.)

### Scenario - Ship to Shore

To inform shipping and other waterway users of possible hazardous situations shore authorities need information about the intentions of the waterway users, such as their intended route.  Based on this information the authorities could organize traffic and, when needed, recommend other routes/possibilities for a safer passage and also provide information about the waterway.

### Scenario - Shore to Ship

Before ships enter a sea area monitored by a shore authority, information about this area could be provided to assist in a safe and efficient passage.  If the route of the vessel is known, information can be tailored to the route.  The shore authority could link the vessel planned route with other information received, such as cargo, and adjust the information as may be required.  Route plans received from ships can be used for detecting possible traffic congestions and high risk situations in advance.  Shore authorities can also send alternative route recommendations to ships when needed.  This allows ships to choose the route that is most suitable for its navigation.  VDES can facilitate reception of route plans which may be integrated with, and portrayed on, external systems ashore.

### Scenario - Navigational Disruption

There may be some event or circumstance that impacts the normal operation of the waterway requiring urgent traffic management to ensure continuity of operations.  VDES could be used to share information on the circumstance, and proposed alternate routing could assist in effective movement of vessels throughout the incident.

### Scenario – High priority ship

In some areas there are ships that have a high priority and should have a “free sailing” in a specific time slot over a specific route. Examples are tide/tidal ships and ships with special cargo.

## Logistics / Services

When sailing from berth to berth before, during and after the voyage there are several logistical aspects that must be addressed.  Most of these are done by an agent on the shore but are changed because of different reasons.  The means of communicating these logistical aspects would depend on the location of the ship / shore elements involved and could include VDES.  In cases where cargo is transferred at sea (transhipment) the location could be out of range of other communications and VDES would be the preferred communication exchange platform.

Information transfer could assist with efficiency of the overall cargo chain. The standard S-211 Port Call Message defines message formats that can help different stakeholders to efficiently coordinate and synchronize processes related to a port call (e.g. pilots, tugs and terminal functions) .  Logistics/Services could use the addressed (unicast and multicast) aspects of VDES.

Sharing of route information could assist with allied services related to shipping and ship movements - this could include locks, pilotage, tug allocation, shore resources, and other logistical aspects.  This use case represents the business to business aspect.

Logistical elements where VDES may be an appropriate communication method include:

Transfer of vessel loading plan;

Tug operations;

Pilotage operations;

Stores / supplies / ship bunkers required; waste removal;

Coordination of ships and vessel traffic with port operations.

### Scenario - Logistic services – ship to shore

Ship arriving at a port forwards revised time of arrival and requests confirmation for stores, fuel, access to waste facilities.  Vessel also requires information on pilots and tugs and other aspects for the transit.  VDES provides an opportunity for automated exchange of information to support these types of requests.

### Scenario - Logistic services – shore to ship

As the ship arrives, the shore authority will provide confirmation regarding offloading, loading of the vessel and respond to requests from the ship.  VDES provides an opportunity for automated exchange of information to support these types of requests.

# DEFINITIONS

The definitions of terms used in this Guideline can be found in the International Dictionary of Marine Aids to Navigation (IALA Dictionary) at http://www.iala-aism.org/wiki/dictionary and were checked as correct at the time of going to print.  Where conflict arises, the IALA Dictionary should be considered as the authoritative source of definitions used in IALA documents.

# ACRONYMS

| ACCSEAS | Accessibility for Shipping, Efficiency, Advantages and Sustainability (EU research project) |
| --- | --- |
| ACK | Acknowledgement |
| ACM | Address Complete Message |
| AIS | Automatic Identification System |
| AIS 1 | AIS Default Channel 1 - 161.975 MHz (Ch. 87B//2087) |
| AIS 2 | AIS Default Channel 2 - 162.025 MHz (Ch. 88B/2088) |
| App | Application |
| ASC | Assignment Channel |
| ASM | Application Specific Messages |
| BBM | Broadcast Binary Messages |
| BITE | Built in test equipment |
| CCTV | Closed-Circuit Television |
| Circ. | Circular (IMO document) |
| COMSAR | Sub-Committee on Communications and Search and Rescue (IMO) |
| DSC | Digital Selective Calling |
| ECDIS | Electronic Chart display & Information System |
| ENC | Electronic Navigation Chart |
| ETA | Estimated Time of Arrival |
| FAL | Facilitation Committee (IMO) |
| FEC | Forward error correction |
| GIS | Geographic Information System |
| GMDSS | Global Maritime Distress and Safety System |
| GMSK | Gaussian Minimum Shift Keying |
| GNSS | Global Navigation Satellite System |
| IAMSAR | International Aeronautical and Maritime Search and Rescue (manual) |
| IEC | International Electrotechnical Commission |
| IHO | International Hydrographic Organization |
| IMO | International Maritime Organization (UN) |
| INS | Information Service |
| ISPS | International Ship and Port Facility Security (Code) |
| ITU | International Telecommunication Union |
| ITU-R | International Telecommunication Union-Radiocommunication Sector |
| kbps | kilobits per second |
| kHz | Kilohertz |
| LPS | Local Port Service |
| LRIT | Long Range Identification & Tracking |
| MARPOL | International Convention for the prevention of pollution from ships (IMO 1973 as amended) |
| MAS | Maritime Assistance Service |
| MCS | Modulation and Coding Scheme |
| MEDEVAC | Medical evacuation |
| METAREA | Geographical sea region for the purpose of co-ordinating the transmission of meteorological information |
| MHz | Megahertz |
| MMSI | Maritime Mobile Service Identity |
| MSC | Maritime Safety Committee (IMO) |
| MSI | Maritime Safety Information |
| MSP | Maritime Service Portfolio(s) |
| NACK | Not Acknowledgement |
| NAS | Navigation Assistance Service |
| NAVAREA | Geographic area in which various governments are responsible for navigation and weather warnings |
| NAVTEX | Navigational Telex (service) |
| NCSR | Navigation, Communications and Search and Rescue (IMO Sub-Committee) |
| OSC | On-scene commander |
| PI | Presentation Interface |
| PSK | Phase-Shift Keying |
| QAM | Quadrature Amplitude Modulation |
| QPSK | Quadrature Phase-Shift Keying |
| RCC | Rescue Co-ordination Centre |
| RA | Random Access |
| RACH | Random Access Channel |
| RF | Radio Frequency |
| RR | Radio Regulations |
| RTCM | Radio Technical Commission for Maritime Service |
| SAR | Search and Rescue |
| SART | Search and Rescue Transponder |
| SAT | Satellite |
| Serv | Server |
| SIP | Strategic Implementation Plan |
| SOLAS | International Convention for the Safety of Life at Sea, (IMO 1974 as amended) |
| SRU | Search and Rescue Unit |
| TBB | Terrestrial Bulletin Board |
| TDMA | Time-division multiple access |
| TMAS | Telemedical Maritime Assistance Service |
| TOS | Traffic Organization Service |
| UDCH | User Data Channel |
| UTC | Co-ordinated Universal Time |
| VDE | VHF Data Exchange |
| VDES | VHF Data Exchange System |
| VDL | VHF Data Link |
| VHF | Very High Frequency (30 MHz to 300 MHz) |
| VOS | Voluntary observing ship |
| VTS | Vessel Traffic Service |
| WRC | World Radiocommunications Conference |
| WWRNS | World-wide Radionavigation System |

Technical Overview of VDES

Introduction:

The VHF Data Exchange System (VDES) extends the Automatic Identification System (AIS) as defined in the ITU-R M.1371 by adding new technology.  VDES is defined in ITU-R M.2092.

This technical overview assumes a basic knowledge of AIS and provides an overview of VDES.

VDES description:

VDES is a combination of technologies that includes AIS, ASM and VDE. VDE consists of a terrestrial (VDE-TER) and a satellite (VDE-SAT) part. The VDES Presentation Interface (PI) uses NMEA sentences, and is similarto  the AIS PI but will include several new IEC 61162-1 sentences to allow for the configuration of the VDES capability and facilitate the transfer of larger amounts of data.

The primary differences between AIS and the two new VDES technologies, ASM and VDE, are:

- New modern Modulation and Coding Scheme (MCS).

- New dedicated Radio Frequencies (RF) used the Radio Frequency bandwidth.

- Higher data bandwidth.

- New methods used by the Link Layer.

- ASM MCS is designed to improve reception on low-earth orbit satellite.

- VDE includes dedicated MCS and Link Layer for satellite uplink and downlink (VDE-SAT)

VDE has been designed to efficiently transfer relatively large volumes of arbitrary data. Unlike AIS, the definitions of the data being transferred is not defined by VDE. VDE only defines the data transfer mechanism. It can provide data transfer up to 32x higher when compared to AIS.

VDE has been designed to co-exist with AIS as to not interfere with AIS.

VDE-ASM is intended to move ASM traffic away from the existing AIS channels in order to improve safety at sea.

The Modulation and Coding Schemes

The modulation and coding schemes range from Gaussian Minimum Shift Keying (GMSK) for AIS to π/4 Quadrature Phase-Shift Keying (π/4QPSK) for ASM and π/4QPSK and 16 Quadrature Amplitude Modulation (16QAM) for VDE.

The Radio Frequency (RF) bandwidth on the technologies differ with AIS and ASM both having a RF bandwidth of 25kHz per channel and VDE having a bandwidth of 25kHz, 50kHz or 100kHz per channel.

AIS uses AIS1 and AIS2; ASM uses ASM1 and ASM2; and VDE uses VDE-TER lower leg and VDE-TER upper leg; VDE-SAT lower leg and VDE-SAT upper leg  (refer to table below).

AIS and ASM are simplex channels (can carry traffic in one direction at a time) and VDE-TER lower leg and VDE-TER upper leg allows for data to be transferred in both directions at the same time.

The radio frequencies used and bandwidth

At an overview level, the AIS and ASM use 2 X 25kHz channels. VDE-TER can use 2 X 25kHz; or 2 X 50 kHz or 2 X 100kHz or VDE-SAT can use 2 X 150kHz channels in the maritime VHF band. The allocated frequencies are as follows:

| Technology | Radio Frequencies used | Radio Regulations channel number |
| --- | --- | --- |
| AIS 1 | 161.975 (25kHz) | 87B |
| AIS 2 | 162.025 (25kHz) | 88B |
| AIS Long Range 1 | 156.775 (25kHz) (ships are TX only) | 75 |
| AIS Long range 2 | 156.825 (25kHz) (ships are TX only) | 76 |
| ASM 1 | 161.950 (25kHz) | 2027 |
| ASM 2 | 162.000 (25kHz) | 2028 |
| VDE-TER (lower leg) | 157.200 to 157.275 (100kHz) | 1024, 1084, 1025 and 1085 combined |
| VDE-TER (upper leg) | 161.800 to 161.875 (100kHz) | 2024, 2084, 2025 and 2085 combined |
| VDE-SAT (lower leg) | 157.200 to 157.325 (150kHz) | 1024, 1084, 1025, 1085, 1026 and 1086 combined |
| VDE-SAT (upper leg) | 161.800 to 161.925 (150kHz) | 2024, 2084, 2025, 2085, 2026 and 2086 combined |

Frequencies allocated to VDES

The data bandwidth

The modulation schemes π/4QPSK (ASM and VDE),  16QAM (VDE only) allow more data to be transferred in the same Radio Frequency (RF) bandwidth than was possible with AIS (GMSK).

The increase in data bandwidth is illustrated in Figure 5.

Increasing data bandwidth in VDES

Note: Figures shown reflect raw bandwidth.

The methods used by the Link Layer

The ASM and VDE technologies use the same AIS slot map of 2,250 slots per RF channel (4,500 slots for two RF channels used in parallel in AIS and ASM technologies).  Due to the larger data bandwidth resulting from the use of π/4QPSK modulation, the number of bits transferred in any one slot is increased in ASM when compared with AIS. VDE-TER can transfer up to 210.6 kbps (Link Id 19), and VDE-SAT has a maximum transfer rate of 47.75 kbps (downlink) (Link Id 29) and 94.70 kbps (uplink) (Link Id 20). The VDE frequencies, Modulation and Coding Scheme (MCS) are dynamic and are chosen by a combination of the Terrestrial Bulletin Board (TBB) (section A 4 refers) and by the Link Layer.

The Link Layers of AIS, ASM and VDE are different and each are optimised for the data bandwidth available.

The VDE Link Layer is more complex and uses a different Link Layer to facilitate communication between vessels (ship to ship) and between the shore and a vessel (shore to ship).

Forward Error Correction

ASM and VDE use Forward Error Correction (FEC) which allows for the correction of errors in the data transferred in both ASM and VDE without retransmissions, i.e. in broadcast messages. FEC thereby increases the integrity and reliability of the transmission, adapting to the maritime channel, optimizing its resources.

The Terrestrial and Satellite Bulletin Board

The VDE technology for terrestrial and satellite use a Terrestrial Bulletin Board (TBB) and Satellite Bulletin Board (SBB), respectively to assign the primary operating environment parameters to the Control Station Service Area. This includes which frequencies are being used and the service area dimensions amongst a range of other technical detail. To provide a level of protection to the VDES communication environment, the TBB may be authenticated.  Authentication confirms that the TBB is transmitted by a trusted entity.

VDE Channel resources for data transfers are managed by the Control Station. Ships are within a Control Station service area when they receive a valid TBB that also overlaps their position.

Logical channels in VDE-TER

Terrestrial VDE Channel resources are managed as logical channels. One of 5 logical channels are dynamically allocated (shown as logical channel 8, 9, 10, 11 and 12 in the diagram below) to carry the data in the User Data Channel (UDCH). The TBB (TBB), Assignment Channel (ASC) and Random Access Channel (RACH) are defined too ensure that the VDE channel is optimally used to carry user data.  Figure 6 provides an example of channel configuration.

A ship station requests access to a logical channel from the Control Station when it needs to transfer data over VDE. The logical channels have been designed in such a way that a VDES terminal can provide both VDE and AIS functionality without affecting AIS. Multiple terminals can simultaneously transfer using a TDMA access scheme that makes optimum use of the available resources.

The Random Access Channel are used to request resources and can also be used for small data messages allowing the User Data Channels to be available for larger messages.

The User Data Channel is allocated to the ship station for  a fixed period. The user can continue to request the resource for as long as it has data to transmit. This is done in an optimal way on the Data Channel without having to use the Random Access Channel again. The Control Station determines if the request is granted or not.

The VDE control station transmits the TBB at the start of a frame. Just as in AIS, the VDE frame is one minute in duration comprised of 2250 slots.

When a ship station is outside the service area of a control station, the ship can transmit data directly to another ship using ship to ship mode.  A ship it is out of the service area when it does not receive a valid TBB from a VDE Control Station for 15 minutes or the vessel is in a position outside of the service area of any received TBB.

Logical Channels IN VDE-SAT

Satellite VDE Channel resources are managed as logical channels to carry link layer and user data between the Control Station on the Satellite and the Ship Stations.

Logical channels are the Satellite Bulletin Board (SBB), Assignment Channel (ASC), Data Channels for broadcast and addressed data in up- and downlink direction, and Random Access Channel (RACH) are defined to ensure that the VDE channel is optimally used to carry user data.

A ship station requests access to a logical channel from the satellite Control Station when it needs to transfer data over VDE-SAT. Due to the power flux density limit chosen for satellite transmissions in ITU-R M.2092-1, VDE-SAT can be received on board of a ship without affecting AIS reception range. Multiple terminals can simultaneously transfer using a TDMA access scheme that makes optimum use of the available resources. TDMA limits ensure that ship transmissions do not interfere with the reception of AIS or DSC messages.

The Random Access Channel is used to request resources and can also be used for small data messages allowing the User Data Channels to be available for larger messages.

A User Data Channel is allocated to a ship station for a fixed period. The user can continue to request the resource for as long as it has data to transmit. This is done in an optimal way on the Data Channel without having to use the Random Access Channel again. The Control Station determines if the request is granted or not.

The VDE-SAT control station transmits the SBB at the start of a frame. Just as in AIS, the VDE frame is one minute in duration comprised of 2250 slots.

An orthogonalization technique is used to distinguish transmissions from different VDE satellites, should these happen simultaneously. Transmissions from ships are always directed to a specific VDE satellite to avoid ambiguity to which satellite a ship intents to transmit a message.

VDES Ship and Control station

A VDES ship station consists of (at least) 6 radio receivers.  There are 2 AIS receivers, 2 ASM receivers and 6 or more VDE receivers.  AIS, ASM and VDE all share the same transmitter. Every transmitter and receiver is connected to the VDES controller that manages the radio protocol. The VDES controller takes its input from the PI, and outputs received radio messages from all the receivers on the same interface.  GNSS for positioning and timing is done in the same way as for AIS. A typical block diagram of a VDES ship station is shown in [Figure 7].

VDES ship station logical description

A VDES control station used for ASM, VDE-TER or VDE-SAT consists of the same logical components as the ship station, but has differences in the PI, Link Layer functionality and the antenna system (the control station allows for connection to a more complex antenna system by separating the receive and transmit paths to the antenna system). A VDES control station in VDE-TER and VDE-SAT controls the resources within its service area and a ship shall request resources from an area’s control station before using resources. A typical block diagram of a VDES control station is shown in [Figure 8].

VDES shore station logical description (for duplex mode, simplex mode is allowed)

The VDES fit into the Common Shore Side Architecture (IALA Guideline 1113) in the same manner as does AIS as illustrated below.

VDES into the Common Shore Side Architecture

Example transaction between ship and shore

The transfer starts with the ship station requesting VDE Control Station resources. The VDE Control Station allocates an exclusive Logical Channel for the transfer. The ship station sends then multiple data fragments that are acknowledged by the VDE Control Station. Fragments that are not acknowledged are to be resent.

Ship control station

SatelLite

ASM overview

Channel access schemes

The access schemes for ASM transmission are MITDMA, RATDMA, and FATDMA. The transmissions slots are typically selected when it's time to transmit an ASM burst, using a rule set that prioritizes the reception of AIS messages and avoids conflict with AIS transmissions. These transmissions may use MITDMA communication states to reserve future transmission slots, thus improving the reliability of the reception of an ASM message by avoiding transmission collisions. A single MITDMA transmission may be used to schedule up to three future transmissions, with each transmission occupying up to three slots. A total of 15 MITDMA transmissions may be chained together.

Data link congestion resolution

A number of rules are in place to address congestions when the ASM channels are heavily loaded. The maximum` number of slots allocated by one station on one channel shall not exceed 50 slots over a period of one minute (2.2% duty cycle). After the completion of a single ASM channel transmission or a complete MITDMA transmission chain, the ASM station shall wait for a specific time before additional transmission can be scheduled. As channel loading goes up, the quiet time's length increases.

ASM messages

The ASM messages listed in the table below will support various applications. ASM messages may use different addressing modes, including individually addressed, geographically addressed, and broadcast. Depending on the environment and application, these messages may or may not utilize FEC. In addition, these messages may or may not use MITDMA communication states.

Message summary

| Message ID | Name | Description | Access scheme | Communication state |
| --- | --- | --- | --- | --- |
| 0 | Broadcast AIS ASM Message | Encapsulated AIS ASM messages. | RATDMA | None |
| 1 | Scheduled Broadcast Message | Broadcast data using communication state. | FATDMA RATDMA MITDMA | MITDMA |
| 2 | Broadcast Message | Broadcast data with no communication state. | FATDMA RATDMA | None |
| 3 | Scheduled Individual Addressed Message | Individual addressed data with communication state. Requires acknowledgement. | FATDMA RATDMA MITDMA | MITDMA |
| 4 | Individual Addressed Message | Individual addressed data with no communication state. Requires acknowledgement. | FATDMA RATDMA | None |
| 5 | Acknowledgment Message | This message is used to provide and acknowledgment for one or more addressed messages. | FATMDA RATDMA MITDMA | None |
| 6 | Geographical Multicast Message | Addressed to a group of stations defined by their geographical location with no communication state. No acknowledgment required. | FATDMA RATDMA | None |

Definition of system related international VDE function messages

IVFM 0: Text using 6-bit ASCII

This IVFM shall be used by applications that use VDE stations to transfer 6-bit ASCII encoded text. The text can be sent over VDE-TER or VDE-SAT as broadcast or addressed message. The parameter, “acknowledge required flag”, should be set to 0 when broadcast is used.

When long text strings are sub-divided, an 11-bit “text sequence number” is used. The text sequence number is used by the originating application to sub-divide the text and by the recipient application to re-assemble the text. The text sequence numbers for each sub-division should be selected to be contiguous and always increasing (110, 111, 112, …). If longer text strings are being transferred, the text sequence numbers should be chosen to associate correctly the sub-divided text with the correct text strings.

For international compatible text, IVFM 1 and IVFM 2 shall be used.

If the application requires authentication and encryption, see IVFM 1 and 2.

Table G.1 – IVFM 0: Text in 6-bit ASCII encoding

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 0. Version = 0 |
| Acknowledge required flag | 1 | 1 = reply is required, optional for addressed messages and not used for broadcast messages 0 = reply is not required, optional for an addressed message and required for broadcast messages |
| Sequence number | 2 | 0 – 3; see ITU-R M.1371-5, § 5.3.1, Annex 2 |
| Text sequence number | 11 | Sequence number to be incremented by the application. All zeros indicates that sequence numbers are not being used |
| Last text in sequence | 1 | 0: more text will follow 1: this is the last message in a sequence |
| Text string | VDE-TER: 6-6000 VDE-SAT: 6-60000 | 6-bit ASCII as defined in ITU-R M.1371-5, Table 47, Annex 8. |
| Spare bits | 0-7 | Byte boundary padding, value of the bits is set to 0; |
| Total number of application data bits | VDE-TER:TBD VDE-SAT:TBD |  |

IVFM 1: Text using UTF-8 encoding

This IVFM shall be used by applications that use VDE stations to transfer UTF-8 encoded text. The text can be sent over VDE-TER or VDE-SAT as broadcast or addressed message. The parameter, “acknowledge required flag”, should be set to 0 when broadcast is used.

When long text strings are sub-divided, an 11-bit “text sequence number” is used. The text sequence number is used by the originating application to sub-divide the text and by the recipient application to re-assemble the text. The text sequence numbers for each sub-division should be selected to be contiguous and always increasing (110, 111, 112, …). If long text strings are being transferred, the text sequence numbers should be chosen to associate correctly the sub-divided text with the correct text strings.

If the application requires encryption, see IVFM 2.

Table G.2 – IVFM 1: Text in UTF-8 encoding

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 1. Version = 0 |
| Acknowledge required flag | 1 | 1 = reply is required, optional for addressed messages and not used for broadcast messages 0 = reply is not required, optional for an addressed message and required for broadcast messages |
| Sequence number | 2 | 0 – 3; see ITU-R M.1371-5, § 5.3.1, Annex 2 |
| Text sequence number | 11 | Sequence number to be incremented by the application. All zeros indicates that sequence numbers are not being used |
| Last text in sequence | 1 | 0: more text will follow 1: this is the last message in a sequence |
| Spare bits | 3 | Byte boundary padding, bit values set to 0; |
| Text string | VDE-TER: 8-8000 VDE-SAT: 8-80000 | UTF-8 encoded text (see The Unicode Standard, Version 11.0.0, 2018). |
| Valid until | 32 | Unsigned integer number of seconds after the 1st of January 1970 00:00:00 UTC. After the 19th of January 2038 03:14:07 UTC, the number is the number of seconds after this new date (wraps around); To avoid replay attacks, this value shall be used by the receiving application to verify the validity of the data together with the signature field. |
| Signature | 512 | Signature over above ASM data, applied over all bits from the start of the VAI field to the end of the last ASM, generated as described for the terrestrial bulletin board, see ITU-R M.2092-1, Annex 4, §4.15 |
| Total number of application data bits | VDE-TER:TBD VDE-SAT:TBD |  |

IVFM 2: Text using UTF-8 encoding, encrypted

This IVFM shall be used by applications that use VDE stations to transfer UTF-8 encoded text between applications that require encryption to provide privacy. The text can be sent over VDE-TER or VDE-SAT as broadcast or addressed message. The parameter, “acknowledge required flag”, should be set to 0 when broadcast is used.

When long text strings are sub-divided, an 11-bit “text sequence number” is used. The text sequence number is used by the originating application to sub-divide the text and by the recipient application to re-assemble the text. The text sequence numbers for each sub-division should be selected to be contiguous and always increasing (110, 111, 112, …). If long text strings are being transferred, the text sequence numbers should be chosen to associate correctly the sub-divided text with the correct text strings.

Encryption is applied after forming the format described in Table G.3.

[Include description on how to do encryption]

Table G.3 – IVFM 2: Text in UTF-8 encoding, encrypted content

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 2. Version = 0 |
| Acknowledge required flag | 1 | 1 = reply is required, optional for addressed messages and not used for broadcast messages 0 = reply is not required, optional for an addressed message and required for broadcast messages |
| Sequence number | 2 | 0 – 3; see ITU-R M.1371-5, § 5.3.1, Annex 2 |
| Text sequence number | 11 | Sequence number to be incremented by the application. All zeros indicates that sequence numbers are not being used |
| Last text in sequence | 1 | 0: more text will follow 1: this is the last message in a sequence |
| Spare bits | 3 | Byte boundary padding, bit values set to 0; |
| Text string | VDE-TER: 8-8000 VDE-SAT: 8-80000 | UTF-8 encoded text (see The Unicode Standard, Version 11.0.0, 2018). |
| Valid until | 32 | Unsigned integer number of seconds after the 1st of January 1970 00:00:00 UTC. After the 19th of January 2038 03:14:07 UTC, the number is the number of seconds after this new date (wraps around); To avoid replay attacks, this value shall be used by the receiving application to verify the validity of the data together with the signature field. |
| Signature | 512 | Signature over above ASM data, applied over all bits from the start of the VAI field to the end of the last ASM, generated as described for the terrestrial bulletin board, see ITU-R M.2092-1, Annex 4, §4.15 |
| Total number of application data bits | VDE-TER:TBD VDE-SAT:TBD |  |

IVFM 3: Capability interrogation

This IVFM shall be used by an application to interrogate another application for the availability of VDE application identifiers for the specified VAIs. One to 255 VAI can be interrogated with one request at a time. The request is made separately for each VAI.

This IVFM can only be used as the data content of an addressed message.

Table G.4 – IVMF 3: Capability interrogation

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 3. Version = 0 |
| n | 8 | number of VAIs requested in this interrogation 0: unused value 1-255: valid values |
| Requested VAI [0] | 40 | See §4.3.11.2 |
| Requested VAI [1] | 40 | See §4.3.11.2 |
| … | … | … |
| Valid until | 32 | Unsigned integer number of seconds after the 1st of January 1970 00:00:00 UTC. After the 19th of January 2038 03:14:07 UTC, the number is the number of seconds after this new date (wraps around); To avoid replay attacks, this value shall be used by the receiving application to verify the validity of the data together with the signature field. |
| Signature | 512 | Signature over above ASM data, applied over all bits from the start of the VAI field to the end of the last ASM, generated as described for the terrestrial bulletin board, see ITU-R M.2092-1, Annex 4, §4.15 |
| Total number of bits | TBD |  |

IVFM 4: Capability reply

This IVFM should be used by an application to reply to a capability interrogation (IVFM 2) VDE function message. The reply contains the availability status of the application for each function identifier for the specified VAI.

The application should use an addressed message to reply to the interrogating application.

Table G.5 – IVMF 4: Capability reply

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 4. Version = 0 |
| n | 8 | number of VAIs requested in this interrogation 0: unused value 1-255: valid values |
| Interrogated VAI availability [0] | 2 | The interrogated VAI is: 0: not available 1: available in requested version 2: available in other version 3: reserved for future use. |
| Interrogated VFI version [0] | 8 | Version available: 0-255 |
| Interrogated VAI availability [1] | 2 | The interrogated VAI is: 0: not available 1: available in requested version 2: available in other version 3: reserved for future use. |
| Interrogated VFI version [1] | 8 | Version available: 0-255 |
| … | … | … (for all n interrogated VAI) |
| Valid until | 32 | Unsigned integer number of seconds after the 1st of January 1970 00:00:00 UTC. After the 19th of January 2038 03:14:07 UTC, the number is the number of seconds after this new date (wraps around); To avoid replay attacks, this value shall be used by the receiving application to verify the validity of the data together with the signature field. |
| Signature | 512 | Signature over above ASM data, applied over all bits from the start of the VAI field to the end of the last ASM, generated as described for the terrestrial bulletin board, see ITU-R M.2092-1, Annex 4, §4.15 |
| Total number of bits | TBD |  |

IVFM 5: Application acknowledgement to an addressed message

When requested, this IVFM should be used by an application to confirm the reception and processing of an addressed message. An application should never acknowledge a broadcast message.

Manual acknowledgement by human interaction is not covered by this mechanism.

If the interrogating application does not receive this IVFM even though it was requested, then the application should assume that addressed VDE until does not have the required application attached to its Presentation Interface (PI).

If there is any application at the VDE station, it should not respond if the “Acknowledge Required Flag” is set to 0.

The application shall respond with this message within 1 second after receiving the addressed message to be acknowledged.

Table G.6 – IVFM 5: Application acknowledgement

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 5. Version = 0 |
| Received VAI | 40 | ? |
| Sequence number | 8 | 0-3; see ITU-R M.1371-5 § 5.3.1, Annex 2 |
| Text sequence number | 11 | Sequence number in the message being acknowledged as properly received 0 = default (no sequence number) 1-2 047 = sequence number of received VFM |
| VAI available | 1 | 0 = received but VAI not available 1 = VAI available |
| VAI response | 3 | 0 = unable to respond 1 = reception acknowledged 2 = response to follow 3 = able to respond but currently inhibited 4-7 = spare for future use |
| Spare bits | 1 | Not used, should be set to zero, reserved for future use |
| Total number of bits | TBD |  |

IVFM 6: Virtual AtoN

This IVFM shall be used to distribute one or multiple Virtual AtoN to be output by the equipment via legacy PI VDL sentences to be able to use the enhanced capacity of VDE-TER and VDE-SAT in combination with legacy application equipment connected to the PI interface.

Table G.7 – IVFM 7: Virtual AtoN

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 6. Version = 0 |
| Source ID | 30 | Identity of the virtual AtoN station (see Article 19 of the RR and Recommendation ITU R M.585) |
| Type of aids-to-navigation | 5 | 0 = not available = default; refer to appropriate definition set up by IALA; |
| Longitude | 28 | Longitude in 1/10 000 min of position of an AtoN (180°, East = positive, West = negative 181 = (6791AC0h) = not available = default |
| Latitude | 27 | Latitude in 1/10 000 min of an AtoN (90°, North = positive, South = negative 91 = (3412140h) = not available = default |
| AtoN status | 8 | Reserved for the indication of the AtoN status [add reference to the IALA guideline] 00000000 = default |
| Virtual AtoN flag | 1 | 0 = default = physical AtoN at indicated position; 1 = virtual AtoN, does not physically exist. |
| Cancellation | 1 | 0 (default) = station is valid and should be stored for VDM PI output until the timeout has passed or until the storage limit is reached. 1 = remove any stored station with matching Source ID and Lat and Long fields. |
| Name of Aids-to-Navigation | 6-204 | Maximum 34 characters 6-bit ASCII “@@@@@@@@@@@@@@@@@@@@” = not available = default. |
| Spare bits | 0-7 | Spare, not used. Should be set to zero. Padding to ensure complete message is complete bytes |
| Valid until | 32 | Unsigned integer number of seconds after the 1st of January 1970 00:00:00 UTC. After the 19th of January 2038 03:14:07 UTC, the number is the number of seconds after this new date (wraps around); To avoid replay attacks, this value shall be used by the receiving application to verify the validity of the data together with the signature field. |  |
| Signature | 512 | Signature over above ASM data, applied over all bits from the start of the VAI field to the end of the last ASM, generated as described for the terrestrial bulletin board, see ITU-R M.2092-1, Annex 4, §4.15 |  |
| Number of bits | TBD | Does not include “Message ID” (the first parameter) |

IVFM 7: ASM Message

This IVFM may be used to embed multiple ASM FM messages using the ASM standard header, in order to utilize the capabilities of VDE-TER and VDE-SAT.

Table G.7 – IVFM 7: ASM Message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 7. Version = 0 |
| n | 8 | number of encapsulated ASMs 0: unused value 1-255: valid values |
| Length of ASM [0] |  |  |
| ASM data [0] (first ASM) |  | 16 bit: Application Identifier[0], see ITU-R M.1371-5, Annex 5 m[0] bits: ASM Data Content [0] |
| Length of ASM [1] |  |  |
| ASM data [1] (second ASM) |  | 16 bit: Application Identifier[1], see ITU-R M.1371-5, Annex 5 m[0] bits: ASM Data Content [1] |
| … | … | … |
| Length of ASM [n] |  |  |
| ASM data [n-1] (last ASM) |  | 16 bit: Application Identifier[n-1], see ITU-R M.1371-5, Annex 5 m[0] bits: ASM Data Content [n-1] |
| Valid until | 32 | Unsigned integer number of seconds after the 1st of January 1970 00:00:00 UTC. After the 19th of January 2038 03:14:07 UTC, the number is the number of seconds after this new date (wraps around); To avoid replay attacks, this value shall be used by the receiving application to verify the validity of the data. |
| Signature | 512 | Signature over above ASM data, applied over all bits from the start of the VAI field to the end of the last ASM, generated as described for the terrestrial bulletin board, see ITU-R M.2092-1, Annex 4, §4.15 |
| Total number of bits | Variable size |  |

IVFM 8: Maritime Messaging System (MMS) message

This IVFM is used for MMS transport over VDE-TER and VDE-SAT, in order to utilized the Maritime Messaging Service as defined by the Maritime Connectivity Consortium. [Add reference when available]

Table G.8 – IVFM 8: Maritime Messaging System (MMS) message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 8. Version = 0 |
| MMS Data | Variable size | See [TBD] |
| Total number of bits | Variable size |  |

IVFM 9: VDE-SAT Network Orbit Data

This IVFM is used by VDE Satellites to inform about updated orbital data for a set of VDE-SAT satellites in a VDE-SAT network or VDE-SAT roaming network. Equipment uses the orbital data to estimate when connectivity to a satellite can be expected.

Orbital data is represented in the two line element (TLE) format, as specified in [TBD].

Equipment shall cache received TLE data organized by primary network ID and satellite ID.

Equipment shall update cached TLE data upon reception of validated new data.

Mobile stations may use that information to estimate the time when a satellite network may be available.

Table G.9 – IVFM 9: VDE-SAT Network Orbit Data

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 9. Version = 0 |
| Primary network ID | 8 | Values 0-255 allowed, see ITU-R M.2092-1, Annex 5, §3.1.9 |
| Roaming network ID | 8 | Identifying which roaming network group the satellites of this list belong to. |
| n | 8 | Number of orbital data sets transferred in this IVFM |
| Satellite ID[0] | 8 | ID [0] of the satellite |
| TLE [0] | 828 | TLE data[0] for the first satellite: 2 lines of 69 ASCII 6-bit encoded characters |
| Satellite ID[1] | 8 | ID [1] of the satellite |
| TLE [1] | 828 | TLE data[1] for the first satellite: 2 lines of 69 ASCII 6-bit encoded characters |
| … | … | … |
| Satellite ID[n-1] | 8 | ID [n-1] of the satellite |
| TLE [n-1] | 828 | TLE data[n-1] for the first satellite: 2 lines of 69 ASCII 6-bit encoded characters |
| Valid until | 32 | Unsigned integer number of seconds after the 1st of January 1970 00:00:00 UTC. After the 19th of January 2038 03:14:07 UTC, the number is the number of seconds after this new date (wraps around); To avoid replay attacks, this value shall be used by the receiven application to verify the validity of the data. |
| Signature | 512 | Signature over above data, applied over all bits from the start of the VAI field to the end of the last TLE, generated as described for the terrestrial bulletin board, see ITU-R M.2092-1, Annex 4, §4.15 |
| Total number of bits | TBD |  |

IVFM 10: VDE-SAT AIS Position Report Retransmit

This IVFM shall be used by VDE Satellites that optionally retransmit AIS position reports (AIS Msg. 1, 2, 3) in remote areas as received to give better situational awareness for navigation.

When output by the receiving equipment to the PI as VDM, the repeat indicator of the retransmitted AIS position report shall be set to the value 3, and the com state shall be set to all 0. If the receiving equipment has a more recent AIS position report from an MMSI it shall discard the retransmitted reports for this MMSI .

The satellite shall only repeat AIS position reports that contain a repeat indicator value of 0.

The signature authenticates the authority who can stand for the reception of the AIS data and retransmission, not for the authenticity of the single AIS data, however.

VDE-SAT capable equipment shall provide means, to configure the radius and maximum age of such received AIS messages, that are output over PI to legacy equipment using the VDL sentence.

Table G.10 – IVFM 10: VDE-SAT standard AIS Position Report Retransmit

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 10. Version = 0 |
| n | 8 | number of encapsulated AIS position reports 0: unused value 1-255: valid values |
| Time [0] | 40 | UTC year, month, day and hours, minutes, seconds for the first AIS Message reception, as defined in ITU-R M.1371-5, Message ID 11, starting with year, ending with seconds. |
| AIS Msg [0] | 168 | AIS Message [0]: first retransmitted message |
| Time [1] | 40 | UTC year, month, day and hours, minutes, seconds for the second AIS Message reception, as defined in ITU-R M.1371-5, Message ID 11, starting with year, ending with seconds. |
| AIS Msg [1] | 168 | AIS Message [1]: second retransmitted message |
| … | … | … |
| Time [n-1] | 40 | UTC year, month, day and hours, minutes, seconds for the n-1th AIS Message reception, as defined in ITU-R M.1371-5, Message ID 11, starting with year, ending with seconds. |
| AIS Msg [n-1] | 168 | AIS Message [n-1]: n-th retransmitted message |
| Valid until | 32 | Unsigned integer number of seconds after the 1st of January 1970 00:00:00 UTC. After the 19th of January 2038 03:14:07 UTC, the number is the number of seconds after this new date (wraps around); To avoid replay attacks, this value shall be used by the receiving application to verify the validity of the data. |
| Signature | 512 | Signature over above ASM data, applied over all bits from the start of the VAI field to the end of the last ASM, generated as described for the terrestrial bulletin board, see ITU-R M.2092-1, Annex 4, §4.15 |
| Total number of bits | TBD |  |

IVFM 11: VDE-TER AIS or ASM Message Authentication

This IVFM shall be used by equipment transmitting an authentication for a message previously sent via the AIS or ASM channels. The AIS or ASM messages may not be older than 59 seconds when the authentication message is transmitted. The transmission may be triggered by interrogation from a coast station or be scheduled by the authenticated message.

The AIS or ASM channel content is not repeated but used to generate a signature for authentication by receiving equipment.

In order to allow receiving equipment to link the authentication message with the authenticated AIS or ASM message the message ID, MMSI, channel ID, slot number and time stamp is provided in the authentication message.

Equipment receiving this IVFM shall output the message as VDE message at the PI.

Applications may use the VDE message to augment received AIS messages on shore and ship.

Table G.11 – IVFM 11: VDE-TER AIS or ASM Authentication message

| Parameter | Number of bits | Description |
| --- | --- | --- |
| VAI | 40 | Registry Holder = 1. VFI = 11. Version = 0 |
| AIS Message ID | 6 | 1, 2, 3, … 27 |
| MMSI | 30 | Ref. 1371-5 |
| Channel ID | 2 | Channel where the authenticated transmission occurred: 0: AIS 1 1: AIS 2 2: ASM 1 3: ASM 2 |
| Slot number | 12 | Slot number value 0 .. 2249, for the authenticated message transmission slot, on the specified channel, as defined in ITU-R M.2092-1. |
| Time stamp | 32 | Unsigned integer number of seconds after the 1st of January 1970 00:00:00 UTC. After the 19th of January 2038 03:14:07 UTC, the number is the number of seconds after this new date (wraps around); Set to the time when the authenticated message was transmitted. To avoid replay attacks, this value shall be used by the receiven application to verify the validity of the data |
| Signature | 512 | Signature over above data, applied over all bits from the start of the VAI field to the end of the last AIS, generated as described for the terrestrial bulletin board, see ITU-R M.2092-1, Annex 4, §4.15 |
| Total number of bits | TBD |  |

Recommendation ITU-R M.493

Recommendation ITU-R M.1371

Recommendation ITU-R M.1842

More information on VDL loading is available in Report ITU-R M.2287-0

IEC standard 61174 Ed. 4 includes route exchange.

GNSS augmentation information today is able to reach different levels of accuracy e.g. submeter, subdecimeter with technologies like RTK and PPP (e.g. Galileo HAS). https://www.gsc-europa.eu/electronic-library/programme-reference-documents#ACCURACY

MSC.1/Circular.1575 - Guidelines for Shipborne Position, Navigation And Timing (PNT) Data Processing – (16 June 2017) - Annex - Guidelines for Shipborne Position, Navigation and Timing (PNT) Data Processing - Module B – Functional aspects (https://www.imorules.com/GUID-7E17E675-A69B-46B1-A58F-7C40EC9AC7BB.html)

https://wwwcdn.imo.org/localresources/en/OurWork/Security/Documents/ANNEX%20Guidelines%20on%20Cyber%20Security%20Onboard%20Ships%20v.4.pdf