| From: VTS Committee | VTS32/output/13 |
| --- | --- |
| To: e-Navigation Committee | 11 March 2011 |

Liaison Note on the revised “Maritime Information Systems (MIS): Scope and Focus” document

# Introduction

This draft working paper (VTS32/14/6) was submitted to the VTS Committee for discussion.  A number of comments regarding systems and equipment were made in Annex A (Glossary of e-Navigation-related Equipment, Systems, and Services) attached.

# Action requested

The e-Navigation Committee is requested to take note of the comments made.

ANNEX A - Glossary of E-Navigation-related Equipment, Systems, and Services

Scope/goal: 	Describe the variety of shore-based and shipborne marine navigation systems that fall under the e-Navigation concept of operation.

Shore-based

.

Radar/ARPA (Automatic Radar Plotting Aid) is a computer assisted radar data processing system. The system creates tracks using radar contacts to calculate the objects course, speed and CPA to reduce danger of collisions.

AIS (Automatic Identification System) is a system which automatically provides other ships and coastal authority with information about the ship’s dynamic (e.g., speed), static (e.g., identity), and voyage-related (e.g., destination) via designated VHF channels.

ECDIS  (Electronic Chart Display and Information System) is a computer-based navigation information system which may be used in lieu of paper nautical charts. This shipborne navigational display continuously provides the ships position on an electronic chart. The position is provided by the ships Integrated Navigation System (INS)  with the aid GPS.  An ECDIS is required to meet specifications set by  IMO.

CCTV (Closed-Circuit Television) is the use of video cameras and associated control equipment (pan, zoom and tilt) to monitor a given area and display the video on a specified network of monitors.

VHF Voice Radio - ?

VHF DSC]

System

LRIT (Long Range Identification and Tracking system – EMSA definition?). There are number of  distinctions between LRIT and AIS :  range, frequency of updates, data content and communications protocol.  LRIT  will only be available to recipients who are entitled to receive such information.

SSN (SafeSeaNet – EMSA definition?) is an EU mandated  maritime data sharing and exchange system for all ships approaching and departing  European ports.

GMDSS (Global Maritime Distress and Safety System) is based on an IMO mandated  agreement regarding equipment carriage requirements , shore infrastructure, provision of maritime safety information and Search and Rescue operations.

GNSS (Global Navigation Satellite System) is the standard generic term for all global satellite navigation systems. The only fully functional GNSS in the world is NAVSTAR GPS (Global Positioning System), developed and operated by United States. Differential GPS  enhances the accuracy of GPS  and  may improve accuracy to the order of 1 meter.

Service

Navigational Warnings (such as unlit lights, drifting objects wrecks, etc.)

VTS (Vessel Traffic Service) provides information service, navigational assistant’s service, and    traffic organization service. Immediately might the role of VTS change and expand if it should be incorporated in a concept on a global level, Vessel Traffic Management.

MRCC (Maritime Rescue Coordination Center) coordinates SAR operations.

Oil polluting response.

Computer systems such as Dynamic MaRA. Dynamic MaRA is used in specific areas with a high environmental vulnerability and difficult weather conditions.

Maritime Coastal Radio stations coordinate information between ship and shore.

Weather Service (e.g., meteorlogic/hydrographic information including wind, wave, current flow, visibility, ice, etc.).

Sea Ice (ice coverage, types of ice, icebergs, etc.)

Search and Rescue is a service that is usually provided by MRCCs.

Shipborne

Equipment

Definition - Navigational devices, sensors or equipment that are directly related to providing information that can be used to enhance navigation and decision-making onboard vessels.

Radar/ARPA – Nick Ward

Satellite-based (e.g., GNSS) – Nick Ward

Terrestrial (e.g., eLORAN) – Nick Ward

ECDIS/ECS – Lee Alexander and Michael Bergmann

AIS [check with AIS WG]

- e.g., AIS Binary Messages

System [need definition]

INS/IBS – [check with Florian Motz]

Service	  [use what is listed in NAV54/WP.2, Annex 1, Section 8 (MIS)

Maritime Information Systems [& Services]:

8.1 SOLAS chapter V, regulation 13, requires Contracting Governments to provide navigational information to mariners. Regulation 13, paragraph 3 states: “Contracting Governments undertake to arrange for information relating to aids-to-navigation to be made available to all concerned. Changes in the transmissions [i.e., status report] of position-fixing systems which could adversely affect the performance of receivers fitted in ships shall be avoided as far as possible and only be effected after timely and adequate notice has been promulgated.”

8.2 The promulgation of information on navigational safety is coordinated by means of the World-Wide Navigational Warning Service that was established jointly by IMO and IHO in 1977.

8.3 The World-Wide Navigational Warning Service is administered through 21 NAVAREAS5. Each NAVAREA has an Area Coordinator who is responsible for collecting information, analyzing it, and transmitting NAVAREA Warnings.

8.4 Lists of aids to navigation are produced by (or for) most national authorities as part of the navigational information made available to mariners in support of SOLAS chapter V, regulation 13.

8.5 The joint IMO/IHO/WMO Manual on Maritime Safety Information (IMO COMSAR/Circ.15) provides definitions of standard terms to describe particular events that should be used when composing navigational warnings. The terms that are relevant to the condition of aids to navigation are defined in the circular.

Processes

Marine Information Overlays (MIOs) - Lee Alexander

- static (	IHO S-57  IHO S-100)

- dynamic (AIS Binary Messages)

Types of e-Navigation Related Data and Information

[Add section describing how the (U)MDM could be used]

It is important to make a clear distinction between “information” and “data.”  “Information” includes voice, text, fax, graphical, geo-spatial (map or chart), etc.  If/when this information is converted into another format for digital transmission, it becomes “data” (e.g., NMEA 0183 or AIS binary message).  However, to be relevant to a mariner onboard a vessel this data must eventually be converted back into a presentation/display of “information” that can be used for situational awareness or decision-support.

Shipborne

Electronic chart data

vector (e.g., IHO S-57 ENCs, SENC Direct Distribution)

raster (e.g., UKHO Admiralty Raster Chart Service, or ARCS)

MIOs (Marine Information Overlays)

Static (e.g., IHO S-57)

Dynamic (e.g., AIS Binary Messages)