AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

ANM11/WG1/WP5

IALA Recommendation O-104

On

“Off Station” Signals for Major Floating Aids

Edition 1

May 1998

20 ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 01 Telefax +33 1 34 51 82 05

E-mail : iala-aism@wanadoo.fr      		Internet : http://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| July 2005 | Entire document reformatted | Reformatting to meet IALA documentation standards |

Recommendation on “Off Station” Signals For Major Floating Aids

(IALA Recommendation O-104, April, 2008)

THE COUNCIL:

RECOGNISING that manned lightships have been phased our worldwide and that many have been converted to automatic operation or replaced;

HAVING CONSIDERED that Superbuoys and Buoys of Primary Navigation Significance (BPNS) have replaced Lightvessels and Lanbys

HAVING CONSIDERED that there have been considerable advances in remote control and monitoring systems

HAVING CONSIDERED that there is a need to prescribe signals which can reasonably be deployed by automatic means by Light Vessels (LtV), Light Floats (LF) and Lanbys (LNB’s) which have dragged or broken adrift from their moorings;

HAVING CONSIDERED the proposals by the IALA ANM Committee;

RECOMMENDS:

- That when any Lightvessel, Light Float or Lanby, is out of position and in order to alert mariners that the AtoN is off station and that there is a consequent danger of navigational error or collision:

- Its Aids to Navigation consisting of Lights, Sound signals, Racon, should be discontinued.

- Warning lights consisting of two all round red lights in vertical line similar to those prescribed by Rule 27 of the COLREGS for a vessel “Not under command” should be displayed.

- AIS be fitted which will indicate in Message type 21 that the AtoN is “off station”.

- The National Authority may also initiate a safety related message by broadcasting a type 14 and/or type12 AIS message.

- If the National Authority requires a Racon signal when off station, it should be coded MORSE “D”.

- That if Superbuoys and Buoys of Primary Navigation Significance are deployed, National Authorities should, if practicable, consider provision of systems as detailed in section 2 above.

- That in any cases where AtoNs are off station, even if fitted with the off station signals described above, Radio Navigation Warnings should be issued to alert mariners.

Annex 1

Definitions:

Lightvessel		Lightvessels are ship shaped vessels.

Lightfloat		Lightfloats

Lanby (LNB)	Lanbys are a particular type of large buoy, generally considered to be over 8metres in diameter.

Superbuoy (SB)	Superbuoys are modern buoys, fitted with

Buoy of Primary Navigational Significance (BPNS) These are designated important buoys, such as Fairway, Turning, Safewater