AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Guideline No. XXXX

On

The establishment of AIS as an Aids to Navigation

Edition 1/3

[Date]

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  http://iala-aism.org

Table of Contents

1	General	3

2	Objective	3

2.1	Scope	4

3	Introduction	4

4	Definitions of AIS AtoN	5

4.1	Real	5

4.2	Synthetic	6

4.2.1	Monitored	6

4.2.2	Predicted	6

4.3	Virtual	6

5	Accessing the VDL	7

6	Application of AIS AtoN to assist navigation	7

6.1	Categorization of Application Area	7

6.2	Elements to be considered	7

6.3	AIS AtoN station assessment criteria	8

6.3.1	Lighthouses and Beacons	8

6.3.2	Buoys and Major Floating Aids	8

6.3.3	Virtual AtoN	8

7	Additional applications of AIS AtoN	9

7.1	Monitoring	9

7.2	Data Collection	9

7.3	Networking	9

7.4	Criteria	9

8	. AIS AtoN message use	10

9	Display of AIS AtoN	10

9.1	On SOLAS vessels	11

9.2	On non-SOLAS vessels	11

9.3	Shore system	11

10	Conclusion	11

11	REFERENCES	12

# General

IALA Recommendation A-126 on the Use of the Automatic Identification System (AIS) in Marine Aids to Navigation provides detailed information on the type of AIS AtoN services that may be provided.  The Recommendation states that an AIS transponder could provide information and data that could:

- Be used as an aid to navigation;

- Complement existing aids to navigation;

- Monitor the performance of aids to navigation;

- Monitor the ‘on station’ position of floating aids to navigation;

- Provide identify, state of ‘health’ and other navigational information such as meteorological and hydrographical data, if available to ships and shore authorities; and

- Be used to assess traffic type and patterns to assist in providing the appropriate level of service and mix of aids to navigation.

Further, IALA A-126 recommends that ‘National Members and other appropriate authorities providing marine aids to navigation services, use appropriate AIS units as part of their marine aid to navigation services for the provision of information data to shipping and monitoring purposes.  In this manner, IALA has recognised that AIS can be applied to AtoN to further improve and enhance services to mariners as well as assisting AtoN authorities in ensuring that the provision of aids to navigation to as the volume of traffic justifies and the degree of risk requires.  The Contracting Government is to undertake to arrange for information relating to aids to navigation to be made available to all concerned, and AIS provides a means of promulgating near-real time information on aids to navigation.

When deploying AIS AtoN it is important to bear in mind that not all vessels are equipped with AIS. Of those vessels that are AIS equipped the displays available can range from no display on some Class B, through the mandatory Class A MKD to full ECDIS and Radar overlay. In the absence of ECDIS or Radar overlay users will not be able to fully utilise AIS AtoN functionality.

# Objective

The criteria for fitting AIS to any Aid to Navigation should be based on the navigational requirement derived from the assessment of risk.  There is a distinction between the application of AIS as an AtoN to assist in navigational purposes and other applications of interest to the AtoN authority.

## Scope

This guideline identifies general criteria to assist AtoN authorities in determining whether AIS functionality should be provided and, if so, what approach should be taken to establish the AIS AtoN.  This guideline should be read in conjunction with IALA Recommendation A-126, ITU 1371 and the relevant references as provided in Section 12.

This document does not provide advice to the mariner on the use of AIS nor should it be considered a comprehensive source of information on AIS regulations and specifications.

# Introduction

The fitting of AIS to AtoN can provide the following services to AIS equipped vessels:

- Providing all weather identification of the AtoN;

- Complementing existing signals from AtoN;

- Transmitting an accurate position for floating AtoN, including indicating if the AtoN is off position;

- Providing an accurate position for fixed AtoN which act as reference targets for verifying radar;

- Status indication of the co-located traditional AtoN;

- Making or delineating tracks, routes areas, and limits (for example wind turbines, wave and tidal energy devices, oil and gas platforms) and

- Providing meteorological and hydrographic information through the use of binary messages; and

- Indicating AtoN status, meteorological and hydrographic information by text messages.

In addition, the fitting of AIS to AtoN provides benefits to the AtoN authority:

- Monitoring of AtoN status in real-time or near real-time;

- Gathering real-time information on the “status of health” of an AtoN and

- Tracking AtoN that are gone-from position;

- Monitoring of the status of other support equipment at the site;

- Remotely controlling changes in AtoN parameters (if so equipped);

- Identifying ships involved in collisions with AtoN

- Providing statistics on malfunctions of AtoNs

- Extend the coverage (range) of monitoring

# VDL Considerations

As noted in A-126, an authority planning the application of AIS for AtoN should bear in mind the message capacity of the VHF data link (VDL).

In crowded shipping areas, since the number of AIS messages signals originated from AIS equipped ships increases, transmission of AIS information of lesser urgency should be kept to a minimum level in quantity in order to avoid overcrowding the messages. The Competent Authority may consider increasing the interval of AtoN AIS messages to reduce congestion.

# Definitions of AIS AtoN and examples of use

There are different aspects to implementing AIS AtoN, based on the provision of an actual unit on a physical AtoN to the transmission of a ‘synthetic’ or ‘virtual’ AtoN by an AIS base station, where they fall within the footprint of the base station.

As with all AIS AtoN, these units should be established taking into account the needs of all waterway users – SOLAS and non-SOLAS craft.

## Real

A ‘real; AIS AtoN is one that is physically located on the AtoN.  This unit transmits:

- Message 21 – identification of AtoN and current status;

- Message 8 – meteorological and hydrographic data;

- Message 21 and 14 – identify obstructions [?]

- [other message?]

There are three types of Real AIS AtoN:

- Type 1 – transmit only station

- Type 2 – similar to type 1, includes a receiver for remote configuration

- Type 3 – full transmit and receive station.

- All three station types are capable of switching off and ‘sleeping’ between transmissions to conserver power use

Where they can be applied:

- to use existing monitoring systems for transmission of AtoN’s status via base station.

- to use existing monitoring systems of meteorological/ hydrological data via base station

Provision plan is to be showed ANNEX ////.

- .

Error: Reference source not found from IEC 62320-2 provides the various capabilities of real AIS AtoN.

## Synthetic

There may be times when, for practical and/or economic reasons it is not appropriate to fit a real AIS to an AtoN.  In these instances, consideration should be given to the use of ‘Synthetic’ AIS AtoN.  There are 2 types of Synthetic AIS AtoN – ‘Monitored Synthetic AIS AtoN’ and ‘Predicted AIS AtoN’.

Where they can be applied:

-  to use for fixed  AtoNs like lighthouses, leading lights, fixed entrance lights, beacons etc. and transmitted via base station

-  to transmit information related with visibility (meter) ??????

### Monitored

A ‘monitored’ synthetic AIS AtoN is transmitted as a Message 21 from an AIS Base Station located in the vicinity of the AIS AtoN.  In this instance, the AtoN physically exists, however there does not have a real AIS AtoN unit.  The communication between the AtoN and the AIS shall confirm the location and status of the AtoN.

Possible areas where it may be appropriate to use ‘Monitored Synthetic AIS AtoN’ may include:

- on an existing monitoring system to transmit the AtoNs status via a base station;

- to feed meteorological / hydrographic data via a base station.

### Predicted

A ‘predicted’ synthetic AIS AtoN is transmitted as a Message 21 from an AIS Station located remotely from the AtoN.  The AtoN exists, but there is no monitoring to confirm either location or status.  Only a monitored synthetic AIS AtoN ensure the integrity of the floating AtoN, therefore the use of predicted synthetic AIS AtoN is note recommended for use on floating AtoN.

Possible areas where it may be appropriate to use ‘Predicted Synthetic AIS AtoN’ may include:

- On fixed AtoN (lighthouses, beacons, etc.);

- To transmit information related to binary messages (meteorological / hydrographic data, additional binary messages).

## Virtual

Virtual AIS AtoN are used where it is not physically possible to put the real AtoN on station, examples include ice conditions, new wrecks and dangers, etc. A ‘virtual AIS AtoN’ is transmitted as a message 21 for an AtoN that does not physically exist.

Possible areas where it may be appropriate to use ‘Virtual AIS AtoN’ may include:

- Replace existing buoys;

- Mark new, dangers, wrecks;

- Indicate deep water channels where physical AtoN cannot be established or where following risk assessment are not required ;

- Indicate landfall or some other key point of interest where it may be impossible to locate a physical buoy;

- Mark underwater cables or structures;

- Mark hydrographic survey areas;

- Mark exclusion zones / SAR or pollution zones.

# Accessing the VDL

As with all AIS units, AIS AtoN require MMSI numbers to access the VDL.

The numbering format for AIS AtoN is as per ITU-R M585-4, Annex 4.  This number is a nine-digit unique number using the format 9192M3I4D5X6X7X8X9 where the digits 3, 4 and 5 represent the MID and X is any figure from 0 to 9.  In this format, the MID represents the territory or geographical area of the administration assigning the call identity for the navigational aid.

The administration may use the sixth digit to differentiate between certain specific uses of the MMSI, as shown in the example applications below:

a)	99MID1XXX		Physical AIS AtoN

b)	99MID6XXX 	Virtual AIS AtoN

# Provision Criteria for AIS AtoN to assist navigation

The criteria for fitting AIS to any Aid to Navigation should be based on the navigational requirement derived from the assessment of risk. AIS messages for an AtoN may be generated from information derived from the AtoN itself and broadcast directly from the AtoN, or broadcast from an AIS unit not located on the AtoN – i.e. a transmitting AIS base station.

## Categorisation of Application Area

The navigation element as noted in the IALA Navguide: Navigation 1 – Ocean, Coastal, Restricted Waters and Docking.

[would this be of use?  Link provision to degree of risk that may exist / the type of service provided – i.e. for Ocean may not be appropriate at all, for Coastal use for landfall, additional met/hydro data, etc.; for Restricted waters (areas of congestion) verify location, state of health, other? Also, need to verify that the AIS AtoN use does not have a negative impact on the VDL]

## Elements to be considered

When deciding to implement AIS AtoN there are a series of steps / elements to be considered. A flow chart summarising some of these considerations is shown at Annex 5.

## AIS AtoN station assessment criteria

### Lighthouses and Beacons

The primary purpose of providing AIS AtoN functionality to lighthouses and beacons is to provide the mariner with a fixed point of reference on the shore, and to confirm the functionality of other AtoN provided on the station. Such a point of reference is the e-Navigation equivalent of the physical station and is of assistance to the mariner in identifying a particular point of land, for spatial awareness, in taking bearings or distances for position confirmation, or in laying off parallel index lines. The provision of AtoN functionality information gives advance information to the mariner on whether key AtoN are performing correctly and allows for revision of the passage plan if required.

#### Criteria

AIS AtoN functionality should be provided on Lighthouses and Beacons where a navigational assessment identifies the requirement set out above. Typical locations for consideration would include offshore stations, headland stations, landfall stations, stations that are commonly used as waypoints, stations that mark points on featureless coastlines, or isolated dangers.

### Buoys and Major Floating Aids

In addition to the point of reference and AtoN information purposes described above for fixed AtoN the provision of AIS AtoN functionality on floating AtoN such as buoys and Major Floating Aids (MFA) provides confirmation of the floating AtoN position. Confirmation of position provides the mariner with an assurance that the AtoN can be used, improves spatial awareness, enables bearings and distances for position confirmation, or in laying off parallel index lines. Advance confirmation of the position of floating AtoN is a significant improvement in the service available to the mariner.

#### Criteria

An important consideration is to identify high risk areas where, in poor conditions, AIS can improve the conspicuity of AtoNs as additional information for users. The importance of the floating AtoN in the navigational solution is the key to assessing need for provision of AIS AtoN functionality. Therefore, it is likely that all Major Floating Aids (MFA) and Buoys of Primary Navigation Significance (BPNS) will be AIS equipped. Of the remaining floating AtoN, typical locations for consideration would include gateway buoys at the approach to narrow channels, buoys that are commonly used as waypoints, buoys marking isolated dangers, buoys marking the extremities of shoal areas and buoys that are critical to the mariner’s spatial awareness.

### Virtual AtoN

Virtual AIS AtoN are used where it is not physically possible to put the real AtoN on station, examples include ice conditions, new wrecks and dangers, etc. However, virtual AtoN may also have a future role in replacing some MFA or buoys and in providing AtoN for special craft such as Wing-In-Ground (WIG) vessels.

#### Criteria

The criteria for use of virtual AIS will be influenced by the practicality of providing a physical AtoN and Real or Synthetic AIS. Where physical AtoN cannot be provided, for example in ice conditions, virtual AIS AtoN can contribute to reducing the risk and provide additional information for the user. The need to rapidly mark wrecks and other new dangers with virtual AIS AtoN in advance of laying physical AtoN will be based on an assessment of the danger posed to shipping in the area. The criteria for replacement of existing AtoN with virtual AtoN will involve an assessment of the navigational value of the physical characteristics. Typical locations would include deepwater AtoN intended primarily for Safety –Of –Life –At - Sea Convention (SOLAS) vessels. Possible future provision for new vessel types such as WIG craft arise from the need to mark a channel without having the physical obstruction of an AtoN.

# Additional applications of AIS AtoN

An AtoN Authority can gain benefit from the provision of AIS AtoN with regards to monitoring of aids to navigation, data collection and networking.

## Monitoring

There is a wide range of possible means of monitoring AtoNs. These range from physical observation, radar coverage, and electronic reporting through to full telemetry control systems. Whether an AIS AtoN Station is required for navigational purposes or not, AIS may still be used as an AtoN monitoring and networking tool.

Where AIS equipment at the AtoN is used for monitoring, consideration must be given to the possibility of the failure or the AIS itself while the AtoN continues to function. If a secondary monitoring system is available, any AIS messages being transmitted to the mariner may still be transmitted from an adjacent station as synthesised messages.

## Data Collection

AIS AtoN can collect and store data regarding the activity on the VDL.  AIS AtoN can also be chained in a parent / child system to extend coverage range and enable data collection throughout a navigational area.

## Networking

Networks of AIS Base Stations can provide a useful means of monitoring the integrity of transmitted AIS messages and for local storage of data.

The availability of detailed AIS vessel track information will contribute significantly to the navigational review process.

## Criteria

Where an AIS Base Station is being provided as part of the AtoN requirement, consideration should be given to utilisation of the station for monitoring of other AtoN in the area and for network, data storage and forwarding purposes. The criteria in this case are likely to centre on communications costs and reliability. While AIS data can be secured using time or event generated polling on dial-up lines, live streaming over broadband or similar high speed lines is preferred.

Where there is no AtoN requirement, the criteria for using AIS as an alternative means of monitoring will centre on cost, reliability and the value attached to the additional functionality available from the various systems.

In establishing an AIS station, consideration should be given to future as well as present requirements

# AIS AtoN message use

AIS Message 21 is the primary AIS AtoN information message.  This message provides details of the identify, position and status of the AtoN and will warn the mariner if any of the AtoN functions are not performing correctly.  In addition, there are a number of other messages that can be used with AIS AtoN, enabling additional information to be transmitted to the mariner and also providing the AtoN authority with status and monitoring capabilities.

ITU-R M.1371 defines the AIS messages used, and IEC 62320 provides a summary of the AIS AtoN station messages. A summary of Message Types (current at date of publication) is available at Annex 4

# Display of AIS AtoN and Symbology

Guidelines for display of AIS AtoN are provided by IMO in IMO SN-Circ 243 Guideline on AIS Symbology. This symbology relates to display on Radar, ECDIS or other similar systems and is based on the use of a diamond symbol.

IEC 62288 also sets out similar AIS Display System symbology and provides for the use of a ‘V’ symbol for virtual AIS in addition to the basic diamond symbol.

AIS symbology on navigational charts is governed by IHO standards. The present symbology consists of a magenta circle with an adjacent legend saying ‘AIS’.

Particular issues arise in relation to virtual AIS. Effective utilization of virtual AIS needs an internationally accepted standard on symbols defined by type so that the user can clearly identify the AtoN area of interest.

Symbols used for AIS AtoN should:

-  comply with the universally established symbols used for charted physical AtoN (real entity), and

-  not contain a symbol depicting a light in the illustration of virtual AtoN.

A liaison note from IALA to IMO on this subject is set out at Annex 2.

[this section to be finalized once response from the liaison note ANM/09/05 is received]

## On SOLAS vessels

## On non-SOLAS vessels

## Shore system

# Conclusion

[text to reference the objective]

# REFERENCES

- IALA, IALA Aids to Navigation Guide (Navguide) 5th Edition,

- IALA Recommendation A-126.

- ITU-R M.1371

- ITU-R M.585-4

- IMO SOLAS Chapter V, Reg. 14

- IEC 62320-2 AIS AtoN

- IEC 62288　ECDIS

- IEC 62388　New Radar

- [other?]

ANNEX 1 – Function of Virtual AtoN AIS

| Application Mode | Function | Description | Type of Virtual AtoN AIS |
| --- | --- | --- | --- |
| Permanent Marking of Obstacles | Marking of Shoals and Reefs | Virtual AtoN can be effectively utilized where it is difficult to place a physical AtoN due to sea state, winds, or other environmental or ecological conditions. A clear marking of the shoals and/or reefs will improve safety of navigation. | Isolated Danger Marks |
| Permanent Marking (navigation support） | Marking of Fairway Limits | Virtual AtoN can be effectively utilized where lighted buoys cannot be installed for some reason (i.e. sharing of waterway). A clear marking of the fairway limits will serve for orderly flow of marine traffic and improved safety of navigation. | Lateral Marks |
|  | Marking of Fairways | Virtual AtoN can be effectively utilized where a physical AtoN placement is difficult due to the water depth, seabed, etc. A clear marking of the fairway will improve safety of navigation. | Lateral Marks, & Safe Water Marks |
|  |  | Virtual AtoN can be effectively utilized in approaches to a harbour entrance where a ship changes its course and where it is difficult to install a physical AtoN. A clear marking of the point of approach will serve for an orderly flow of ships at a harbour entrance and improve safety and efficiency of shipping. | Safe Water Marks |
|  | Marking of Fairways & Marking of the Limits of Safe Water | Virtual AtoN can be effectively utilized where navigation becomes difficult due to a thick fog, heavy rain, etc. (This application can also be adapted as a temporary marking during a limited visibility.) Marking of a recommendable fairway during times of limited visibility will serve to improve safety of navigation and efficiency of shipping. | Lateral Marks, & Safe Water Marks |
| Temporary Marking | Marking of a Navigation Restricted Areas | Virtual AtoN can be effectively utilized when navigation restriction is required due to e.g., marine accidents or when marking of a wreck. A temporary navigation restriction can prevent subsequent incidents from developing. | Cardinal Marks, & Special Marks |
|  | Designation of Temporarily Recommendable Fairways | Virtual AtoN AIS can be effectively utilized for indication of fairways when a large scale disaster hits the area. A clear marking of temporarily recommendable fairways will be expected to serve for the relief ships dispatched to the site and to support safe and effective relief activities. | Lateral Marks & Safe Water Marks |
|  | Marking of Aids to Navigation in Trouble | Virtual AtoN AIS can be effectively utilized when a physical AtoN has lost ability to perform regular functions due to a natural disaster. When a physical AtoN lost ability to perform regular functions due to natural disasters, recovery actions are required at the earliest opportunity. Virtual AtoN can respond to the circumstance even if the actions by personnel cannot be achieved due to meteorological and/or hydrographical conditions, and keep the influence of the trouble reach to ships in navigation at a minimum level. | Cardinal Marks, Lateral Marks, Isolated Danger Marks, Safe Water Marks, Special Marks, & Other Position Marks |

ANNEX 2 – Display of AIS AtoN – IALA Liaison Note to IMO

| From: IALA | Formerly e-NAV 4-output-11 |
| --- | --- |
| To: IMO NAV Sub-Committee | February 29, 2008 |

Liaison Note to IMO NAV Sub-Committee

Proposal on Symbols for AtoN AIS

Introduction

IALA has been developing the Recommendation A-126, however it does not include the symbols for AtoN AIS.  In 2005, Japan established the Study Committee to study the operations and indication of the AtoN AIS for both real and virtual AtoNs.  The Committee was organized by the maritime experts such as professors, experienced engineers, pilots, captains and manufactures etc.  The Committee considered and developed the user friendly draft AtoN symbols for shipborne navigation display based on the paper chart.  By using the draft symbols, the Committee member conducted and evaluated the field trials in Tokyo area in 2006.  The trial report was submitted in September 2007and noted by IALA e-Navigation Committee.

In the same period, the real and virtual AtoN symbols were considered and decided by International Organizations which would be suitable for the ECDIS and other navigation displays.

This Liaison note proposes the draft AtoN symbols be considered for the future shipborne navigation display as user options.

Study and Field Trials for AtoN AIS Symbols

The Study Committee in Japan considered the draft AtoN AIS symbols in ANNEX 1 which is similar to the paper chart.  ANNEX 1 only shows the virtual AtoN symbols, however the real AtoN symbols are defined using solid line.

Using the draft symbols, the field trials were conducted and ANNEX 2 shows the display example of the shipborne navigation display.

As the result of field trials, the draft AtoN symbols were generally satisfied by the evaluators, however the following issues were identified as in ANNEX 3:

- readabilities depends on the display resolutions

- size and shape of the symbols need further studies at zooming

- symbols need simplification

Proposal

IALA has identified the several issues need to be further studied, however IALA kindly requests IMO NAV Sub-Committee to consider the proposed draft symbols as user options for the future shipborne navigation display in conjunction with the related organisations.

***

Table 1  Draft “Symbols of AtoN AIS”

| No. | Type of AtoN |  | (draft) Symbols of AtoN (Real AtoN symbols are in solid line) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Floating Aids | Fixed Aids |
| 1 | Cardinal Marks | North Cardinal Mark | Virtual AIS | Virtual AIS |
| 2 |  | East Cardinal Mark | Virtual AIS | Virtual AIS |
| 3 |  | South Cardinal Mark | Virtual AIS | Virtual AIS |
| 4 |  | West Cardinal Mark | Virtual AIS | Virtual AIS |

Table 1  Draft “Symbols of Virtual AtoN AIS” (continued)

| No. | Type of AtoN |  | (draft) Symbols of Virtual AtoN (Real AtoN symbols are in solid line) |  |
| --- | --- | --- | --- | --- |
|  |  |  | Floating Aids | Fixed Aids |
| 5 | Lateral Marks | Port Hand Mark | Virtual AIS | Virtual AIS |
| 6 |  | Starboard Hand Mark | Virtual AIS | Virtual AIS |
| 7 |  | Mark for Preferred Channel to Port | Virtual AIS | Virtual AIS |
| 8 |  | Mark for Preferred Channel to Starboard | Virtual AIS | Virtual AIS |

Table 1  Draft “Symbols of Virtual AtoN AIS” (continued)

| No. | Type of AtoN | (draft) Symbols of Virtual AtoN (Real AtoN symbols are in solid line) |  |
| --- | --- | --- | --- |
|  |  | Floating Aids | Fixed Aids |
| 9 | Isolated Danger Marks | Virtual AIS | Virtual AIS |
| 10 | Safe Water Marks | Virtual AIS |  |
| 11 | Special Marks | Virtual AIS | Virtual AIS |
| 12 | Lighthouse | Virtual AIS Virtual AIS |  |

Display Example of AtoN Symbols at Field Trials

ANNEX 3

Improvement of Symbols based on Results of Field Trials

In the field trial, a mobile type personal computer and the dedicated software were used as the display. The readable symbol size was approximately 2 cm in long sides due to the display size (14 inches) and resolution of XGA (1024 x 768).  During the trials, the symbols of the ship near the AtoN AIS were hidden due to the symbol overlapping.

As the result of the trials, the following issues were identified:

- readabilities depends on the display resolutions

- size and shape of the symbols need further studies at zooming

- symbols need simplification

As one of the solutions, the following simplified symbols are under considerations in the Study Committee in Japan.

Simplified symbol example for floating AtoN AIS

Simplified symbol example

for fixed AtoN AIS

Table 1 – Description of AIS AtoN Stations

| Requirements | Type 1 AIS AtoN Station | Type 2 AIS AtoN Station | Type 3 AIS AtoN Station | Alternatives |
| --- | --- | --- | --- | --- |
| VDL Receiver | No Receiver | Receiver for control functions only | 2 Receiving processes for autonomous mode |  |
| Transmitted Messages | 21 |  |  | 21 plus one or more of 6, 8, 12, 14, 25 and other appropriate messages (Types 1, 2 and 3) Plus 7, 13 (Type 3 only) |
| Access Mode for Message 21 | FATDMA |  |  | FATDMA and RATDMA (Type 3 only) |
| Access Mode for Messages other than 21, if implemented |  |  |  | FATDMA (Type 1 and 2) One or more of FATDMA, RATDMA or CSTDMA (Type 3) |
| Configuration / Communication method | Defined by manufacturer |  |  | Defined by manufacturer with Standard Sentences (Type 1, 2 and 3) |
| Physical Communication Interface | None |  |  | The electrical and physical characteristics shall be defined by manufacturer. (Type 1, 2 and 3) |
| Transmit Power | 12,5 W |  |  | As defined by manufacturer (Type 1, 2 and 3) |
| Transmitter capability | Dual channel |  |  | Single channel (Type 1 and 2) |
| Synthetic and Virtual AtoN | No |  |  | Yes (Type 1, 2 and 3) |
| Positioning Device | EPFS and Surveyed Position |  |  | Surveyed Position Only (no EPFS) (Type 1, 2 and 3) |
| UTC synchronisation | Direct Only |  |  | Direct, Indirect or semaphore (Types 3) |
| Assignment | Shall not respond to assignment Messages 16 and 23 |  |  |  |
| Interrogation | Shall not respond to interrogation Message 15 |  |  |  |

Annex 4 – Summary of Current Message Types

| Message ID | Name | Description | Priority | Access scheme | Communication state | M/B |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Position report | Scheduled position report; (Class A shipborne mobile equipment) | 1 | SOTDMA, RATDMA, ITDMA | SOTDMA | M |
| 2 | Position report | Assigned scheduled position report; (Class A shipborne mobile equipment) | 1 | SOTDMA | SOTDMA | M |
| 3 | Position report | Special position report, response to interrogation; (Class A shipborne mobile equipment) | 1 | RATDMA | ITDMA | M |
| 4 | Base station report | Position, UTC, date and current slot number of Base station | 1 | FATDMA RATDMA | SOTDMA | B |
| 5 | Static and voyage related data | Scheduled static and voyage related vessel data report; (Class A shipborne mobile equipment) | 4(5) | RATDMA, ITDMA | N/A | M |
| 6 | Binary addressed message | Binary data for addressed communication | 4 | RATDMA, FATDMA, ITDMA | N/A | M/B |
| 7 | Binary acknowledgement | Acknowledgement of received addressed binary data | 1 | RATDMA, FATDMA, ITDMA | N/A | M/B |
| 8 | Binary broadcast message | Binary data for broadcast communication | 4 | RATDMA FATDMA, ITDMA | N/A | M/B |
| 9 | Standard SAR aircraft position report | Position report for airborne stations involved in SAR operations, only | 1 | SOTDMA, RATDMA ITDMA | SOTDMA ITDMA | M |
| 10 | UTC/date inquiry | Request UTC and date | 3 | RATDMA, FATDMA, ITDMA | N/A | M/B |
| 11 | UTC/date response | Current UTC and date if available | 3 | RATDMA, ITDMA | SOTDMA | M |
| 12 | Addressed safety related message | Safety related data for addressed communication | 2 | RATDMA, FATDMA, ITDMA | N/A | M/B |
| 13 | Safety related acknowledgement | Acknowledgement of received addressed safety related message | 1 | RATDMA, FATDMA, ITDMA | N/A | M/B |
| 14 | Safety related broadcast message | Safety related data for broadcast communication | 2 | RATDMA, FATDMA, ITDMA | N/A | M/B |
| 15 | Interrogation | Request for a specific message type (can result in multiple responses from one or several stations)(4) | 3 | RATDMA, FATDMA, ITDMA) | N/A | M/B |
| 16 | Assignment mode command | Assignment of a specific report behaviour by competent authority using a Base station | 1 | RATDMA, FATDMA | N/A | B |
| 17 | DGNSS broadcast binary message | DGNSS corrections provided by a Base station | 2 | FATDMA, RATDMA | N/A | B |
| 18 | Standard Class B equipment position report | Standard position report for Class B shipborne mobile equipment to be used instead of Messages 1, 2, 3(8) | 1 | SOTDMA, ITDMA, CSTDMA | SOTDMA, ITDMA | M |
| 19 | Extended Class B equipment position report | Extended position report for class B shipborne mobile equipment; contains additional static information(8) | 1 | ITDMA | N/A | M |
| 20 | Data link management message | Reserve slots for Base station(s) | 1 | FATDMA, RATDMA | N/A | B |
| 21 | Aids-to-navigation report | Position and status report for aids-to-navigation | 1 | FATDMA, RATDMA | N/A | M/B |
| 22 | Channel management | Management of channels and transceiver modes by a Base station | 1 | FATDMA, RATDMA | N/A | B |
| 23 | Group assignment command | Assignment of a specific report behaviour by competent authority using a Base station to a specific group of mobiles | 1 | FATDMA, RATDMA | N/A | B |
| 24 | Static data report | Additional data assigned to an MMSI Part A: Name Part B: Static Data | 4 | RATDMA, ITDMA, CSTDMA, FATDMA | N/A | M/B |
| 25 | Single slot binary message | short unscheduled binary data transmission (Broadcast or addressed) | 4 | RATDMA, ITDMA, CSTDMA, FATDMA | N/A | M/B |
| 26 | Multiple slot binary message with Communications State | scheduled binary data transmission (Broadcast or addressed) | 4 | SOTDMA, RATDMA, ITDMA | SOTDMA, ITDMA | M/B |

Synthetic monitored

Synthetic predicted

Determine type of AIS AtoN (Real, synthetic or virtual)

Determine area of highest risk to deploy AIS Aton

Determine particular AtoN within area concerned

New wreck

No possibility of Real Aton

Determine particular AtoNs to be equipped with AIS

Real AIS-what type 1,2 or 3?

Virtual

Section 2.1.2.2 IALA Navguide, Edition X, 2006.