IALA Recommendation No. XXXX

On

The Emergency Wreck Marking Buoy for New Dangerous Wrecks

Edition #

[Date approved by Council]

[Date of previous edition]

AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

20ter, rue Schnapper, 78100

Saint Germain en Laye, France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05

E-mail - iala-aism@wanadoo.fr      		Internet -  www.iala-aism.org

[note – add in recommendation terminology – this is the annex to the recommendation

Add a liaison note to the front of this draft recommendation for IALA Committees – requesting comments to be sent to ANM7

Note – perhaps requirement to go to other organization?]

IALA Recommendation for wreck marking buoy to specifically mark new dangerous wrecks.

# INTRODUCTION

Following an incident as a result of which a new and dangerous wreck or obstruction is formed, certain measures will have to be taken by the responsible authorities in order to avoid further incidents and to prevent loss of life and/or pollution.

The wreck of the ‘Tricolor’ in the Dover Straits has brought into sharp focus the effective responses required to adequately mark such dangerous wrecks and to prevent further collisions with the wreck.

Therefore administrations need to assess their areas of responsibility and response capability as part of their national contingency planning.

This recommendation should be read in conjunction with the IALA Guidelines on the Response Plan for the Emergency marking of Wrecks.

# Background

## There has been considerable unease as to whether the IALA Maritime Buoyage System is sufficient to adequately mark new dangerous wrecks. Increased light pollution, the presence of multiple aids to navigation in areas of high traffic density and restricted waterways means that existing cardinal and lateral buoyage may not be sufficient to adequately alert the navigator to a new danger. Recent groundings and collisions have indicated a need for a revision of how dangerous wrecks are to be marked and the issues to be considered.

# scope

## This recommendation introduces a proposal for a specific new wreck marking buoy which has “high visual effect” and will be conspicuous to the mariner, both by day and by night.

# Definitions  and Acronyms

[ensure definitions reflect guideline definitions]

EWMP - Emergency Wreck marking Plan

Dangerous wreck – any wreck in navigable water that poses a danger to shipping.

Wreck Buoy – a blue/yellow buoy fitted with blue/yellow alternating lights specifically intended for the emergency marking new dangerous wrecks.

Alternating light – a light showing different colours alternately

# Considerations

Administrations need to assess their areas of responsibility and response capability. The wreck of the Tricolour in the Dover Straits has brought into sharp focus the effective responses required to adequately mark such dangerous wrecks and to prevent further collisions with the wreck. Administrations need to carry out risk assessments, assess their response capabilities and consider plans to deal with such situations.  All aspects of such a risk assessment are included in the EWMP recommendations.

# Initial Marking of a new dangerous wreck with a wreck marking buoy.

A new wreck or obstruction which can cause danger for shipping must be marked.  Weather conditions, sea state and unknown facts about the new wreck can all hamper timely marking.  Irrespective of these circumstances it is of great importance that the wreck location is marked as soon as possible and can be readily recognised by ships as a new wreck location.

Administrations are recommended to deploy an emergency wreck buoy. This buoy is termed a wreck buoy and is specifically for new and dangerous wrecks. It is designed to have “high impact recognition” for the navigator. It should be placed as close to the wreck as possible and within any other marks that may be subsequently deployed.

The surface colour of the buoy is blue and yellow vertical stripes.

It is fitted with an alternating blue and yellow light.

Ideally it should be fitted with a racon Morse Code “D”.

[If fitted with a topmark it should be a yellow sphere above a blue sphere.]

[note – consider a new daymark, for example a diamond]

The alternating blue/yellow lights should have equal flash characters.

The range of the light should be not less than 5 nautical miles.

The sole purpose of such a buoy is the emergency marking of a new dangerous wreck.

When navigators become aware of such a buoy they should immediately reduce speed in order to fully assess the navigational and safety considerations.

Placing such a wreck buoy does not relieve an Authority of its obligation to establish other aids to navigation to fully indicate the position of the wreck.

The wreck buoy should be maintained in position until the wreck is well known and has been promulgated in nautical publications or until the wreck has been fully surveyed and exact details such as position and swept clearance above the wreck are known and permanent markings of the wreck has been carried out.

# Conclusions

These recommendations contain details of a wreck marking buoy which is to be used solely for marking a new dangerous wreck.

[Info Note:

It is important that details regarding the introduction of this buoy are promulgated widely in the marine world prior to its introduction.

IALA publications such as the NAVGUIDE and the MBS will require updating.]

# DIAGRAM

[Paste diagram of buoy here.]