Colourwash Floodlighting of Lighthouses

Executive Summary

The use of modern Radionavigation technology has encouraged mariners to venture into waters not previously navigated and this has subsequently increased requirements for visual Aids to Navigation (AtoN). The growth in coastal residential areas has led to an increase in street lighting and when viewed from the sea it is sometimes difficult to distinguish the AtoN lights against this background illumination. In other words, the light exhibited by the lighthouse is nowadays less conspicuous than when the AtoN was originally installed, even though the light itself hasn’t changed.

A method is required of ensuring that fixed AtoN lights can provide a quick visual confirmation to the mariner in areas of high intensity concentrations of background lighting. One method of achieving this is to colour-wash or floodlight the existing structure of a fixed AtoN at night. In order to determine the most suitable colours, the most effective geometry and the amount of intensity required to floodlight a structure and meet a navigational requirement of 2M, a trial was held at Lowestoft Lighthouse.

The results show that the floodlighting of lighthouses can be effective as a short range AtoN to give the mariner a quick, recognisable fixed point of reference.

The following recommendations should be applied when considering the use of floodlighting:

- the most appropriate colour to contrast with the surroundings be chosen;

- the geometry used to floodlight the lighthouse should evenly illuminate the area;

- that area should subtend a minimum angle of 3 minutes of arc at the eye of the observer at the maximum distance of recognition; and,

- that a suitable value of illuminance at the eye of the observer is chosen to overcome any adjacent/background lighting.

The methods to achieve the above criteria are described herein.

# Introduction

Mariners require the ability to detect, recognise and verify a fixed point of reference at short range, during inbound/outbound and transit voyages of ports and harbours. This is made difficult during night time conditions in certain areas due to the plethora of background lighting that has built up in recent times.  This has led to some fixed AtoN being unable to meet their primary purpose.  Whilst at distance, the mariner will be able to use the rhythmic character of the fixed AtoN; at short range they often require a quick, visual confirmation of the AtoN location which does not rely on the AtoN rhythmic character.

One method of achieving this is to colour-wash, or floodlight, the existing structure of a fixed AtoN at night.

# Methods

An initial feasibility study was carried out using floodlight housings with different coloured BLV 240V 250W (E40) TOPFLOOD Lamps. Blue, Orange, Green, Magenta and Near White were the colours used. The conclusion of this study was that colour contrast was the dominant factor; therefore, in order to evaluate the optimum colours for floodlighting, more saturated colours would be desirable. LED floodlights were evaluated in a further trial where it was confirmed that colour contrast was indeed the dominant factor. With that in mind, a trial at Lowestoft lighthouse was designed to meet the objectives. Three Pulsar Chromaflood 200(W) RGB LED Floodlights were positioned 1.5m away from the tower at an angle of 50o to ‘down light’ the lighthouse. Three more floodlights were positioned 3m away from the base of the tower at an angle of 45o to ‘up-light’ the lighthouse as shown in Figure 1.

Figure 1  Geometry of floodlight positions

The sequence shown in Table 1 was chosen to provide the most effective colour contrast; firstly, with each other and secondly with the surroundings. For each colour in turn, the up-lights were exhibited first, then the down-lights and then both combined. This was in order to determine the most effective geometry. Three trials were conducted following this method. The first trial was at quarter intensity, the second at half intensity and the third at full intensity. This was to determine the most appropriate intensity level to meet the navigational requirement (2M in this case).

Table 1 Colours exhibited in the trial and percentage RGB mix

|  | Floodlight Software setting to achieve colours (%) |  |  |
| --- | --- | --- | --- |
| Colour | R | G | B |
| Blue | 0 | 0 | 100 |
| Orange | 50 | 100 | 0 |
| Green | 0 | 100 | 0 |
| Cyan | 0 | 100 | 100 |
| Magenta | 100 | 0 | 100 |
| Yellow | 100 | 50 | 0 |
| White | 100 | 100 | 100 |
| Red | 100 | 0 | 0 |

# Observations

Four experienced observers were used in these trials and all were asked not to confer until observations had been made. The exhibited lights were identified by numeric indicators when presented to the observers. This was to not influence the outcome with expectation [RD4]. An observation sheet, developed by R&RNAV (see appendix 1) in conjunction with international best practice was used to obtain the optimum results from observations. All observations were made on board THV Alert at a distance of 2 nautical miles due east of the lighthouse. A total of 72 observations were made during this trial.

# Results and Interpretation

Figure 2  Results of the First viewing trial – Quarter Intensity

It can be seen from the results in Figure 2 that when the floodlights were set at quarter power none of them were acceptable. The orange, yellow and white colours were the worst performers. This was due to the poor colour contrast with the orange/yellow appearance of the adjacent street lighting. The red, green and cyan were the best colours by contrast with the adjacent street lighting. It was interesting to note the comments from the observers with regard to the blue and magenta colours. The blue light appeared to be blurred, probably due to an effect known as ‘tritanopic myopia’ – the average eye being short-sighted to blue. The magenta appeared to be red in foveal (direct) vision, but magenta in peripheral vision. The explanation for this is twofold: the atmosphere preferentially scatters shorter wavelengths of light (e.g. blue); and the dark adapted eye becomes more sensitive to blue content of the light under dark conditions [RD7] [RD9]. As the magenta colour is achieved by mixing red and blue light, the blue light is scattered by the atmosphere so the magenta appears red when viewed directly. However, in peripheral vision, when the observer looks slightly away from the light, the colour appears magenta. Due to these reasons, the use of magenta as a colour for floodlighting may be questionable.

Figure 3 Results of the Second viewing trial – Half Intensity

When the intensity was increased to half intensity for the second trial (Figure 3), it can be seen that the results follow the trend of the first trial with the green, cyan and red being observed as instilling the most confidence as an AtoN. The cyan colour was the most conspicuous. Again the orange, yellow and white were unacceptable and the blue and magenta colours were only acceptable when the down-lights and up-lights were combined.

Figure 4 Results of the Third Viewing Trial – Full Intensity

The third trial saw a continuation of the previous trends with the cyan colour shown to be the most conspicuous and instilling the most confidence as an AtoN. The combination of luminous efficacy of the green content and the colour contrast of the blue content against the adjacent street lighting making it the best solution for this application. The green and red were observed to be very good, but not as conspicuous as the cyan colour. The blue colour was easy to identify, but the blurring as mentioned previously made it difficult to resolve the outline shape of the tower. The magenta colour again appeared red in foveal vision and magenta in peripheral vision with the orange, yellow and white colours not being acceptable.

# Measurements and Calculations

Further to the trial, a number of measurements and calculations were made in order to try and quantify the results gained. Table 2 shows the measured and calculated intensity for each of the colours used in the trial.

Table 2 Measured and Calculated Intensity for Colours used in the Trial

Measurements were made on the R&RNAV Outdoor Light Range and calculations were made using a percentage of the colours mixed i.e. Orange = 100% Red + 50% Green.

Figure 5 shows the geometry involved in floodlighting [RD3]. To calculate the illuminance at point p, the formula in Equation 1 can be used.

Ep=ID2cos3γ

Equation 1 Calculation of illuminance

Where

Ep is the illuminance at point p of the surface (assumed to be vertical)

I is the luminous intensity of the floodlight

D is the distance between the floodlight and the surface

γ is the angle between the ray and the normal to the surface

Figure 5 Geometry involved in floodlighting

N.B. cos γ= cos A cos B, where A is the Azimuth angle, and B is the elevation angle.

Table 3 Beam Areas of Floodlights used in the trial

The Chromaflood 200 floodlights used in this trial were fitted with 12 x 25 degrees lens plates. These figures (A & B respectively) are used to calculate the beam areas of the floodlights as in Table 3. Once the illuminance at point p is calculated, the formula in Equation 2 can be used to calculate the luminance emanating from the tower.

L=Eρπ

Equation 2 Calculation of Luminance

IALA guidance states that illumination levels; ‘For night time use, a minimum of 20 microlux and a maximum of 200 microlux illuminance at the eye of the observer is required depending on the level of background lighting.’ [RD1] From the calculated luminance and the projected surface area, the illuminance at the eye of the observer can be calculated. Tables 3-10 show the calculations for each of the colours used in the trial. It is interesting to note that although the illuminance at the eye of the observer for the up-lights and down-lights are similar, the angle subtended for the up-lights is 4.8 minute of arc, but only 1.4 minutes of arc for the down-lights as shown in Table 12. The minimum angle for the recognition of simple shapes is 3 minutes of arc [RD10]. This would suggest that in order to recognise a floodlit structure as a marine AtoN the shape should subtend a minimum angle of 3 degrees at the eye of the observer at the maximum distance of recognition.

Table 4 Calculations for Luminance and Illuminance for Blue

Table 5 Calculations for Luminance and Illuminance for Orange

Table 6 Calculation for Luminance and Illuminance for Green

Table 7 Calculations for Luminance and Illuminance for Cyan

Table 8 Calculations for Luminance and Illuminance for Magenta

Table 9 Calculations for Luminance and Illuminance for Yellow

Table 10 Calculations for Luminance and Illuminance for White

Table 11 Calculations for Luminance and Illuminance for Red

N.B. Reflectance factor was measured using a Lovibond RT962 Reflectance Tintometer.

Table 12 Angular Subtense of Floodlit Area at the Eye of the Observer

Table 13 Observed Brightness vs. Intensity for Second Viewing Trial

It is interesting to note from table 13 that the luminous intensity does not correlate to the perceived brightness. This demonstrates that colour contrast is indeed the dominant factor.

# Conclusions

- It can be seen from the results that the floodlighting of lighthouses can be effective as a short range AtoN to give the mariner a quick, recognisable fixed point of reference.

- In this trial, cyan was the most recognisable colour, appeared to be the brightest, was the most conspicuous and instilled the most confidence as an AtoN.

- The green and red colours used in the trial, although effective may be confusing to mariners because of the lateral significance associated with these colours.

- The blue colour used in the trial, although effective as a colour does not lend itself to outlining of shapes because of the perceived blurring effect.

- Similarly magenta although effective as a colour does not lend itself to floodlighting because of atmospheric scattering.

- The colours orange, yellow and white used were not perceived as suitable colours in this trial because they did not provide enough colour contrast with the adjacent street lighting.

- The shape of the floodlit structure should subtend a minimum angle of 3 degrees at the eye of the observer, at the maximum distance of recognition, in order for the observer to recognise shape.

- Between 60 and 120 microlux illuminance at the eye of the observer was required for the acceptable recognition of the lighthouse tower in this trial.

# Recommendations

It is recommended that when considering the use of floodlighting the GLAs should:

- Use floodlighting at locations where the mariner is in close proximity to an AtoN and where background or rival lighting adversely affects the performance of the AtoN light at night;

- Use the most appropriate colour to contrast with the surroundings;

- Use a geometry that evenly illuminates an area that subtends a minimum angle of 3 degrees at the eye of the observer at the maximum distance of recognition;

- Choose a suitable value of illuminance at the eye of the observer to overcome any adjacent/background lighting.

Reference Documents

- IALA Guideline 1061 On Light Applications Illumination of Structures

- Lighting the Environment – A guide to good urban lighting (CIBSE, ILE)

- CIE 94-1993 GUIDE FOR FLOODLIGHTING

- Vision and Visual Perception – Graham (1965)

- Colour Science – Wyszecki & Stiles Second Edition (2000)

- INF-159-I03-MN-05 Emergency Wreck Marker Buoy Light Beacon Viewing Trial

- IALA Recommendation E-200-1 On Marine Signal Lights Part 1 – Colours

- IALA Recommendation E-200-2 On Marine Signal Lights Part 2 – Calculation, definition and notation of luminous range

- RPT-28-MN-04 Conspicuity – Colour Observed and measured comparison of a White LED Beacon versus a White Halogen Beacon

- Farbgebung und Tagessichbarkeit von Tonnen – Frank Hermann German Federal Waterways Administration – Traffic Technologies Centre (Koblenz)

- R&RNAV Observer Sheet

This was achieved by altering the control software levels.