DRAFT

Report on a Workshop on Developments in Lights and Power Sources for use in Marine Aids to Navigation – IALABATT/IALALITE 5

Sept. 28 – Oct. 1, 2004

Dublin, Ireland

EEP5 / 4 / info1

# Introduction

A workshop on developments in lights and power sources for marine aids to navigation was held at the Clontarf Castle, Dublin, Ireland from Sept. 28 – Oct. 1, 2004.  The workshop was attended by 50 delegates from 19 countries and 13 equipment providers.

The objective of the Workshop was to assist IALA in the review of the “Guidelines on Solar Photovoltaic Systems for Aids to Navigation” and the “Guidelines for New Light Sources and Associated Power Supplies” and IALA Recommendation E-122 (all issued in 2001).  Presentations were made on the latest developments in lights/lighting systems and power source technology.

The workshop also provided an environment where experience can be shared on the use of lights such as LED lanterns, both in technical and financial terms.  With increasing use of LED lanterns, discussions included measurement and calculation procedures for determining effective luminous intensity of LED signals as well as the visual range of LED lanterns compared to incandescent technology.  The use of small modern light sources such as metal halide lamps in original large lenses were considered. A number of manufacturers of visual Aids to Navigation demonstrated and discussed different products.

The Programme of the workshop is attached at Annex 1 and a list of the participants is given at Annex 2.

# Opening

Mr S. Doyle opened the workshop and invited Mr T. C. Johnson, Chairman, Commissioners of Irish Lights to speak on behalf of the Host Organization, CIL, Ireland.  Mr Johnson welcomed the participants to Ireland, and stressed the importance of the objectives of the workshop.  He highlighted specific concerns over the relevant luminosity of LEDs when compared to incandescent lights and the requirement to provide valid and reliable information on AtoNs to the mariner.  The speech by Mr Johnson is given at Annex 3.  In reply Mr Kruuse thanked the Commissioners of Irish Lights for the kind invitation to hold the workshop in this historic city and, on behalf of IALA, welcomed all participants.

Ms. J. Carson-Jackson gave a review of the past IALABATT and IALABATT/IALALITE Workshops.  She also provided an overview of the objectives of the workshop, and the planned organisation of the meeting.  Participants were then asked to respond to the question:

Within the context of new and developing lights and power sources for aids to navigation, what do you feel is the most significant challenge facing your organization today?

The responses to this question were brought to the attention of each working group, and provided the basis for a discussion session of related topics.  An overview of the responses is provided in Annex 4.

## User Experiences

Mr. S. Burrows, CIL and Mr. L. Jaeger, USCG provided an overview of the experiences and concerns identified by their organizations in the implementation of the use of developing lights and power sources.  These presentations provided an overview of practical experience gained in the implementation of the two IALA Guidelines presented for review at the workshop.

# Presentations

Presentations on New Light Sources and New Power Sources provided a basis for further development of the Guidelines.

## Presentations on New Light Sources

| - New Light sources – Clusters and Metrology Issues - The Photometry of Flashing Lights – Are We Getting The Right Answer? - Developments in LED Technology and Recent Research on Human Perception of LED’s - New Light Sources – Vision and Colour Aspects - Demonstrations - Perception of LED versus Incandescent lights - LED based sector light | I. Tutt, The GLA R&D Department D. Gibbs, National Physics Laboratory T. Nurmi, Tampere University M. Nicholson, The GLA R&D Department O.Eriksson J. Unterderweide |
| --- | --- |

These presentations highlighted the concerns over determining effective intensity of lights, indicating that the existing formulas (Blondel-Ray, Schmidt-Clausen) may not be accurate when measuring light intensity for rapidly fluctuating light sources.  In addition, the difference between accurate and precise measurement was noted, indicating that it is often difficult to provide a testing and measuring environment that responds to the requirement for both accurate and precise measurements.  The conspicuity of lights was addressed, with information provided on the tendency for the eye to ‘remember’ light intensity, thereby reducing conspicuity.  Discussion noted the possible use of additional colours (for example Blue) to increase conspicuity, however IALA has not yet provided information or guidance on the use of Blue Lights in AtoNs.

The session included an opportunity for participation by delegates in viewing different light sources at differing luminous intensities.  O. Eriksson presented a device which enabled viewing of an incandescent based light and a LED based light through the same optical path.  An observation exercise was performed to determine if any difference could be observed between the perception of these light sources.  Workshop delegates were asked to act as observers in the exercise, and results of the experiment are included in Annex 5.

A demonstration was provided by J. Unterderweide on the research being done into sector lights using LEDs.

In addition to the presentations during the session, demonstrations were provided in the evening to make use of natural darkness lighting conditions and greater viewing distances.  These demonstrations were provided a very practical basis for the viewing of different light sources and intensities.  Generally, the presentations and demonstrations highlighted the difficulties in specifying the performance of LEDs.

## Presentations on New Power Sources

| - Developments in Power Sources since 2001 - Developments in Solar Photovoltaic Systems - Battery Technology (Vanadium) - Battery Technology (Lithium) - Fuel Cell Developments - Italian Lighthouse Fuel Cell Project | A. Trojanowski (Automatic Power Inc) R. Wurtz (BPSolar) O. Ibsen (OI Electric) G.Cunty (CETMEF) J. Larminie (Brookes University, Oxford) A. Tomasi (Society SGS Future) |
| --- | --- |

The session began with an overview review of the existing IALA documentation, highlighting areas where developments have occurred.  Various approaches to power sources were identified, including the developments in solar cell technology, vanadium cells and lithium ion cells.  The possibility for practical applications of fuel cell technology in aids to navigation was presented, with an update on an existing project in Italy to use fuel cells for a small lighthouse.

The workshop expressed its great appreciation to all the authors and speakers for the valuable information provided and the high quality of the presentations

# Technical break-out Sessions

Three groups were set up to consider the existing Guidelines and prepare comments and suggestions on them to assist IALA in finalising the document.

WG 1 – Review of IALA Guidelines on Solar Photovoltaic Systems for Aids to Navigation (Chairman Roger Beauchesne, assisted by Roddy MacKay) (amended to WG1 – Power)

WG 2 – Review of IALA Guidelines for New Light Sources and Associated Power Supplies (Chairman Lars Mansner, assisted by Malcolm Nicholson) (amended to WG2 – Light)

WG 3 – Review of IALA Recommendation E-122 on the Photometry of Marine Aids to Navigation Signal Lights (Chairman Ian Tutt, assisted by Rory McGee) (amended to WG3 – Photometry)

It was determined that the existing Guidelines on Solar Photovoltaic Systems for Aids to Navigation and Light Sources and Associated Power Supplies could be better presented if split into two new Guidelines – one concentrating on Power and the other on Light Sources.  The Working Groups were adjusted accordingly, with WG1 concentrating on the creation of a Guideline on Power Sources and WG2 concentrating on the creation of a Guideline on Light Sources.  The presentations and existing documents were used to provide the basis for the work.

The technical sessions concluded with a plenary meeting at which the Chairmen reported on the outcome of the discussions of the groups.

Through discussion it was agreed that the IALA workshop process provided an excellent forum for exchange of ideas, however the timing was such that the resulting documentation requires further review.  To formalize this process, an IALA documentation quality assurance process for use after IALA workshops was developed.  This process will be used on a trial basis with the results of this workshop, and the process will be provided to Council for formal review and approval.

In accordance with this draft QA process, the outputs from the three groups will be prepared by the IALA Secretariat and then forwarded to all participants for review and comment.  Comments will then be tabulated and grouped as Editorial, Clarification or Technical in nature.  Revisions will be made, and the document will then be sent to the Technical Committee (Workshop Steering Committee and Working Group Chairmen) for final revisions.  The documents will then be taken to IALA Council for review and approval.

## Report from Working Group 1 – Power

R. Beauchesne reported on behalf of the WG.  The working process included both WG plenary sessions and sub-group work.  The Draft Guidelines on Power Sources for Aids to Navigation were created using existing text, input from the presentations and input from the experts in the group.  Section 6 – Solar Photovoltaic Systems – was significantly expanded and a new Section 7 – Lifecycle Management was added.  It was noted that Section 3 of the Draft Guideline deals with Calculating Electrical Load, which is also covered in the IALA Guidelines on a standard method for defining and calculating the load profile of aids to navigation, Dec. 1999, and that this section should be reviewed carefully for possible duplication and inconsistencies.

The Working Group requested that the tables in the document be reviewed carefully by experts, including experts in the EEP Committee.

## Report from Working Group 2 – Light

L. Mansner reported on behalf of the WG.  The working process included both WG plenary sessions and sub-group work.  The initial step was to identify the scope of the new document, which focuses on the light source itself, and not the lantern.  The Draft Guidelines on Light Sources for Aids to Navigation were created using existing text, input from the presentations and input from the experts in the group.  A section on Definitions needs to be finalized, providing definitions for those terms used within the document.

It was noted that the focus of the Guideline is very specific, and that there is a requirement for development of further guidance on the lanterns for aids to navigation, including considerations when purchasing an integrated unit.

## Report from Working Group 3 – Photometry

I. Tutt reported on behalf of the WG.  The working group process dealt with IALA Recommendation E-122 in WG plenary sessions.  The Draft Revision to E-122 included a review of the document layout, amendment and update of content and the identification of further work required.

The update of the document included:

- Inclusion of topics on new technology (LED’s);

- Clarification of measurement procedures with the addition of diagrams;

- Deletion of all reference to colour measurement, except where directly relevant to photometry;

- Revision and update of presentation of measurement results was reviewed and updated.

It was noted that there is requirement for further work, which will include the inclusion of a definitions section, as well as the creation of a shortform test sheet example.  Long term requirements include close liaison with CIE as they develop the Uncertianty aspect, as well as the requirement to promote work to secure international agreement on methods to measure effective intensity.

# Discussion Sessions

A session to discuss other matters relating to lights and power sources that were of concern to participants was held.  The topics for this session were developed from the results of the Challenge Question introduced during the opening of the Workshop.  The Session was chaired by S. Doyle, assisted by P. MacMahon, and provided an opportunity to identify many matters of relevance to the participants.  A report of the discussion session is included in Annex 4.

# Technical Tour

A technical tour to the Baily Lighthouse provided…. [text from Colin Day to come]

# Conclusions

The following conclusions were agreed by the workshop:

- This workshop was considered to have been successful and to have met its objective by amending not only the content but also the format of the IALA Guidelines on Solar Photovoltaic Systems for Aids to Navigation and the IALA Guidelines for New Light Sources and Associated Power Supplies and the IALA Recommendation E-122.

- There is a requirement for guidance on aspects relating to the integration of both light and power sources into modular units (e.g. self-contained lanterns, reduction of light output in times of poor solar insolation).

- There is a requirement for a new model for the determination of effective intensity of pulsed light sources applicable to all flash profiles.  Further work needs to be carried out on intensity measurement methodology for light sources. The results of the observations carried out during the workshop indicate that low intensity coloured LED based light sources are not perceived to be brighter than incandescent based light sources, however differences in flash profile may result in differing perception of brightness.

- There is a requirement for a quality assurance process to be put in place to ensure that documents developed through the workshop process are complete, accurate and clearly understandable.

- The existing IALA Recommendation for the colours of light signals on aids to navigation, December 1977, needs to be reviewed to ensure it reflects recent developments including guidance on suitable methods for colour measurements.

- Current developments in lithium battery technology indicate the promise of extended maintenance free intervals for AtoN.

- The trend in power systems for AtoN is towards PV solar technology. The developments in efficiency of the PV technology allow a greater use of this power source without increase in cost per watt.

- The trend towards short production life of PV solar products necessitates a careful consideration of the spares required over the full life cycle of the system.

- Fuel cells continue as an emerging technology and may be applicable for use on small to medium sized AtoN.  Direct methanol systems show the greatest promise.

- Many authorities now regard LED light sources as the desirable option for low to medium intensity AtoN lights.  At present, LED options are not available for higher intensity AtoN installations.

- When considering AtoN light sources, there is a need to address the complete optical system.

- Traditional optic equipment can be re-used with modern light sources, such as clusters of lamps or LEDs or single high-powered LEDs.

# Recommendations

The following Recommendations were made by the workshop:

- IALA should develop guidance on aspects relating to the integration of both light and power sources into modular units (e.g. self-contained lanterns, reduction of light output in times of poor solar insolation).

- IALA should work proactively with other international organizations to promote research and development on aspects of light and power sources for aids to navigation.

- IALA should develop a quality assurance process for documents created through the workshop process.

- IALA should undertake to review the existing IALA Recommendation for the colours of light signals on aids to navigation, December 1977.

- IALA should monitor and encourage current developments in [rechargeable] lithium battery technology which indicate the promise of extended maintenance free intervals for AtoN.

- IALA should monitor the trend in power systems for AtoN towards PV solar, where developments in efficiency of the PV technology allow a greater use of this power source without increase in cost per watt.

- IALA should monitor the trend towards short production life of PV solar products as this necessitates a careful consideration of the spares required over the full life cycle of these systems.

- IALA should monitor Fuel cells as an emerging technology, noting that direct methanol systems show the greatest promise.

- IALA should develop guidance on the use of modern light sources, such as clusters of lamps or LEDs or single high-powered LEDs in traditional optics.

# Final Plenary and Closing of the workshop

A draft report of the workshop was considered, amended and accepted as representing the results of the meeting.

During the closing of the meeting Mr. M. Boyd expressed the appreciation of the Commissioners of Irish Lights to participants for the part they had played toward achieving such a successful outcome.  He then presented a placque to IALA to commemorate the event of IALABATT/IALALITE 5.

Mr Kruuse expressed the appreciation of the workshop for the excellent arrangements that had been made by CIL and, in particular, the efforts of S. Doyle, F. Lee and P. MacMahon.  He also thanked the Steering Committee of the workshop for the arrangements of the workshop, including the facilities that had been made available, the demonstrations that had been arranged and the social programme.  Mr T. Kruuse then expressed the gratitude of IALA for the work that participants had undertaken and for the very good results that had been achieved and wished participants a safe return to their homes.  He then presented Mr. T. Johnson of CIL with a placque to commemorate the event of IALABATT/IALALITE 5.

# Acknowledgements

The Workshop wishes to acknowledge the contributions made by the Commissioners of Irish Lights for sponsorship of the Ice Breaker Reception and the Workshop Banquet.

## Displays

During the Workshop there were many displays and demonstrations of light and power sources.  The workshop wishes to acknowledge those who displayed:

- Hydrosphere / Veg Industries Limited

- BP Solar / BecoSolar

- Sealite

- Sabik

- Tideland Signal Corporation

- Automatic Power / Pharos Marine

- GLS Research and Development

*    *    *

## Annex 1 - Programme

Tuesday, Sept. 28th, 2004

| Time | Activity |  | Presenter | Chairman |  |
| --- | --- | --- | --- | --- | --- |
| 0830-0900 | Registration / Welcome Coffee |  |  |  |  |
| 0900-1000 | ### Session 1 – Opening Chairman, Commissioners of Irish Lights Secretary General, IALA |  | T. C. Johnson T. Kruuse | S. Doyle |  |
|  | Review of Past Workshops / Overview of Workshop aims User Experience User Experience |  | J. Carson-Jackson, IALA S. Burrows, CIL L. Jaeger, USCG |  |  |
| 1000-1030 | Coffee Break |  |  |  |  |
| 1030– 1245 | Session 2 – New Light Sources |  |  |  |  |
|  | New Light sources – Clusters and Metrology Issues The Photometry of Flashing Lights – Are We Getting The Right Answer? Developments in LED Technology and Recent Research on Human Perception of LED’s New Light Sources – Vision and Colour Aspects Demonstrations - Perception of LED versus Incandescent lights - LED based sector light | I. Tutt, The GLA R&D Department D. Gibbs, National Physics Laboratory T. Nurmi, Tampere University M. Nicholson, The GLA R&D Department O.Eriksson J. Unterderweide |  |  | O. Eriksson |
| 1245-1345 | Lunch |  |  |  |  |
| 1345-1545 | Session 3 – New Power Sources |  |  |  |  |
|  | Developments in power sources since 2001 Developments in Solar Photovaltaic Systems Battery Technology (Vanadium) Battery Technology (Lithium) Fuel Cell Developments Italian Lighthouse Fuel Cell Project | Mr. A. Trojanowski (Automatic Power Inc) Mr. R. Wurtz (BPSolar) Mr. O. Ibsen (OI Electric) Mr. G. Cunty (CETMEF) Mr. J. Larminie (Brookes University, Oxford) Mr. A. Tomasi (Society SGS Future) |  |  | S. Doyle |
| 1545-1615 | Coffee Break |  |  |  |  |
|  | Session 4 – Set up of Work Groups |  |  |  |  |
|  | General discussion on status of IALA documents | J. Carson-Jackson |  |  | O. Eriksson |
|  | Establishment of three working groups: WG 1 – Review of IALA Guidelines on Solar Photovoltaic Systems for Aids to Navigation (Chairman Roger Beauchesne) WG 2 – Review of IALA Guidelines for New Light Sources and Associated Power Supplies (Chairman Larry Jaeger) WG 3 – Review of IALA Recommendation E-122 (Chairman Ian Tutt ) |  |  |  |  |

Evening Activity – 1800 – 1930 Welcome Reception – Drawbridge Tavern followed by AtoN light demonstration, weather permitting.

Wednesday, Sept. 299h, 2004

| Time | Activity | Location |
| --- | --- | --- |
| 0900-0910 | Administrative Details – Viking 2 |  |
| 0910-1030 | ### Session 5– Working Groups (develop preliminary Texts) WG1 WG2 WG3 | Viking 1 Viking 2 Viking 3 |
| 1030-1100 | ## Coffee Break |  |
| 1100-1230 | ### Session 6– Working Groups (develop preliminary Texts) WG1 WG2 WG3 | Viking 1 Viking 2 Viking 3 |
| 1230-1400 | ## Lunch |  |
| 1400-1530 | ### Session 7– Working Groups Continue WG1 WG2 WG3 | Viking 1 Viking 2 Viking 3 |
| 1530-1600 | ## Coffee Break |  |
| 1600-1715 | ### Session 8– Working Groups Continue WG1 WG2 ### WG3 | Viking 1 Viking 2 Viking 3 |
| 1715-1730 | WG Chairmen report discussions to Steering Committee |  |

Evening Activity – 1800 – 2000 Bus Tour of Dublin with Commentary.

Thursday, Sept. 30h, 200

| Time | Activity | Location |
| --- | --- | --- |
| 0900-0910 | ## Administrative Details – Viking 2 |  |
| 0910-1030 | ### Session 9– Working Groups (Finalize Texts) WG1 WG2 WG3 | Viking 1 Viking 2 Viking 3 |
| 1030-1100 | ## Coffee Break |  |
| 1100-1230 | ### Session 10– Working Groups (Finalize Texts) WG1 WG2 WG3 | Viking 1 Viking 2 Viking 3 |
| 1230-1400 | ## Lunch |  |
| 1400-1530 | ### Session 11– Plenary Working Groups report on results of discussions | Viking – Chaired by J. Carson-Jackson |
| 1530-1600 | ## Coffee Break |  |
| 1600-1730 | ### Session 12– Discussion ### Discussions on matters relating to lights and power sources raised by participants. | Viking – Chaired by S. Doyle |

Evening Activity – 1900 – 2200  Workshop Banquet (Viking Rooms).  
(Dress - Smart Casual)

Friday, Oct. 1st, 2004

| ##### Time | Activity | Presenter | Chairman |
| --- | --- | --- | --- |
| 0800-1230 | ### Session 13-14 – Technical Tour |  |  |
|  | Baily Lighthouse and museum | CIL |  |
| 1230-1400 | ## Lunch |  |  |
| 1400-1500 | Session 15 – Conclusion Review of draft Report, Conclusions and Recommendations |  | J. Carson-Jackson |
| 1500-1530 | Closing of the workshop Commissioners of Irish Lights; IALA | M. Boyd T. Kruuse | S. Doyle |

## Annex 2 – Participants [corrections to be included]

|  | Country | Name | Organization/ Address | E-mail | Tel/Fax |
| --- | --- | --- | --- | --- | --- |
|  | ARGENTINA | GARCIA , Sebastian | Hidrovia SA _ Associate Member | gbasgar@iplanmail.com.ar | + 54 11 4320 6900 + 54 11 4320 6931 |
|  | AUSTRALIA | PROCTER, Chris | SEALITE_ | Chris.procter@sealite.com.au | + 61 35 977 6128 + 61 35 977 6124 |
|  | CANADA | BEAUCHESNE, Roger | CCG | Rbeauchesne@dfo-mpo.gc.ca | + 1 613 998 1404 |
|  | CANADA | DRABIT, Mimi | CARMANAH TECHNOLOGIES_ | mdrabit@carmanah.com | + 1 250 380 0052 + 1 250 380 0062 |
|  | CHINA | YONGQIANG, Lu | China MSA | luyongqiang@shamsa.gov.cn | + .86.21.539.315.57 + .86.21.539.315.00 |
|  | CHINA | ZHIJIANG, Shen | China MSA | northsea@enorth.com.cn | + 86.22.283.019.70 +. 86.22.283.019.70 |
|  | DENMARK | ERIKSSON, Omar Frits | ROYAL DANISH ADMIN | ofe@frv.dk | + 45 32 68 95 98 + 45 32 68 96 34 |
|  | DENMARK | IBSEN, Ove | Oi ELECYTRIC_ | oi@oi-electric.dk | + 45 873 764 01 |
|  | DENMARK | J R PETERSEN | ROYAL DANISH ADMIN | jrp@frv.dk | + 45 58 36 00 42 + 45 58 36 00 31 |
|  | DENMARK | SCHMIDT, Dan Kaarsberg | ROYAL DANISH ADMIN | dks@frv.dk | + 45 32 68 95 85 + 45 32 54 08 82 |
|  | ESTONIA | KAARMANN, Leo | ESTONIAN MARITIME ADMINISTRATION | Leo@vta.ee |  |
|  | ESTONIA | PALGI, Tiit | ESTONIAN MARITIME ADMINISTRATION | Tiit.palgi@vta.ee | + 372 6 205 692 + 372 6 205 656 |
|  | ESTONIA | REBANE, Raul-Vello | CYBERNETICA AS | Raul.rebane@cyber.ee | + 372 551 4620 + 372 639 7992 |
|  | FINLAND | JORO, Risto | FINNISH MARITIME ADM | Risto.joro@fma.fi | + 358 400 834 330 + 358 20 448 4680 |
|  | FINLAND | MANSNER, Lars | SABIK OY_ | Lars.mansner@sabik.com | + 358 956 601100 + 358 195 601120 |
|  | FRANCE | CUNTY, Guy | CETMEF | Guy.cunty@equipement.gouv.fr | + 33 4 42 52 74 25 +33 4 42 52 74 01 |
|  | FRANCE | KERGADALLAN, Xavier | CETMEF |  |  |
|  | GERMANY | BOSCHER, Guido | PINTSCH BAMAG _ | g.boscher@pintschbamag.de | + 49 20 64 60 22 83 |
|  | GERMANY | CHRISTENSEN, Morten | SABIK OY_ | mhc@sabik.com | + 49 38 53 99 37 05 + 49 38 53 99 33 90 |
|  | GERMANY | JABER, Khaled | PINTSCH BAMAG_ | k.jaber@pintschbamag.de | + 49 20 64 60 22 83 |
|  | GERMANY | LENHART, Aleksandar | PINTSCH BAMAG_ | a.lenhart@pintschbamag.de | + 49 20 64 60 22 83 |
|  | GERMANY | UNTERDERWEIDE, Jôrg | TRAFFIC TECHNOLOGIES CENTRE | junterderweide@fvt.wsv.de | + 49 261 98 19 24 00 |
|  | IRELAND | BURROWS, Steve | CIL | S.Burrows@cil.ie | + 353 1 632 1978 + 353 1 632 1979 |
|  | IRELAND | McGEE, Rory | CIL | R.Mcgee@cil.ie | + 353 1 632 1982 + 353 1 632 1979 |
|  | IRELAND | FOREMAN, Susie | CIL | s.foreman@cil.ie | + 353 1 632 1937 + 353 1 632 1946 |
|  | ITALY | CONTI, Davide | ITALIAN NAVY |  |  |
|  | ITALY | RINALDI, Vincenzo | ITALIAN NAVY | Navlog.4r.capo1uff@marina.difesa.it |  |
|  | JAPAN | TOYAMA, Osamu | ZENI LITE BUOY CO LTD | o-toyama@zenilite.co.jp | + 81 86 523 0053 |
|  | JAPAN | YAMAKOSHI, Yoshio | OKI ELECTRIC | Yamakoshi223@oki.com |  |
|  | MALAYSIA | ABU HASSAN, Nazri | LIGHT DUES BOARD | nazri@marine.gov.my | + 603 316 852 27 + 603 316 850 20 |
|  | MALAYSIA | KAMARUDIN, Mohamed Razif | LIGHT DUES BOARD | razifk@tm.net.my | + 603 316 852 10 + 603 316 850 20 |
|  | NEW ZEALAND | BARRY, Michael | Vega |  |  |
|  | NORWAY | HOLTET, Rolf | NORWEGIAN COASTAL ADMINISTRATION | Rolf.holtet@kystverket | + 47 701 60175 + 47 701 60101 |
|  | NORWAY | VIK, Oddbjorn | NORWEGIAN COASTAL ADMINISTRATION | Pddbjorn.vik@kystverket | +47 701 601 78 + 47 701 601 701 |
|  | SPAIN | ROMERO, Fernando | LA MAQUINISTA | Fernando.romero@lmvsa.com | + 34 96 140 21 74 + 34 96 1400 962 |
|  | SPAIN | BERNABEAU, Enrique | LA MAQUINISTA | Enrique.bernabeau@lmvsa.com | +34 96 140 21 74 + 34 96 14 00 962 |
|  | SPAIN | CALVO, Carlos | PUERTO DE SANTANDER | carlosccgmja@teleline.es | +.34.94.281.003 |
|  | SPAIN | MARTINEZ, Carmen | PUERTOS DEL ESTADO | cmartinez@puertos.es | + 33 91 524 55 05 |
|  | SWEDEN | KURIN, Sven | SWEDISH MARITIME ADM | sven.kurin@sjofartsverket.se | + 46 1119 1187 + 46 1112 6791 |
|  | SWEDEN | LAGERWALL, Christian | SWEDISH MARITIME ADM | Christian.lagerwall@sjofartsverket.se | + 46 1119 1193 + 46 1112 6791 |
|  | UK | ADAMS, Rob | BECOSOLAR (future member) | rob@becosolar.com | +44.1.803.833.636 +44.1.803.835.379. |
|  | UK | BAKER, Clive_ | THLS | clive.baker@thls.org | + 44 1983 203 408 + 44 1983 294 317 |
|  | UK | MACKAY, R. | NORTHERN LIGHTHOUSE BOARD | roddym@nlb.org | + 44 131 473 31 70 |
|  | UK | MITCHENER, Allen | TIDELAND SIGNAL LTD | amitchener@tidelandsignal.ltd.uk | + 44 1444 87 22 40 |
|  | UK | REID, Andy | HYDROSPHERE UK Ltd (P/O VEGA Industries) | andy@hydrosphere.co.uk | +44. 142 052.03.74 +44.142.052.03.73 |
|  | UK | NICHOLSON, Malcolm | THLS | malcolm.nicholson@thls.org | +44 1983 203465 +44 1983 294317 |
|  | UK | TUTT, Ian | THLS | ian.tutt@thls.org | +44 1983 203410 +44 1983 294317 |
|  | USA | JAEGER, Larry | Signal & Power Team Ocean Engineering Division U.S. Coast Guard | LJaeger@comdt.uscg.mil |  |
|  | USA | TO, P C | TIDELAND SIGNAL CORP | pcto@tidelandsignal.com | + 1 713 681 62 33 |
|  | USA | TROJANOWSKI, Alan | AUTOMATIC POWER,INC | atrojanowski@automaticpower.com | + 1 713 228 5208 |

## Annex 3 – Welcome address by T. Johnson

Mr Torsten Kruuse, Secretary General IALA, Ms Jillian Carson-Jackson, IALA secretariat, guest speakers and  IALA international delegates, you are all most welcome to Dublin. Over the course of the next few days, (and nights) in addition to your workload here, I hope you will have the opportunity to enjoy the hospitality of the city. I warn you however, that in this city, pleasure can very quickly and easily interfere with work!

I have great pleasure in opening this workshop. I think of IALA Workshops as the engine rooms of the organisation. It is at sessions such as these that the practical elements of the various subjects are discussed, dissected and analysed. It is a result of this direct process that recommendations can be made back to the various committees for incorporation in the IALA Guidelines. Such Guidelines are of course the blueprints for marine activities.

This workshop in particular will examine advances in power supplies for Aids to Navigation and also the evolvement of LEDs and LED applications. I myself first became aware of LEDs at the 1998 IALA conference in Hamburg, where it seemed that suddenly we were moving from filament lamp technology to LED technology which was much more robust, dependable and required less power.

Within CIL we have embraced this technology wholeheartedly. All our buoys are now solar powered and most have LED lanterns fitted. I’m pleased to say there has been a corresponding decrease in maintenance costs, as the technology is proving so dependable. Not only that, as a mariner myself, I note the clarity and increased range of such lights.

I have a personal experience of this. One dark evening, on my usual coastal walk near my home, I noted a new flashing red light visible out to sea. I rang up the Inspector and told him I didn’t remember approving a Statutory Sanction for a new AtoN. There was much head scratching and denials of new buoys. There was a search for any applications for local AtoNs such as meteorological or wind farm masts but nothing turned up. It the end the ship was despatched to identify this new light. It turned out that the answer was that an existing buoy off the coast had just been fitted with a new LED, and it was now visible at a far greater range than the previous filament lamp!

I wish to thank our Engineering Department for the organisation of this workshop. Such organisation almost always goes unseen yet there is much work and effort in the background. As we say here, somebody always has to do it and in this case it was Seamus Doyle, ably assisted by Fiona Lee and Pauline MacMahon….or was that the other way around?? So, Seamus, Fiona and Pauline, many thanks for your hard work in organising a superb location and programme.

I wish you all well in your workgroups and your deliberations, conclusions and recommendations. The motto of the Commissioners of Irish Lights is “For the Safety of All”. The motto of this conference is “Lighting the way to a safer navigation”. I think both reflect the aims of this workshop and on behalf of the Commissioners of Irish Lights,  IALA and seafarers in general, I wish to thank you for your continuing contribution to a very important aspect of Safety at Sea.

Thank you.

Mr. Terence Johnson

Chairman, Commissioners of Irish Lights.

*    *    *

## Annex 4 – Challenges Facing AtoN Providers

During the IALABATT/ IALALITE 5 Workshop, participants were asked the question “within the context of new and developing lights and power sources for aids to navigation, what do you feel is the most significant challenge facing your organisation today?” The following summarises the main challenges identified.

- How can new light sources be used in conventional lens systems?

- How can LED lights be used to provide aids to navigation, either alone or with conventional lenses?

- How can LED lights be monitored?

- What will be the function and extent of traditional aids to navigation in say 2025?

- How to collect experiences, knowledge and information from different countries and Authorities and make that information available to other Authorities.

- What standards are required and how should they be provided?

- What are the options for providing reliable AtoNs? What causes unreliability and what are the solutions to these causes?

- What are the maintenance requirements for new light and power sources technology?

- What are the options for extending the maintenance intervals with new technology?

- What are the options for providing self contained lanterns? What are the limitations of self contained lanterns? What are the options for adding AIS and Remote Monitoring to self contained lanterns?

- What methods are available for full life cycle maintenance approach to provision of AtoNs?

- Is vandalism a problem? What are the options for controlling vandalism?

The challenges were categorised and rated to determine the priority of challenges facing AtoN providers today.

LEDs					353

Finance				114

Standards / Specs			77

Fuel Cells				69

Environmental			68

Maintenance 			51

Miscellaneous			147

A number of the above challenges were discussed in Session 15.

How long will lights  be required as AtoNs?

It was generally agreed that lights will be required for the foreseeable future.

What range will be required for AtoN lights?

It was agreed that radio navigation is replacing the longer range lights and the trend is towards reducing the range of long range lights while there is a need to increase the range of short range buoy lights. Range is determined by what technology can provide economically. One suggestion was to replace long range lights with a combination of shore based short range light and a buoy.

It was noted that the range required for a light is very dependent on the background lighting. The increase in the intensity of street lighting obscures AtoN lights and determination of the intensity of an AtoN light must consider its conspicuity in the presence of background lighting as well as the range required.

What are the options for using new light sources in traditional optic systems?

It was agreed that arrays of LEDs can be used in traditional lens systems to provide effective AtoN lights. Clusters of modern lamps such as tungsten halogen surrounded by a vertical diffuser can replace high power lamps to provide very effective long-range lights. A cluster of four 150W lamps was demonstrated during the Workshop and considered to be very effective.

What is the role of LEDs in providing AtoN lights?

The main application of LEDs is to use high power LEDs with inbuilt lens, arranged in clusters, to create lights with or without any additional lens systems. The range is typically up to 10 nautical miles. LEDs can also be used in traditional lens systems as above.

It was agreed that it is unlikely that AtoN lights will convert to 100% LED lights.

Are LEDs brighter than incandescent lights?

Omar Eriksson from the Royal Danish Administration of Navigation and Hydrography presented the results of an experiment that he had carried out earlier in the Workshop. Full details of this experiment are presented in a separate Annex.

The test used a population of 44 participants with age predominantly above 30 years. Four participants had some degree of colour vision impairment. The workshop concluded that, under the conditions of the experiment, the spectral performance of LED and incandescent lights did not affect the perceived intensity of lights but the flash profile or spatial characteristics of the flash did affect the perceived intensity. It was suggested that this resulted from the lower energy in the incandescent flash with sloping leading edge. Results are consistent with Schmitt Clausen predictions. It was agreed that further work using the experimental apparatus could provide valuable additional information. The further work should include equalising the flash energy while varying the pulse shape and the effects of high frequency pulsing within the flash. This will be considered further in EEP Committee.

The relevance of this to on-going CIE work on photometric standards was noted. It was suggested that IALA should take a proactive approach to advancing the CIE work in this area. The need for a single photometric standard that covers all flash shapes was agreed.

What methods are available for full life cycle management approach to providing AtoN’s?

It was noted that the Australian Maritime Safety Agency are using a management system of whole life costs which can include environmental cost implications as well as direct costs of AtoN provision.

It was noted that there is a tendency for Administrations to purchase equipment based on capital cost only and not to include the subsequent operating and maintenance costs or cost of disposal in the purchasing decision. This was contrasted with the demand for LED lights where they objective is to reduce operating and maintenance costs while increasing the level of service through greater reliability. The problems associated with providing AtoN systems in areas prone to severe storm damage were noted. It was suggested the cost of disposing of batteries can be the same as the capital cost of purchasing the battery.

What are the options for providing self contained lanterns?

Self-contained lanterns were defined as a single unit incorporating a light, solar array and battery. It was noted that the specification of self-contained lanterns depended very much on the environment in which it will be used. Parameters such as the amount of sunlight have major importance. Some self-contained lanterns are constructed in modular fashion that allows for greater flexibility in applications.

It was reported that small AIS engines with energy consumption of 1 amp-hour per day can be installed in self-contained lanterns but will cost approximately $4000. As volume increases, it is expected that price will decrease significantly.

AIS can be used as a monitoring system. Alternatively, if remote monitoring is fitted in the lantern, a buoy can be marked with synthetic AIS.

Five delegates indicated a wish to include AIS in self-contained lanterns.

It was agreed that AIS will not replace racons in the foreseeable future, as radar is an independent system under user control and AIS may be susceptible to overloading of the data channels in busy shipping areas.

Do administrations have a problem with vandalism of AtoNs?

Vandalism was considered to be a problem by 10 delegates.

The most effective solution to vandalism was considered to be education and signs on the AtoNs indicating the purpose and importance of the structure. Education of the local population that the AtoN systems are not usable for other purposes reduces the incentive to steal equipment. Anti tamper devices are not very effective as vandals often damage the systems in an attempt to defeat the anti tamper devices. For lighthouse and fixed structures, intruder alarms, video surveillance, etc can provide some protection.

How do maintenance requirements change with the new technologies?

It was agreed that longer life modular devices increases the maintenance intervention intervals. The time on station for maintenance is also reduced. The simpler systems can be maintained from a small boat rather than a large buoy tender. The modular nature of modern technology means that less skilled personnel can be used in the field with module repair carried out in central maintenance workshop or by contractors.

It was noted that administrations practices in visiting buoys varies from monthly visits for position check and cleaning to annual or bi-annual visits for system performance verification. Refurbishment of buoys varies from annual to every five years and up to eight in some cases.

*    *    *

## Annex 5 – Test Results

# Perception of LED light sources - the Dublin Observation

## Introduction

Following the introduction of LED light sources in Aids To Navigation, IALA members have been receiving reports that LED lights are being perceived as brighter and having longer visual range than the traditional incandescent light sources.

As a part of the development of the IALA Recommendation on the Photometry of Marine Aids to Navigation Signal Lights (E-122) in June 2001, attention was drawn to the need for colour correction when performing photometric measurements, in particular with narrow band light sources like LED’s.

Presumably most IALA member laboratory practices now include colour correction, but still the general opinion of IALA members seems to be, that LED lights are perceived as brighter and having a higher effective luminous intensity than incandescent light sources.

IALA recommends the use of the Schmidt Clausen method for calculating the effective intensity of pulsed light sources, and members are now questioning the accuracy of this model when applied to LED based lights, while assuming that measurement errors due to insufficient colour correction now can be regarded as negligible.

At the 2004 IALABATT IALALITE workshop in Dublin, some observations of LED lights versus incandescent lights were carried out using two experimental light sourcing devices developed by the Royal Danish Administration of Navigation and Hydrography.

This report describes the light sourcing devices, and the results of the observations that were carried out.

## Light Sourcing Devices

Two experimental light sourcing devices were used for the observations, one producing red light and one producing green light. Each device contained an integrating sphere, one LED light source, one incandescent light source, a light sensor and a microcontroller based control circuit.

LED light was injected on one side of the integration sphere and incandescent light on the opposite side of the sphere. A light output port was placed at a right angle to both light injection points. The radius of the integrating sphere was about 20mm and the diameter of the output port aperture was 19.2mm. An appropriate red/green optical filter was placed in front of the output port of each device. The devices are illustrated in figures 1 and 2 below.

| fig. 1 The optical/electrical arrangement of the light source devices | fig 2 The light source devices as viewed by the observers. |
| --- | --- |

The microcontroller based control circuit was capable of controlling the current/voltage of each light source individually as well as measuring luminance on the outside surface of the ping pong ball comprising the integrating sphere. The LED’s were driven by a digital current generator while the incandescent light sources were driven by a similar voltage generator using 11 bits A/D and D/A converters.

Using a spectroradiometer, the luminous output of the light sources was carefully matched. Judging from the variation in the reading of the spectroradiometer, the maximum relative difference between the luminous outputs of all four light sources is estimated to be about 5%. The absolute luminous intensity of each light source is estimated to be around 0.05 cd.

The current generator driving the LED’s was capable of dynamically controlling the current of each LED. This was utilised to produce LED based light having a pulse shape similar to that of the incandescent based light sources. The incandescent based light sources were driven by a step function of one second duration resulting in a rise time of about 100 mSec. The pulse shapes obtained with LED’s versus the pulse shapes of the incandescent light sources are shown in figures 3 and 4 below.

| fig. 3 “Incandescent” pulse shapes of the red light sources device | fig 4 “Incandescent” pulse shapes of the red light sources device |
| --- | --- |

The spectral distribution of the light sources in each light source device was measured using a spectroradiometer. These are depicted in figures 5 and 6 below:

| fig. 4 Spectral distribution of the red light source device (both LED and incandescent sources) | fig 5 Spectral distribution of the green light source device (both LED and incandescent sources) |
| --- | --- |

## Observers

The 2004 IALABATT IALALITE workshop participants were screened for colour vision deficiencies using a simple Ishihara Colour Vision Test (http://webvision.med.utah.edu/KallColor.html).

This resulted in 40 observers believed to have a fairly normal colour vision. The age distribution of these observers is depicted in figure 6 below:

fig. 6 Age distribution histogram of the 40 observers from the 2004 IALABATT IALALITE Workshop

## Observations

During the first observation, the observers viewed a pulse sequence of four pulses two red and two green. Due to the design of the light source devices, the spatial/optical properties of the light pulses, as seen by the observers, were very similar. All pulses had a similar incandescent type of pulse shape and thus the only principal difference between the pulses was their spectral distribution. The observers were asked to answer the following questions for the red and green light pulses respectively:

Observe the light pulses carefully and mark one of the statements below as being true:

    A and B are equally bright

    A is brighter than B

    B is brighter than A

Where A and B were the first and second pulse of each colour respectively. The results of the observations are shown in figure 7 below:

fig. 7 Results of the first observation

During the second observation, the observers again viewed a pulse sequence of four pulses two red and two green. As before, the spatial/optical properties of the light pulses where very similar. This time the pulses were generated using the LED sources only. The first LED pulse was driven by a square step function while the second pulse had an incandescent type of waveform. The only principal difference between the pulses was therefore the pulse shape. The observers were again asked to answer the same questions. The results of the observations are shown in figure 8 below: [correction required from Omar – far right column 1<2]

fig. 8 Results of the second observation

## Conclusions

On the basis of the results from the observations carried out, the workshop participants agreed to the following conclusion:

The results of the Dublin Observation indicate that low intensity colored LED based light sources are NOT perceived brighter than incandescent based light sources.

However differences in flash profile may result in differing perception of brightness.

*    *    *