| From: RNAV | ## RNAV21/output/04 |
| --- | --- |
| #### To: EEP; ANM | ## Sept. 17, 2004 |

Liaison Note

from the IALA Radionavigation (RNAV) Committee to

Engineering / Environment / Preservation of Historic Lighthouses (EEP) Committee  and the Aids to Navigation Management (ANM) Committee

# Subject:  Availability, Continuity & Integrity for Maritime Applications

# Introduction

The terms availability, continuity and integrity are defined in IMO Resolution A.915(22) “Future Requirements for GNSS” (Ref. 1). The definition of availability is similar to that used in the analysis of reliability of machinery, but is not the same. The “classic” definitions of availability and reliability are used in the IALA Guide to Availability and Reliability (Ref. 2). The terms continuity and integrity have special meanings for navigation systems and originate in aviation applications, but there are some important differences for maritime applications.

# Background

Use of the terms for regional and global systems as opposed to local systems also presents some difficulties.

As a general principle, service providers should be more concerned with worst-case scenarios than statistical averages. The purpose of this paper is to set out these issues for discussion in IALA.

# Action Requested

The EEP and ANM Committees are invited to review this document, noting that it is a work in progress, and provide any comments and suggestions back to the RNAV Committee.

# Availability, Continuity & Integrity for Maritime Applications

## Availability

Availability is the probability that an aid to navigation is performing a required function under stated conditions at any randomly chosen instant in time. This standard definition does not include any reference to a time period. In the case of radio-navigation systems a time period is usually specified because this determines the maximum period of outage that could occur.

IMO Resolution A.915(22) specifies availability in two contexts:

- Signal availability – the availability of a  radio signal in a specified coverage area

- System availability- the availability of a system to a user, including signal availability and the performance of the user’s receiver

This paper uses signal availability as the individual receiver capabilities and performance are outside its scope. The term signal availability, as used in this paper, does not take into account propagation factors such as atmospheric noise. For a more complete analysis see Ref. 3.

In calculating both availability and continuity, it is necessary to adequately define an outage or failure incident. In this paper an outage refers to both planned (i.e. maintenance) and unplanned system downtime, whereas failure refers only to the latter, on the assumption that users are made aware of planned downtime and modify their intentions accordingly.

Long term catastrophic failures, such as that of a power supply or transmission antenna, are relatively simple to quantify. Short term failures (‘momentaries’ or ‘soft fails’), generally related to software issues in electronic equipment or transient satellite constellation issues, are more difficult to define but are generally assumed to have negligible impact on the user.

It is usually necessary for the service provider to define a threshold limit at which a short term failure reaches a critical level; this is generally considered to be a two dimensional position error of x metres persisting for y seconds. (As an example, DGNSS services are widely considered to reach a critical level if an error of 10 metres persists for greater than 21 seconds (Ref.4). This approach is considered to be both valid and quantifiable.)

## Global Navigation Satellite Systems (GNSS)

High levels of availability can be achieved with a global, multi-satellite system, where the criterion is the number of working satellites visible. The figure of 99.8% over 30 days, in the original resolution, is achievable with a constellation of 24 or more satellites, since more than 4 satellites can generally be seen from any point on the Earth’s surface.

It is possible to produce statistics for the overall availability of a multi-satellite system based on a global average of individual satellite availabilities; however this is limited by geometry and time dependence.

It is also possible to calculate availability for a sample of locations averaged over time. This is measurable, but unlikely to be representative of overall system performance unless the number of stations is sufficient, representative and of good geographic distribution. This is difficult to achieve for a system with world-wide coverage.

The GPS SPS Service Performance Standard (Ref. 5) defines a global average horizontal service availability (HSA) of 99%, based on the expected horizontal error being less than 36 metres (95%). The expected positioning error is a predictive statistic, and is based on a combination of position solution geometry and predicted satellite ranging signal errors. It incorporates a separately stated standard for PDOP, is based on a 24 hour period and uses only satellites flagged as “healthy”. The standard further defines a worst-case location HSA of 90%, based on the same criteria.

The latter in particular represents an improvement on previous statements, however uses a (time domain) availability percentage (90%), based on a distance percentage (95%) which may result in some confusion.

The Galileo Mission Requirements Document (Ref. 6) indicates an availability of integrity of 99.5% throughout the coverage area, to be achieved by a combination of the Signal in Space (SIS) availability (99.8%) and receiver provided integrity (RAIM) with availability of 60%.

## Satellite Based Augmentation Systems (SBAS)

The availability of usable SBAS corrections is dependent on the individual availabilities of  ground based reference and uplink stations, and of the geo-stationary broadcast satellites. The former are sufficiently numerous for a small number of simultaneous failures not to cause loss of usable correction data, the uplink stations have multiple redundancy, hence the satellites themselves are the single greatest point of failure. Most locations within the coverage area will receive signals from two broadcast satellites; where this is the case the site can be considered as a dual coverage scenario, with an OR function. There will however be some sites where local topology masks one or even both satellites, resulting in single satellite coverage or no coverage, particularly at extreme latitudes.

Additionally, there may be periods of non-availability of signal resulting from man-made or atmospheric interference to the L-band signals, although the beamwidth is narrower, and hence signal levels are greater, than with GNSS. These effects are most pronounced at low or very high latitudes.

## Terrestrial Systems - Ground Based Augmentation Systems

High levels of availability are difficult to achieve with terrestrial systems such as beacon DGNSS or Eurofix, relying on individual transmitters in remote locations. 99.8% over 30 days means a maximum single outage period of less than 1.5 hours. This cannot be guaranteed on a remote site, since access times can often be more than a day. For this reason the probabilities for augmentation systems such as DGNSS are calculated over two years and 99.8% would only be achieved in areas of dual coverage (99.5% in single coverage).

Where genuine dual coverage exists, the signal availability at that location can be considered as an OR function of the individual adjacent station broadcast availabilities, however there is still a risk of common mode failures due to use of identical reference station firmware. Where single coverage is achieved, and assuming propagation factors are excluded, the signal availability can be considered as equal to the individual station broadcast availability.

One approach is to divide the coverage area into sub-areas, such as those used for maritime weather forecasts (see below). The coverage in each area can then be expressed as a function of the local broadcast station availabilities e.g.

Availability (Malin) = Tory Island + Butt of Lewis

Butt of Lewis

Tory Island

Figure 1: Availability sub-areas

Another method of producing availability statistics is to conduct a periodic review of adjacent station availabilities to view any correlated outages.

A further method of calculating and presenting availability, continuity and coverage regions for both single radiobeacons and networks of beacons has been developed by Grant [Ref. 3].  This model takes into account signal propagation factors such as atmospheric noise, interference and the two modes of propagation, and has the capability to model achieved availabilities as well as operating as a predictive tool.

Fig 2 shows an example output of this model, plotting availability; the different colours are used to identify where the standards are met.  Rather than supply a single worst case figure for availability, by utilising this software the service provider can identify and present the availability for all locations of their responsibility and show in a simple manner where the service is achieving its goal.

Figure 2: Two-year signal availability across the EMA.

Light blue = 99.8% (critical areas requirement met),

Dark blue = 99.5% (other areas requirement met).

An alternative approach would be to assess the average of a number of sample locations. This is valid, if the number of stations is sufficient, representative and of good geographic distribution, however these are subjective issues and service providers should be more concerned with worst-case scenarios than average values.

## Terrestrial Systems – Hyperbolic Systems

For a hyperbolic system availability is an AND function of individual transmitter sites, requiring full functionality of the relevant Master and two Secondary transmitters. In some areas coverage is provided by more than one transmitter chain, introducing some redundancy, however this is weakened by the re-use of transmitter sites in more than one chain.

Example: A user transitting the Dover Straits could use the Lessay Chain, with secondaries at Soustons and Sylt. If this became unavailable he could switch to the Sylt Chain, with secondaries  at Lessay and Værlandet. It can be seen, however that a complete failure of either the Lessay or Sylt sites will make both chains unusable in this area.

Statistically this is represented by availability = L.Sy.So + Sy.L.V

= L.Sy.(So+V)

The use of “all-in-view” receivers will lead to some increases in usable signal availability.

## Continuity

This term as defined in Ref. 1 is essentially the same as availability, but over a much shorter period, corresponding to the duration of a typical manoeuvre. For maritime applications a period of 3 hours, corresponding to a typical harbour approach or transit through a confined area, is widely accepted. A shorter period of, perhaps, 5 minutes would correspond to a typical docking evolution. As with availability, the discussion below does not include signal propagation factors or short-term failures.

Continuity requirements of 99.97% over 3 hours in high risk areas, or 99.85% in areas of lower risk, are specified in IMO Resolution A.953(23) “World-Wide Radionavigation System” (Ref.7). Continuity (reliability) is defined in the IALA Guide in the same way, but measured in terms of MTBF (Mean Time Between Failures) and MTTR (Mean Time To Repair). That approach has also been proposed for Continuity (Ref. 8), but using MTBF alone, since that determines the probability of continued service over a short operational period.

Basing the calculation of continuity on MTBF alone makes the assumption that non-failure system outages will be notified to the user in good time by appropriate means, and that the user will take appropriate action to nullify the predictable effects, such as by delaying the manouvre until the outage period is complete or by using other aids to navigation to complete the manouvre. For GNSS, this raises questions as to whether planned outages (e.g. NANUs) are notified to users in a usable format, and whether the user would be able to recognise a fault scenario at the beginning of a manouvre period.

Assuming the service is functioning at the beginning of the operation, the probability that the service will be available after a time CTI, that is the continuity is:

C = exp (-CTI/MTBF)

If MTBF is very much greater than CTI, this can be approximated to:

C = 1 – (CTI/MTBF)

Or, where an aid is provided independently from two sources, e.g. two GNSS sources or beacon DGNSS with genuine dual coverage and a ‘seamless’ two-channel receiver:

C = 1 – {(CTI/MTBF1 ).(CTI/MTBF2  )}

Where:

MTBF = Mean time between failures; based on a 2 year averaging period

CTI = Continuity time interval; in the case of maritime continuity equal to 3 hours

In each case the availability of ‘seamless’ two-channel receivers and the algorithms used to switch between channels are points of concern.

The GPS SPS Performance Standard quotes an average reliability of 99.94% for GPS, with a worst case of 99.79%. Both these figures are based on a threshold User Range Error of 30 metres for any one satellite, a measurement interval of one year and an average of daily values. The Galileo Integrity Concept Discussion Paper (Ref. 9) quotes a continuity risk (i.e.1-C) of 3.10-4 over a 3 hour period for all maritime applications.

## Integrity

One of the key factors in any navigation system is integrity, which is defined as the capability of warning the user when the system should not be used. The role of integrity is to reassure the user that the system is functioning correctly, whilst minimising the probability of false alarms.

Integrity has a number of criteria:

- (Horizontal) Alert Limit (HAL); HAL is customarily set at 2.5 x the (95%) accuracy. The probability of HAL being exceeded can be predicted for normally distributed errors.

- Time To Alarm (Alert) (TTA); The TTA requirement has been accepted as 10 s, although a figure of 2 s has been proposed for High Speed Craft (HSC).

- Probability of Missed Detection (Pmd). Pmd is referred to as Integrity Risk in Ref. 2 and the requirement is stated to be 10-5 over 3 hours. Pmd is dependent on the monitoring procedures in place and may not be predictable.

- Probability of False Alarm (Pfa); No figure is given for Pfa. The achieved levels of Pfa will vary from system to system and will be related to Pmd, TTL and HAL.

The Galileo Integrity Concept Discussion Paper defines an integrity risk as the overall probability that a position error alert shall not be given within the TTA of 6 s  shall be less than 1.10-5 over any continuous 3 hour period (for maritime applications). This includes the user integrity risk contribution of 1.5.10-7 over any 150 s. A HAL of 25 metres is further defined.

The GPS SPS Performance Standard quotes the probability of (receiving) Hazardously Misleading Information (HMI) shall be less than 0.002 (presumably over one year), and defines this in terms of the individual satellite User Range Error and User Range Accuracy figures.

## References

- International Maritime Organisation (IMO), 2001. Maritime Policy for a Future Global Navigation Satellite System (GNSS). Resolution A.915(22).

- IALA 1989. IALA Guide to the Availability and Reliability of Aids to Navigation.

- International Maritime Organisation (IMO), 2003. World-Wide Radionavigation System. Resolution A.953(23).

- Revised. IALA Recommendation on the Performance and Monitoring of DGNSS Services in the Frequency Band 283.5-325 kHz. R-121. 2004.

- US Department of Defense (DoD), October 2001, Global Positioning System Standard Positioning Service Performance Standard.

- GISS 2003. Galileo Mission Requirements Document, Issue 5, Rev 1.1.

- Grant. 2002. Availability, Continuity and Selection of Maritime DGNSS Radiobeacons. PhD Thesis, University of Wales, Bangor.

- Fairbanks, 2003. Augmentation Systems Study, RASR001. Draft Requirements Report for the GLA.

- ESA 2003. Discussion Paper on the Galileo Integrity Concept, Draft Issue 1.0.