Draft SN/Circ.XXX

July 2006

# INTERFACING OLDER GNSS UNITS WITH AIS

GNSS is used as the primary source of position, navigation and timing information to onboard systems, including AIS. However, there is lack of awareness of the operational risk inherent in interfacing older GNSS units with other onboard systems.  Member Governments are therefore requested to bring this information to the attention of the relevant authorities and all seafarers for guidance and action, as appropriate.

# Background

There are currently two IMO performance standards for GNSS receiver equipment:

- if installed on board on or after 1 July 2003, it needs to conform with performance standard MSC 112(73) (the "current" standard);

- if installed on board before 1 July 2003, it needs to conform to standards specified in Res. A.819 (19)  (the "old" standard).

The differences between the two standards are considerable.  In the ‘old’ standard  GNSS receivers are not required to have Receiver Autonomous Integrity Monitoring (RAIM), and are therefore unable to offer an indication of the integrity of the positional information being broadcast. While some older receivers may have had a course over ground (COG) and speed over ground (SOG) output, there was no standard specified for these parameters. Older units may have a lower update rate (two seconds, as opposed to one second updates for newer units) and be less able to cope with signal interference.

The issue

A number of vessels are equipped with GNSS receivers that conform to the ‘old’ standard.

With the increasing reliance on GNSS by systems such as ECDIS, AIS and INS, it is vital that the maritime community is made fully aware of the operational risks of interfacing such systems with older GNSS units.  The lack of integrity monitoring represents a defect in the system, whereby older receivers can continue to use a faulty satellite and ships fitted with them may use an incorrect position and transmit that incorrect position by AIS.

It is strongly recommended that only GNSS receivers conforming to the ‘current’ standard  should be used on SOLAS ships.

Although ‘current’ standard GNSS receivers have RAIM capability, this functionality relies on a certain number of satellites being available and integrity cannot always be guaranteed. The use of DGNSS receivers to further enhance integrity is therefore recommended.

Note that neither IMO Resolution A.917 (22) (Guidelines for the onboard operational use of AIS) nor the SN/Circ.227 (Guideline for the installation of a shipborne AIS) make reference to this issue.

Note that AIS units installed before 1 July 2003 will not necessarily be fitted with internal GNSS receivers conforming to the new standard.