# Liaison Note

## Assignment, Channel Management and Interrogation for AIS AtoN Stations

# 1 Introduction

In Liaison Note WG14-13-output-01, the IALA e-Nav Committee is requested to comment on the proposal regarding the removal of the requirement for AIS AtoN Stations to respond to Message 15 (Interrogation), Message 16 and 23 (Assignment), and Message 22 (Channel Management).

# 2 Reply

The AIS Technical working group of the IALA e-Nav Committee considered this issue at the committee meeting in September 2006, and considering that

- some AIS AtoN stations may not be equipped with receivers, and thus not able to respond

- if AIS AtoN Stations were to respond to assignment using Messages 16 or 23 or Channel Management using Message 22, their behaviour could be changed via the VDL in an unauthorised manner

- other means of managing parameters of AIS AtoN stations via the VDL using binary messaging are being considered and standardized by WG14

- these other means of managing parameters can take the issue of secure authentication of the originator into account

The IALA e-Nav WG4/AIS agrees to the proposal, that AIS AtoN Stations should not be required to respond to Message 15 (Interrogation), Message 16 and 23 (Assignment) and Message 22 (Channel Management).

The e-Nav WG4/AIS will take this into account during the revision of IALA recommendation A-126 and the Technical Clarifications to ITU-R M.1371-3.

The e-Nav WG4/AIS encourages the WG14 to carefully consider a secure authentication mechanism, taking into account a time factor or similar, to ensure that management commands sent via the VDL cannot be misused by simply copying a command from the VDL and retransmitting this command at a later point in time.