Page 194 of 220

5.2.2 Transmission of Base Station Report (msg 4) at the default report-ing

rate

Assignment of the transmission of Base Station Report (msg 4) at the default report-ing

rate is justified, because a msg 4 with a valid position allows all mobile AIS sta-tions

to correlate this position with the transmission(s) of Data Link Management msg

20 using the same MMSI. The correlation is used by a mobile AIS station to deter-mine

whether the 120 nm intentional slot reuse rule should be used. This is an essen-tial

AIS VDL management rule because it effectively counteracts the potential block-ing

of a large number of slots by the reception of non-relevant msg 20 from distant

good RF propagation.16

When defining the usage of Base Station Report (msg 4) at the default reporting rate,

the following aspects should be taken into account:

- Table 1B of Recommendation ITU-R M.1371-1 states, that the default reporting

rate by a base station should be once per 10 seconds.

- Therefore, the transmission of Base Station Reports exhibits a short-term repeti-tive

pattern.

- This translates into six (6) slots reserved for that purpose per frame for a reporting interval of 375 slots, this translates into a FATDMA  increment of 750 (msg  20) in one FATDMA reservation on each channel.

Usage recommendation:

When transmitting Base Station Reports (msg 4) with default reporting rate by one

base station,

a) there should be one (1) slot reserved with an increment of 750 on each channel, and

b) the required FATDMA reserved slots should be allocated by a Continuous FAT-DMA

reservation (see above).