| Received: 25 August 2006 | INTERNATIONAL TELECOMMUNICATION UNION |  |
| --- | --- | --- |
|  | RADIOCOMMUNICATION STUDY GROUPS | Document 8B/501-E |
|  |  | 25 August 2006 |
|  |  | English only |

Source:		Doc. 8B/146

Telenor AS

DRAFT NEW RECOMMENDATION ITU-R m.xxx

Characteristics of VHF Radio System and Equipment for the Exchange of Data and E-Mail on Maritime Appendix 18 Channels

# Scope

1	This document describes a digital communications system for the exchange of data and e-mail in the maritime mobile service on VHF Appendix 18 channels. It contains proposals for channel usage and technical characteristics of such a system.

2	These proposals are within the scope of resolution 342 (Rev. WRC-00) of agenda item 1.14 WRC-07.

3	By adopting Resolution 342, WRC-00 modified Appendix 18 to permit maritime VHF frequencies to be used for data transmission on a trial basis, and WP8B has considered reports from the operation of systems for the exchange of data and E-mail on VHF channels.

The ITU Radiocommunication Assembly,

considering

a)	that resolution 342 of World Radio Communication Conference (Rev. 2000) invites ITU-R to finalize studies currently ongoing, inter alia:
–	identify the future requirements of the maritime mobile service;
–	identify suitable technical characteristics of the system or interoperable systems;
–	identify necessary modifications to the table of frequencies contained in Appendix 18.
b)	that IMO has stated that the maritime industry has need for safe, fast and inexpensive communications for business and safety. At IMO COMSAR 8 the future need for harmonization of systems using maritime VHF channels were considered, and the sub-committee decided to send a liaison statement to ITU WP 8B, bringing to the attention of the working party the possible future need for world-wide systems for exchange of data and e-mail on maritime VHF channels.

recommends

1	that the characteristics for VHF data described in annex 1 to this recommendation should be considered as an example of such systems:
2	this recommendation should be used as a guideline for future digital technologies in the maritime VHF bands;
3	that efforts should be made to make the most efficient use of spectrum resources;
4	that the frequency table in Appendix 18 should be modified according to Annex 2 to accommodate the digital transmissions. The bandwidth needed is dependent upon the wanted speed of data exchange, and it is important to enable flexible usage of the maritime VHF duplex channels allocated for data transmission.

Any questions regarding this document may be addressed to:

Ånund Haktorson
Telenor Maritime Radio
P.O. Box 7150
N-5020 BERGEN
NORWAY
E-mail:		anund.haktorson@telenor.com
Phone:		+47 55 96 83 80
Fax:		+47 55 96 83 91
Mobile:		+47 913 34 862

Annex I

Proposed VHF Data System Characteristics

The following characteristics should be met by a VHF radio system for the exchange of data and e-mail in the maritime mobile service, operating on frequencies specified in ITU RR Apendix 18 for data transmissions.

# 1	General Characteristics

1.1	The class of emission should be 25K0W1DDN / 225KW1DWN.

1.2	The necessary band should cater for at least 9 narrowband duplex channels, each with 25 kHz bandwidth, and 2 broadband simplex channels, each with 225 kHz bandwidth, actual bandwidth usage being dependent upon the data exchange transmission speed required.

1.3	The modulation should be 4-level or 2-level GMSK, depending on transmission speed.

1.4	The access method should be Time Division Multiple Access (TDMA).

1.5	The following area coverage techniques should be used:
	–   Cellular channel reuse
	–   Time sharing transmission.

1.6	The following handover techniques should be utilised:
	–   Uninterrupted handover (channel and base station)
	–   Uninterrupted file transfer.

1.7	The equipment should be designed so that frequency changes between assigned channels can be carried out in less than 100 ms.

1.8	Emissions should be vertically polarized at the source.

1.9	Switching between reception and transmission should not take more than 5 ms.

1.10	The serial communication channels (SCC) on a single radio modem should be:

–   Ethernet
	–   RS232 (NMEA).

1.11	The radio equipment should meet the following norms:
	–   Radio parameters:	   ETSI EN 300 113-1
	–   EMC:		   ETSI EN 301 489-5.

# 2	Transmitters

2.1	The frequency tolerance for coast station transmitters should not exceed 5 parts in 106, and that for ship station transmitters should not exceed 10 parts in 106.

2.2	Spurious emissions should be in accordance with the provisions of RR Appendix 3.

2.3	The carrier power for coast station transmitters should not exceed 50 W.

2.4	The carrier power for ship station transmitters should not exceed 25 W.

2.5	The cabinet radiated power should not exceed 25 µW.

# 3	Receivers

3.1	The receiver sensitivity for Bit Error Rate (BER) 10-³ should be better than –105 dBm.

3.2	The adjacent channel selectivity should be at least 70 dB.

3.3	The spurious response rejection ratio should be at least 70 dB.

3.4	The radio frequency intermodulation rejection ratio should be at least 70 dB.

3.5	The power of any conducted spurious emission at the antenna terminals should not exceed 2.0 nW.

Annex 2

Proposed Maritime VHF Frequency Usage Table – Appendix 18

| Channel 5esign-nator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ship stations | Coast stations |  | Single frequency | Two frequency |  |
| 60 | m), o) | 156.025 | 160.625 |  |  | x | x |
| 01 | m), o) | 156.050 | 160.650 |  |  | x | x |
| 61 | m), o) | 156.075 | 160.675 |  | x | x | x |
| 02 | m), o) | 156.100 | 160.700 |  | x | x | x |
| 62 | m), o) | 156.125 | 160.725 |  | x | x | x |
| 03 | m), o) | 156.150 | 160.750 |  | x | x | x |
| 63 | m), o) | 156.175 | 160.775 |  | x | x | x |
| 04 | m), o) | 156.200 | 160.800 |  | x | x | x |
| 64 | m), o) | 156.225 | 160.825 |  | x | x | x |
| 05 | m), o) | 156.250 | 160.850 |  | x | x | x |
| 65 | m), o) | 156.275 | 160.875 |  | x | x | x |
| 06 | f) | 156.300 |  | x |  |  |  |
| 66 | m), o) | 156.325 | 160.925 |  |  | x | x |
| 07 | m), o) | 156.350 | 160.950 |  |  | x | x |
| 67 | h) | 156.375 | 156.375 | x | x |  |  |
| 08 |  | 156.400 |  | x |  |  |  |
| 68 |  | 156.425 | 156.425 |  | x |  |  |
| 09 | i) | 156.450 | 156.450 | x | x |  |  |
| 69 |  | 156.475 | 156.475 | x | x |  |  |
| 10 | h) | 156.500 | 156.500 | x | x |  |  |
| 70 | j) | 156.525 | 156.525 | Digital selective calling for distress, safety and calling |  |  |  |
| 11 |  | 156.550 | 156.550 |  | x |  |  |
| 71 |  | 156.575 | 156.575 |  | x |  |  |
| 12 |  | 156.600 | 156.600 |  | x |  |  |
| 72 | i) | 156.625 |  | x |  |  |  |
| 13 | k) | 156.650 | 156.650 | x | x |  |  |
| 73 | h), i) | 156.675 | 156.675 | x | x |  |  |
| 14 |  | 156.700 | 156.700 |  | x |  |  |
| 74 |  | 156.725 | 156.725 |  | x |  |  |
| 15 | g) | 156.750 | 156.750 | x | x |  |  |
| 75 | n) | 156.775 |  |  | x |  |  |

| Channel 5esign-nator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ship stations | Coast stations |  | Single frequency | Two frequency |  |
| 16 |  | 156.800 | 156.800 | DISTRESS, SAFETY AND CALLING |  |  |  |
| 76 | n) | 156.825 |  |  | x |  |  |
| 17 | g) | 156.850 | 156.850 | x | x |  |  |
| 77 |  | 156.875 |  | x |  |  |  |
| 18 | m) | 156.900 | 161.500 |  | x | x | x |
| 78 | m) | 156.925 | 161.525 |  |  | x | x |
| 19 | m) | 156.950 | 161.550 |  |  | x | x |
| 79 | m) | 156.975 | 161.575 |  |  | x | x |
| 20 | m) | 157.000 | 161.600 |  |  | x | x |
| 80 | m) | 157.025 | 161.625 |  |  | x | x |
| 21 | m) | 157.050 | 161.650 |  |  | x | x |
| 81 | m) | 157.075 | 161.675 |  |  | x | x |
| 22 | m) | 157.100 | 161.700 |  | x | x | x |
| 82 | m), o) | 157.125 | 161.725 |  | x | x | x |
| 23 | m), o) | 157.150 | 161.750 |  | x | x | x |
| 83 | m), o) | 157.175 | 161.775 |  | x | x | x |
| 24 | m), o) | 157.200 | 161.800 |  | x | x | x |
| 84 | m), o) | 157.225 | 161.825 |  | x | x | x |
| 25 | m), o) | 157.250 | 161.850 |  | x | x | x |
| 85 | m), o) | 157.275 | 161.875 |  | x | x | x |
| 26 | m), o) | 157.300 | 161.900 |  | x | x | x |
| 86 | m), o) | 157.325 | 161.925 |  | x | x | x |
| 27 |  | 157.350 | 161.950 |  |  | x | x |
| 87 |  | 157.375 |  |  | x |  |  |
| 28 |  | 157.400 | 162.000 |  |  | x | x |
| 88 |  | 157.425 |  |  | x |  |  |
| AIS 1 | l) | 161.975 | 161.975 |  |  |  |  |
| AIS 2 | l) | 162.025 | 162.025 |  |  |  |  |

|  | Public correspondence (Voice) |
| --- | --- |
|  | VHF Data |
|  | Distress and safety |
|  | Ship to ship communication |
|  | VTS, harbor and pilots |
|  | AIS |

____________