# Next Generation Combined AIS and VHF Data Exchange

## Summary

This document describes a combined multifunction system that provides a fully-functional ship-ship, ship-shore, and ship-satellite-shore AIS and also performs VHF data exchange of safety and security related information.

Connect with “Three Essential Elements”

## Background, including the IALA e-Navigation MRCP

Via its e-Navigation Committee, IALA has created a Marine Radio Communications Plan for e-Navigation. This plan includes various communications technologies and systems. It includes three communications services which IALA believes are essential for the implementation of IMO’s e-Navigation:

- Next generation AIS

- VHF Data exchange

- 500 kHz Digital broadcast

This document describes in more detail the first two of these, and indicates how they may be implemented in a combined operating system.

## Scope of the Next Generation AIS and VHF Data Exchange

## Next Generation AIS

Four simplex channels (single frequencies) and two duplex VHF channels (frequency pairs) are required for the next generation of AIS. The purpose of each is described fully in the table on the following page, but can be summarised as follows.

- AIS frequencies AIS-1 and AIS-2 (both are simplex channels) will be used exclusively for safety of navigation, primarily position reporting and identification, ship to ship and ship to shore

- AIS frequencies AIS-3 and AIS-4 (the simplex channels 75 and 76) will be used for satellite detection of AIS using AIS Message 27 for identification and vessel tracking by shore authorities

  - These two frequencies are adjacent to Channel 16 and were originally guard bands for Channel 16

  - Analysis has shown that the relatively infrequent and shorter Messages 27 will not affect the service in Channel 16 (Reference Report ITU-R M.2169 Improved Satellite Detection of AIS)

    - This report considers the probability of satellite detection and the adjacent channel interference effects on Channel 16

- AIS channels  AIS-5 and AIS-6 (preferably the duplex channels 27 and 28) will be used for terrestrial marine safety and security related information including meteorological data and S/N Circ 289 international AIS binary messages

The next generation of AIS will address the present problem of increasingly congested conditions on AIS-1 and AIS-2, enable much better satellite detection of AIS even in crowded sea areas, and ensure that marine safety information can be disseminated effectively.

Channel 70 will also be retained for channel management of the AIS VDL, as now.

Next Generation AIS

| AIS channels (simplex or duplex pairs) | AIS-1 & AIS-2 (simplex) Safety of Navigation | AIS-3 & AIS-4 (simplex) Vessel tracking and future GMDSS | AIS-5 & AIS-6 (duplex pairs) Data Communication | Channel 70 (simplex) AIS Channel Management |
| --- | --- | --- | --- | --- |
| Purpose | - Safety of navigation - Maritime and inland distress and safety communications | - Space detection of AIS - Future GMDSS | - Marine safety information - General purpose information communication | - Management of VHF VDL in harmony with current ITU-R M.822-1 |
| Frequency allocation | - Allocated / needs to be dedicated | - New / dedicated channels required | - New / shared channels required | - Allocated and dedicated |
| AIS Message types | - Vessel identification - Vessel dynamic data - Vessel static data - Aids to Navigation | - space detection of AIS - Other messages for support of future GMDSS | - Regional AIS binary messages - S/N Circ 289 international AIS binary messages | - TDMA system management messages e.g Message 20, Message 22, - Other messages for support of coordinating channel sharing |
| Typical applications | - Ship to ship collision avoidance - VTS tools - Tracking of ships - Locating in GMDSS | - Detection of vessels by coastal states beyond range of coastal AIS base stations - Future distress alerting | - Area warnings and advice - Meteorological and hydrological data - Traffic management - Channel management of AIS channels and future VHF digital data channels - Ship-shore data exchange | - Channel switching - FATDMA allocation, - Assignment |
| Proposed channels in Appendix 18 | - As now | - Channels 75 and 76 | - World-wide dedicated channels 27 and 28 are preferred | - As now but requires Appendix 18, Footnote j), and ITU-R M.822 modification |

### VHF Data Communications

VHF data communications will provide robust high-speed data exchange between ships and between ship and shore. The AIS system is not capable of handling, nor is intended for, this high-speed data exchange.

Six contiguous 25 kHz VHF channels will be required in accordance with Recommendation ITU-R M.1842-1 (preferably channels 24, 84, 25, 85, 26, and 86). These may be used as discrete data communications channels, or a number may be combined into a single wide-bandwidth channel.

A typical scheme would be to allocate four channels for data exchange in areas such as ports and crowded waterways, with the other two channels allocated to operation in coastline between these areas.

Where a number of the 25 kHz channels are combined, a typical scheme might have a 100 kHz bandwidth, allowing a much higher data throughput than a single 25 kHz channel.

## AIS contributions toward future Search and Rescue

The next generation AIS will contribute to distress alerting and search and rescue as follows:

- Existing AIS devices (transmit-only)

  - AIS SART

  - AIS MOB

  - EPIRB-AIS

- Future AIS mobile stations using message exchange (two-way)

  - Proposed AIS Message-28 distress alerting message

  - Proposed AIS Message-29 distress alert acknowledgement

  - Proposed Navigation status 13, distress alert, in another AIS message

## Functionality

### AIS ship-ship and ship-shore

### AIS ship-satellite-shore

### AIS future SAR service

### VHF high speed data exchange

### Interface to other equipment (IEC-61162 series)

### BIIT, indicators, and alarms

## Capability

### xx

### xx

### xx

## Architecture

### (??) Block diagram

### Operating system

### Hierarchy of functionality

## Technical Characteristics

### AIS

Refer to existing standards etc.

### VHF data exchange

### xx

## User Interface

### xx

### xx

## xx

### xx

### xx

## xx

### xx

### xx

### xx