| From: e-NAV Committee | e-NAV11/10/4 Formerly e-NAV10/output/18 |
| --- | --- |
| To: The IALA Council e-NAV Committee members IMO Correspondence Group on e-Navigation | 30 September 2011 |
| Information (if approved): IALA National Members e-NAV Committee members IMO Correspondence Group on e-Navigation |  |

Information Paper

Three Essential Elements of e-Navigation Communications

# introduction

This document describes three data communications elements, which are vital for e-Navigation.  All three are non-commercial services.  The viability of these services for these applications depends strongly on regulatory action of international organisations (IMO, ITU, IEC), especially with respect to securing the necessary radio spectrum.

It has been prepared with a view to informing those who may not read the IALA MRCP of the essential elements of e-Navigation communications and could be of use for distribution beyond the e-NAV Committee.

# Background, including the IALA e-Navigation MRCP

Via its e-Navigation Committee, IALA has created a Maritime Radio Communications Plan for e-Navigation.  This plan includes various communications technologies and systems.

This document describes three vital communications services which IALA believes are essential for the implementation of IMO’s e-Navigation.

- Next generation AIS;

- VHF Data exchange;

- 500 kHz Digital broadcast.

Each of the three is a non-commercial system, meaning that no charges are involved in the sending or receiving of messages.  All messages are data messages.

# Next Generation AIS

Four simplex channels (single frequencies) and two duplex VHF channels (frequency pairs) are required for the next generation of AIS.  The purpose of each is described fully in the table on the following page, but can be summarised as follows.

- AIS frequencies AIS-1 and AIS-2 (both are simplex channels) will be used exclusively for safety of navigation, primarily position reporting and identification, ship to ship and ship to shore;

- AIS frequencies AIS-3 and AIS-4 (the simplex channels 75 and 76) will be used for satellite detection of AIS using AIS Message 27 for identification and vessel tracking by shore authorities:

  - These two frequencies are adjacent to Channel 16 and were originally guard bands for Channel 16.

  - Analysis has shown that the relatively infrequent and shorter Message 27 will not affect the service in Channel 16 (Reference Report ITU-R M.2169 Improved Satellite Detection of AIS).  This report considers the probability of satellite detection and the adjacent channel interference effects on Channel 16.

- AIS channels AIS-5 and AIS-6 (preferably the duplex channels 27 and 28) will be used for terrestrial marine safety and security related information including meteorological data and S/N Circ. 289 international AIS binary messages.

The next generation of AIS will address the present problem of increasingly congested conditions on AIS-1 and AIS-2, enable much better satellite detection of AIS even in crowded sea areas, and ensure that marine safety information can be disseminated effectively.

Channel 70 will also be retained for channel management of the AIS VDL, as now.

NEXT GENERATION AIS

| AIS channels (simplex or duplex pairs) | AIS-1 & AIS-2 (simplex) Safety of Navigation | AIS-3 & AIS-4 (simplex) Vessel tracking and future SAR | AIS-5 & AIS-6 (duplex pairs) Data Communication | Channel 70 (simplex) AIS Channel Management |
| --- | --- | --- | --- | --- |
| Purpose | - Safety of navigation - Maritime and inland distress and safety communications | - Space detection of AIS - Future SAR | - Marine safety information - General purpose information communication | - Management of VHF VDL in harmony with current ITU-R M.822-1 |
| Frequency allocation | - Allocated / needs to be dedicated | - New / dedicated channels required | - New / shared channels required | - Allocated and dedicated |
| AIS Message types | - Vessel identification - Vessel dynamic data - Vessel static data - Aids to Navigation | - Space detection of AIS - Other messages for support of future SAR | - Regional AIS binary messages - S/N Circ 289 international AIS binary messages | - TDMA system management messages e.g. Message 20, Message 22, - Other messages for support of coordinating channel sharing |
| Typical applications | - Ship to ship collision avoidance - VTS tools - Tracking of ships - Locating in SAR | - Detection of vessels by coastal states beyond range of coastal AIS base stations - Future distress alerting | - Area warnings and advice - Meteorological and hydrological data - Traffic management - Channel management of AIS channels and future VHF digital data channels - Ship-shore data exchange | - Channel switching - FATDMA allocation - Assignment |
| Proposed channels in Appendix 18 | - As now | - Channels 75 and 76 | - World-wide dedicated channels 27 and 28 are preferred | - As now but requires Appendix 18, Footnote j), and ITU-R M.822 modification |

# VHF Data Communications

VHF data communications will provide robust high-speed data exchange between ships and between ship and shore.  The AIS system is not capable of handling, nor is intended for, this high-speed data exchange.

Six contiguous 25 kHz VHF channels will be required in accordance with Recommendation ITU-R M.1842-1 (preferably channels 24, 84, 25, 85, 26, and 86). These may be used as discrete data communications channels, or a number may be combined into a single wide-bandwidth channel.

A typical scheme would be to allocate four channels for data exchange in areas such as ports and crowded waterways, with the other two channels allocated to operation in coastline between these areas.

Where a number of the 25 kHz channels are combined, a typical scheme might have a 100 kHz bandwidth, allowing a much higher data throughput than a single 25 kHz channel.

## 500 kHz Band

This band is 495 kHz to 505 kHz.

The 500 kHz band will be used for broadcasting maritime safety and security related information from shore to ship.

This band will allow maritime safety and security related information to be received at ranges up to 400 nautical miles at data rates up to 25 kbps.

The present 424 kHz, 490 kHz, and 518 kHz Navtex service would continue in parallel with the 500 kHz band service for the foreseeable future.

# AIS contributions toward future Search and Rescue

The next generation AIS will contribute to distress alerting and search and rescue as follows:

- Existing AIS devices (transmit-only):

  - AIS SART;

  - AIS MOB;

  - EPIRB-AIS.

- Future AIS mobile stations using message exchange (two-way)

  - Proposed AIS Message-28 distress alerting message

  - Proposed AIS Message-29 distress alert acknowledgement

  - Proposed Navigation status 13, distress alert, in another AIS message

# IMPACT IF spectrum is NOT SECURED

Without securing the reservation of the additional VHF bandwidth identified in this document for maritime use:

- Existing basic AIS service VDL capacity will be exceeded in a number of areas within the next decade.

- e-Navigation services above those of the existing basic AIS service cannot be delivered between ship and shore using currently available technologies economically.

- Continued shared use of this spectrum will result in AIS and VHF Data channels being at risk of in-band interference from land mobile services.

- Increasing risk of co-channel interference from licensed land mobile users of the re-allocated VHF Marine / land mobile spectrum to the latter.

This will have a significant negative impact on safety of navigation and vessel safety.

# Action requested

- The Council is requested to consider the information provided, and to approve its distribution to National members as a matter of urgency, with a request to National members to propose and support action at the 2012 World Radio Conference (WRC) to secure the radio spectrum described above, which is essential for e-Navigation.