e-NAV11  	Input paper

Agenda item	8.3

Task Number	7

Author(s)	WG2 Correspondence group on PNT

Performance Standard for Multi-system Navigation Receiver Equipment

# Summary

The attached document (ANNEX A) sets out a proposed performance standard for ship-borne receiving equipment capable of using signals from multiple systems, including GNSS and its augmentations, as well as terrestrial alternatives. This will avoid the proliferation of standards for individual systems and combinations of systems and reduce delays in introducing new services and enable the full benefits of availability, accuracy, integrity and continuity of multiple systems to be realised.

# Background

A submission to MSC 90 was made by France to put performance standards for a multi-system receiver on the work program for NAV. If the proposal is approved then it is likely that work would begin at NAV 59 in 2013 and if completed at that session, a resolution on the new standards could be approved by MSC 93 (May 2014). If further work is needed at NAV 60, the standards could be approved at MSC 94 (Nov 2014).

If the proposal will not approved at that session, then the next opportunity to get the subject on the work program would be at MSC 91 in November. However, in that case, it might be better to propose the broader topic of resilient PNT for e-Navigation, or Integrated PNT, in order to ensure stronger support.

# Discussion

To further support this initiative it could be helpful for IALA to send an Information Paper to NAV 58, setting out the case for a modular and open Integrated PNT System, showing the architecture envisaged and the role of the multi-system receiver in it.  A revised multi-system receiver standard could be included as an annex, to initiate discussion at NAV 58.

For this purpose the PNT correspondence group revised the submitted performance standard for a multi-system receiver as provided in ANNEX A.

# Action requested of the Committee

The Committee is requested to:

- Consider the revised performance standard on a multi-system receiver Annex A.

- Further develop the performance standards at eNAV11.

- Forward the revised performance standard on a multi-system receiver as an annex together with an information paper on a modular and open concept of Integrated PNT System to NAV58.

SUB-COMMITTEE ON SAFETY OF	NAV 58/xx/xx

NAVIGATION	25 April 2012

58th Session	Original: ENGLISH

Agenda item

Performance Standard for Multi-system Navigation Receiver Equipment

Submitted by

| SUMMARY |  |  |
| --- | --- | --- |
| Executive summary: |  | This document sets out a proposed performance standard for ship-borne receiving equipment capable of using signals from multiple systems, including GNSS and its augmentations, as well as terrestrial alternatives. This will avoid the proliferation of standards for individual systems and combinations of systems and reduce delays in introducing new services and enable the full benefits of availability, accuracy, integrity and continuity of multiple systems to be realised. |
| Strategic direction: |  | 5.2 |
| High-level action: |  | 5.2.4 |
| Planned output: |  | 5.2.4.3 |
| Action to be taken: |  | Paragraph 8 |
| Related documents: |  | A.953(23), A.915(22), MSC 86/26 |

Introduction

1.	A generic performance standard for a combined Electronic Position Fixing System (EPFS) receiver is proposed. At present there are separate standards for GPS, GLONASS, Galileo, and differential GPS and GLONASS receivers and for terrestrial systems. Some of these standards are no longer relevant due to system modernisation and will become less relevant as future changes emerge.

2.	As modernized and new GNSS systems (such as modernised GPS, GLONASS, Galileo, and Compass); terrestrial navigation system (such as eLoran and R-Mode), and enhanced augmentation systems (such as next generation IALA Beacon DGNSS and SBAS) approach operational status there will be a need to introduce new, and generic, receiving equipment performance standards for these systems and for each combination of systems.

3.	 MSC 90 has agreed to include the task of developing a multi-system performance standard as a new work programme item for NAV, with a target completion date of 2013 (MSC 90/26).

Requirements

4.	Performance standards should define the minimum performance requirements, not how they are met. It should be possible to meet those requirements with a variety of equipment and systems, ranging from a standalone, single system hardware receiver to an integrated, multi-system, software defined receiver. It should not be necessary to have individual standards for each system.

5.	Requirements should be set by the users, not determined by what is technically possible, or economically achievable.

6.	Requirements can be set out in terms of four main parameters: availability, continuity, accuracy and integrity.

Further considerations

7.	Introducing a generic performance standard should benefit ship operators, manufacturers and system providers by avoiding costly replacements, simplifying updates, providing market flexibility and making type approval and inspection much easier.

Action requested of the Sub-Committee

8.	The Sub-Committee is invited to note the above observations and to consider the draft Performance Standard annexed to this document.

***

ANNEX1 of ANNEX A

PERFORMANCE STANDARD FOR SHIP-BORNE RECEIVING EQUIPMENT USING MULTIPLE [RADIONAVIGATION] SYSTEMS

1 	INTRODUCTION

  - Global Navigation Satellite Systems such as Galileo, GLONASS, GPS and Compass are space-based positioning, velocity and time systems. Each GNSS space segment will normally be composed of up to 30 satellites, which may be deployed in several orbital planes.  The spacing of satellites in orbit will normally be arranged so that a minimum of four satellites will be in view to users world-wide, with a position dilution of precision (PDOP) <6.

  - Terrestrial Electronic Position Fixing Systems (EPFS) use ground-based transmitting stations. The signals can be processed in a similar way to those from GNSS, using times of arrival from at least three stations to establish a two-dimensional position.

  - Augmentation systems use ground-based or satellite based transmitters to provide augmentation to improve accuracy and integrity for specific service areas (such as coastal and port).

  - An integrated receiver, using a combination of signals from one or more GNSS and/or terrestrial EPFS, offers improved availability, integrity, accuracy and resistance to intentional and unintentional interference.

  - Receiving equipment capable of combining individual measurements from a number of GNSS and/or terrestrial EPFS to form a single solution can be used for navigational purposes on ships of speeds not exceeding 70 knots.  Such equipment should, in addition to the general requirements contained in resolution A.694(17), comply with the following minimum performance requirements.

  - These standards cover the basic requirements of position-fixing, determination of course over ground (COG), speed over ground (SOG) and timing either for navigation purposes or as input to other functions.

1.7 	The words “combined EPFS receiver equipment” as used in these performance standards include all the components and units necessary for the system to properly perform its intended functions.  The equipment should include the following minimum facilities;

- antennas capable of receiving GNSS and/or terrestrial EPFS signals;

- combined GNSS and/or EPFS receiver and processor;

- means of accessing the computed latitude/longitude position, COG, SOG, timing, PVT source and navigational phase requirements that can be achieved;

- data control interface;

- means to display the position, COG, SOG, timing, PVT source and navigational phase requirements that can be achieved or interfaced to a separate display;

- additional raw data interface to applications (such as Integrated Navigation Systems (INS)) for an enhanced assessment of provided PVT data; and

- Alert management together with Bridge Alert Management (BAM) and INS.

2.	SENSOR REQUIREMENTS

- Antennas should be capable of receiving GNSS and/or terrestrial EPFS signals.

- The antenna design should be suitable for fitting at a position on the ship which provides the best environment for the reception of GNSS and/or terrestrial EPFS signals.

- The combined receiver should be designed to reject interference from out-of-band sources.

### 3.	OPERATIONAL REQUIREMENTS

The combined EPFS receiver equipment should:

3.1	operate on at least one GNSS navigation signal provided in protected frequency band, carrying a range code (e.g. GPS: L1 and L5, GLONASS: L1, GALILEO: L1 and E5,…) with civil access.

3.2 	be capable of receiving and processing GNSS in one or more protected frequency bands (e.g. L1, L5, E5…) and provide position information in latitude and longitude in a common worldwide geodetic reference system with co-ordinates in degrees, minutes and thousandths of minutes.

3.3	have the facilities to process differential correction data fed to it, in accordance with the appropriate ITU-R Recommendations;

3.4	be provided with means to transform the computed position into data compatible with the datum of the navigational chart in use.  The display and/or any data output should indicate that the co-ordinate conversion is being performed and should identify the co-ordinate system in which the position is expressed;

3.5 	be capable to provide assessment of requirements against each navigation phase (navigation in ocean waters, harbour entrances, harbour approaches and coastal waters) simultaneously.

3.6 	be capable to provide COG in degrees to 2 decimal places, SOG in knots to 2 decimal places and timing information in UTC.

3.7	be provided at least one output from which position information, COG, SOG, timing, PVT source and estimated navigation phase for which performance requirements are met, can be supplied to other equipment.

3.8	be provided with at least a second output from which raw data (e.g. code and carrier phase, navigation data, augmentation data) from the applied radio navigation systems can be supplied to other equipment.

3.9	be capable of meeting the navigation in ocean water phase requirements under the various dynamic conditions that could be experienced onboard ships.

3.10	optionally, be capable of meeting the requirements of other navigational phases, under the various dynamic conditions that could be experienced onboard ships.

3.11	be capable of acquiring position to the navigation in ocean water phase, within 15 min of powering up;

3.12	be capable of re-acquiring position, within 2 min, when subjected to a power interruption or loss of signal of <60s, where the position requirement is same as the criterion in force when the power/signal was lost.

3.15	The combined EPFS receiver should provide receiver autonomous integrity monitoring (RAIM), against each navigation phase requirement, simultaneously.

3.16	The combined EPFS receiver equipment should provide as a minimum:

.1	a caution, should any of the following conditions exist for 3s for conventional craft and 2s for high speed craft:

.1.1 unable to estimate the navigation phase;

.1.2 a new position, COG or SOG has not been calculated.

.2	If after 5s (high speed craft) or 7s (conventional craft) a new position, COG or SOG have not been provided a warning shall be raised.

.3	When it is not possible to provide a new position update, the last known position and the time of the last valid fix, with explicit indication of this state so that no ambiguity can exist, should be output until normal operation is resumed.

.4	augmentation status indication of:

.1 the receipt of augmentation signals; and

.2 the validity of augmentation signals; and

.2 whether augmentations are being applied to the indicated ship’s position;

.3 provide the identification of the augmentation signals.

.5	provide position, COG, SOG, timing, PVT source, estimated navigation phase for which performance requirements are met and augmentation text display either to a local display or interfaced to a separate display.

4. 	INTERFACING AND TECHNICAL REQUIREMENTS

The combined EPFS receiver equipment should:

4.1	generate a new position, COG and SOG at least once every 1s for conventional craft and at least once every 0.5s for high-speed craft;

4.2	provide a minimum resolution of position, i.e. latitude and longitude, of 0.0001 minutes;

4.3	provide COG in degrees with 2 decimals, SOG in knots with 2 decimals and timing information in UTC.

4.4	be provide at least one output from which position information, COG, SOG, timing, PVT source and estimated navigation phase for which performance requirements are met can be supplied to other equipment. Such outputs should have a validity mark aligned with that on the position output.

4.5	The output of position, COG, SOG, timing, PVT source and estimated navigation phase for which performance requirements are met should be in accordance with the relevant international standards. For the VDR services, additional data on the multiple PVT sources applied should be provided.

4.6	be provided with at least a second output from which raw data (e.g. code and carrier phase, navigation data, augmentation data) from the applied radio navigation systems can be supplied to other equipment.

4.7	have the facilities to take an input from at least one augmentation source

4.8	be capable of operating satisfactorily in typical L-band interference conditions.

4.9	Precautions should be taken to ensure that no permanent damage can result from an accidental short circuit or grounding of the antenna or any of its input or output connections or any of the combined receiver equipment inputs or outputs for a duration of 5 min.

4.10	Provide an interface with the BAM or INS for alert management.

4.11	provide at least one normally closed contact, which should indicate failure of the combined EPFS receiving equipment;

### 5	SYSTEM AND EQUIPMENT DOCUMENTATION

5.1 Operating manuals that should contain:

- An overall function description of the combined EPFS receiving equipment;

- The redundancy concept and the availability of functions given the loss of one input;

- A description of possible failures and their effects on the system (e.g. by using part of the failure analysis); and

- Guidance for the adjustments of the navigation phase requirements

- Explain the method used for the applied indicators and the thresholds applied.

- Explanation of the priority matrix applied within the receiver when multiple systems are available.

5.2 Installation manuals that should contain:

- Details of the sources, components and the interconnections between them;

- Details of the interfaces and connections for data import and export and the interconnection diagrams;

- Power supply arrangement

- Recommendations on the physical layout of equipment, including antenna mounting requirements and necessary space for maintenance;

5.3 The onboard familiarization material should explain all configuration, functions, limitations, controls, displays, alerts and indications of the combined EPFS receiving equipment.

5.4 	A failure analysis, at the combined EPFS receiving equipment functional level, should be performed and documented for the receiver.  The failure analysis should verify that the receiver is designed on a “fail-to-safe” principle.  The failure analysis should consider the impact of all failure modes “e.g. those caused by electrical, component, signal jamming…etc).

70 knots taken from Galileo performance standard (Italic footnotes for PS development only)

Refer to Publication IEC 60945

The requirements for different navigation phases are set out in IMO Resolution A.1046(27) & A.915(22) [Exact values TBD].

PVT source refers to the Multi-system receiver and can form a unique identifier.

Ref ITU (TBD)

International Terrestrial Reference Frame, or some implementation of it, such as World Geodetic System  (WGS) 84 in the case of GPS, or PZ-90 in the case of GLONASS.

ITU-R M823-4 or other relevant standards

Refer to IMO A.1046(27) & A.915 [exact values TBD].

2 decimal places based on (TBC)

Estimation of time required for 12m cold start for GPS receivers + 3min grace period

Estimated – no reference

3s for slow craft and 2s for fast craft are estimated.  Along with the 3s and 4s in 3.16.2

Time durations include that expressed in 3.16.1 for each craft type.

Figure selected to provide sub metre positioning information

Refer to IEC 61162

Raw data should be provided in an open, interoperable format (refer to IEC 61162-450)

ITU-R M823-4

Figure taken from Galileo performance standards

Refer to IEC 61162 and IEC 61924-2