ANNEX 3.

The Human Element Analyzing Process (HEAP) in e-navigation.

1	MSC/Circ.878 MEPC/Circ.346, Nov 1998 define the HEAP a practical tool designed to address the human element, to be used for consideration of maritime safety and environmental issues at IMO. The steps outlined in the flowchart list a series of questions that should be considered to appropriately address the human element in the regulatory development process.

2	The circular states also that the guidelines are intended to facilitate trial applications of the HEAP and they should remain interim as long as it is necessary to gain experience. Such trial applications were expected to lead to a greater understanding of HEAP by all parties, and identify improvements to the process. Member governments were invited to carry out trial applications of HEAP and to submit the results to IMO.

3	According to MSC/Circ.1022 (Guidance on the use of Human Element Analyzing Process and Formal Safety Assessment in the IMO rule making process), the HEAP "is a method developed by IMO for the use of IMO and should be seen as a practical and non-scientific checklist to assist regulators in ensuring that human element aspects related to the ship and its equipment, training, the master and the crew, management ashore and on-board and work environment conditions have been taken into consideration when introducing or amending IMO instruments.

4	The e-navigation Working Group established at COMSAR15 adopted a template for identifying practical e-navigation solutions based on operational, technical, regulatory and training aspects of the gap analysis. It was agreed that the Correspondence Group on e-navigation would endeavor to develop practical e-navigation solutions for gaps taking into account the human element.

5	Yet HEAP has not been used during e-navigation Working Group sessions. To explore HEAP applicability to the gap-analysis, a list of possible limitations follows.

6	The HEAP is designed to assist regulators in ensuring that human element aspects are taken into account across different areas only from a ship-borne perspective. Hence no consideration is given to the ship-shore integration and harmonization which are the basis of the e-navigation concept.

7	In order to “review all areas affected” a classification of human elements is provided at the technical, manning, training, management and work environment level, therefore not consistently with the four levels of the e-navigation gap analysis: technical, operational, training and regulatory.

| Technical (The vessel and/or its equipment) | Manning (Master and crew of the vessel) | Training (Ashore and aboard) | Management (Ashore and aboard) | Work Environment / conditions (aboard ship) |
| --- | --- | --- | --- | --- |
| Design Ergonomics Manufacture Construction Installation Initial and periodic testing Approval Maintenance Repairs Modifications Renewals Expected marine environment Operations | Qualifications Number of crew members Composition of crew Culture3 Working Language Medical Conditions Competence | Basic Safety Training Familiarization Drills Extended safety training Training of personnel ashore | Policy Safety culture Motivation Communication links Responsibility Authority Work planning Contingency planning Emergency response Manuals Procedures Instructions Work methods Checklists Education and Training | Hazardous materials Man-machine interface Personnel protection Physical hazards Hours of work Hours of rest Fatigue Estimated workload Actual marine environment Living conditions |

Note: Table created using the list of elements provided by MSC/Circ.878 MEPC/Circ.346 par. 4 (HEAP Human Element Checklist).

8	In the verification phase, where the HEAP user is asked to verify that the safety concerns identified in the original issue were addressed. HEAP proposes also a series of questions designed to ensure that the solution is considering aspects that contribute to unsafe acts and accidents, yet neither a classification of unsafe acts and accidents nor any measurable criteria are provided.

The criteria are to refer to derive from the Assembly resolution A.850-20 (Human Element principles adopted by the organization) according to which “consideration of human element matters should aim at decreasing the possibility of human error as far as possible”.

HEAP and e-navigation

9	HEAP applied to e-navigation may be seen as an extension of the actual HEAP.

The flow diagram below exemplifies how the “HEAP e-nav” concept might be summarized:

.1	 HEAP e-nav proposes a table of specific human elements to consider while developing guidelines and regulations related to practical e-navigation solution. New elements are introduced in the area of the ship-shore integration. In order to clarify the underlaying principles of each element of the table, a short definition will be provided as well (at the last page of the present annex).

| Table “R” |  |  |  |
| --- | --- | --- | --- |
| OPERATIONAL LEVEL interactions between humans and the operational environment | TECHNICAL LEVEL human-technology interaction | REGULATORY regulatory style | TRAINING human-element oriented training |
| OPR1 - Human attention OPR2 - Mental workload OPR3 - Decision Making OPR4 - Situation Awareness Lev 1 (Perception of external cues) OPR5 - Situation Awareness Lev 2 (Understanding of current state) OPR6 - Situation Awareness Lev 3 (Projection of current state in future) OPR7 - Cooperation (Team work) OPR8 - Language differences (communication breakdowns) OPR9 - Others (specify) | TCH1 - Software ergonomics TCH2 - Hardware ergonomics TCH9 - Others (specify) | REG1 - Prescriptive REG2 - Performance based REG3 - Deregulated REG4 - Industry self regulated REG9 - Others (specify) | TRN1 - Bridge Team Management training (BTM) TRN2 - Shore-based Team Management training (STM) TRN3 - Bridge-Shore Teams management (BSTM) TRN4 - Skill-based training TRN5 - Knowledge-based training TRN9 - Others (specify) |

.2	HEAP e-nav offers a classification of threats (table “T”) and errors (table “E”).

The lists provided in the tables have been adapted for the use in the e-navigation domain but it is not exhaustive.

The correlation between a set of practical e-navigation solutions and a set of threats and errors could allow the verification of the effectiveness of the HEAP e-nav process. How well a solution is addressing a gap from a human element perspective will depend on the impact the solution has on the successful management of threats and errors to be assessed through:

- real world test beds;

- full bridge simulations and;

- accident analysis.

This approach addresses NAV57/6/2 which states that “simulation might be used to evaluate the development and usability of e-navigation applications. A procedure has to be developed for testing how to evaluate the various concepts and applications”.

A sample of possible application of the “HEAP e-nav” tables to the gap analysis template is shown below.

|  | e-navigation SOLUTIONS |  |  |  |
| --- | --- | --- | --- | --- |
|  | OPERATIONAL Human Element (Table R) | TECHNICAL Human Element (Table R) | REGULATORY Human Element (Table R) | TRAINING Human Element (Table R) |
| IDENTIFIED GAP Threats and Errors addressed by the identified set of solutions T -- (from table “T”) E -- (from table “E”) | ....... OPR -- (from table “R”) | ....... TCH -- (from table “R”) | ....... REG -- (from table “R”) | ......... TRN -- (from table “R”) |

| Table “T” - Threats Events that originate outside the influence of the bridge team but requires active management to maintain navigation safety |  |  |  |
| --- | --- | --- | --- |
| Threat categories | Definitions | Sub-categories | Codes |
| Meteo-oceanographic threats | Situations generated by meteo-oceanographic uncontrollable factors | Wind force close to operational limit Restricted visibility Sea state close to operational limit Ice conditions Tidal streams in restricted waters Other meteo-oceanographic threats | T01 T02 T03 T04 T05 T000 |
| Other ships | Situations generated by ship-borne traffic and interaction with other ships | High density traffic Ships non-compliant with COLREGs Ships using non-standard phraseology Faulty AIS information Other ships threats | T11 T12 T13 T14 T15 T111 |
| Shore support | Situations generated by shore parties errors in operations (procedural, communication, competency) or by inadequate support services | VTS error Tug boat error Port facility issue Aids to Navigation reliability Low accuracy of nautical charts (for ENC: CATZOC “C”, “D”, “U”) Unavailability of updates for nautical charts & publications Radio communication congestion Other shore support threats | T21 T22 T23 T24 T25 T26 T27 T222 |
| Own Ship | Situations generated on board own ship but beyond the control of bridge team | Substandard bridge equipment ergonomics (hardware and software) Substandard bridge layout (not compatible with SOPs) Information overload Navigation equipment malfunction Radio Communication equipment malfunction Engineering malfunction Cargo issues Passenger issues Late change of voyage plan Commercial pressure Other ship threats | T31 T32 T33 T34 T35 T36 T37 T38 T39 T40 T333 |

| Table “E” - Errors Failures, by the bridge team, of planned actions to achieve their desired outcome |  |  |  |
| --- | --- | --- | --- |
| Error categories | Definition | Sub-categories | Codes |
| Procedural error | Willful or non-intentional deviations from SOPs. | Negligence (due to fatigue or lack of motivation) Routine violation (“it has always been done this way”) Reasonable violation (“to get the job done”) Non intentional (due to lack of familiarity with SOPs) Other procedural errors | E01 E02 E03 E04 E000 |
| Communication errors | Use of non-standard communication phrases, misinterpretation or failure to communicate pertinent information or intentions within the bridge team, or between the bridge team and external parties. | Within bridge team (including pilot) With Engine Control Room With other ships With VTS With tug boats With shore authorities Other communication errors | E11 E12 E13 E14 E15 E16 E111 |
| Competency errors | Lack of skill, knowledge or context specific behaviors which cause inadequate performance in STCW competencies. | Wrong collision avoidance decision (COLREGs) Misuse of INS (integrity monitoring of sensors, radar navigation, management of ARPA/AIS information and alarms) Misuse of ECDIS (plan and conduct of passage) Misuse of automatic steering control systems Ship-handling error Cargo handling error Passengers mismanagement Inadequate controlling of trim, stability and stress Inadequate response to own ship emergencies Inadequate response to distress signals at sea (SAR) Failure in adopting pollution prevention actions (MARPOL) Other competency errors | E21 E22 E23 E24 E25 E26 E27 E28 E29 E30 E31 E222 |

| Code | Element | Table “R” - OPR codes description |
| --- | --- | --- |
| OPR1 | Human attention | It refers to the process by which humans can purposefully direct their sensory apparatus towards specific environmental stimuli (not only visual) and direct the focus of their perception. It is commonly accepted that there are three major forms of attention (Sanders & McCormick, 1987): - selective attention: involves selective scanning the environment and directing the focus of our perception at a number of sequential cues; - focused attention: involves maintaining the focus of perception on a particular subject; - divided attention: involves “sharing” attentional resources between two or more concurrent tasks. |
| OPR2 | Mental workload | Mental workload can be seen in terms of the amount of mental resources available to a person and the amount of mental resources demanded by a task or situation (Sanders & McCormick, 1987). Mental workload is a function of a number of interacting elements of information processing. These include among others: - attentional demands: whereby the demands of having to pay attention to increasing amounts of information increase overall workload; - multiple competing tasks: whereby increasing the number of concurrent tasks that are required to be performed increase overall workload; - expertise: whereby tasks undertaken by a novice requires significantly more mental resources than the automatized tasks undertaken by an expert. High workload and complex tasks can be source of psychological stress, as well as substandard workloads and lack of complexity in a task. Consequently, all can contribute to decrease in performance (Wickens, 1998). If level of arousal is low and coupled with a complex task, the resulting performance will be poor. Similarly low arousal coupled with a simple task, such as a vigilance task, can be problematic, as evidenced by much research that has found low-arousal environments to be as prone to human error as high workload circumstances (Wickens,1998). |
| OPR3 | Decision Making | Decision Making is a mental process which involves the selection of an action from a number of alternatives, with a given set of available information and knowledge, a timeframe in which consideration of alternatives is possible, and where the choice is associated with some uncertainty (Wickens et al. 1998). |
| OPR4 | Situational Awareness * Level 1 (Perception) | It refers to the perception of the elements in the environment. The first step in achieving SA involves perceiving the status, attributes, and dynamics of relevant elements in the environment (Endsley, 1995). |
| OPR5 | Situation Awareness * Level 2 (Comprehension) | Comprehension of the current situation. Comprehension of the situation is based on a synthesis of disjointed Level 1 elements. Level 2 situation awareness goes beyond simply being aware of the elements that are present to include an understanding of the significance of those elements in light of the operator's goals (Endsley, 1995). |
| OPR6 | Situation Awareness * Level 3 (Projection of current state into future) | Projection of future status. It is the ability to project the future actions of the elements in the environment, at least in the near term, which forms the third and highest level of situation awareness. This is achieved through knowledge of the status and dynamics of the elements and a comprehension of the situation. both Level 1 and Level 2 SA (Endsley, 1995). |
| OPR7 | Team work | It refers to ship borne and shore-based teams internal cooperation along with the ship-shore cooperation dynamics. Examples of good behaviors for an effective team work can be categorized in 4 main categories: - team building; - considering others; - supporting others; - conflict solving. |
| OPR8 | Language differences | Language differences can potentially trigger both communication breakdowns within operators in a team or between different teams while managing routine or contingency response operations. |

| Code | Element | Table “R” - TCH codes descriptions |
| --- | --- | --- |
| TCH1 | Software ergonomics | Traditionally usability is considered in terms of attributes of the user interface. However, ISO/IEC CD 25010.3 has given broader view of usability as sub characteristic of “Quality in use”. “Quality in use” is defined in terms of: - usability : according to ISO 9241-11 (1998) a product is usable to the extents to which can be used by specified users to achieve specified goals with effectiveness (task completion by users), efficiency (Task in time) and satisfaction (responded by user in term of experience) in a specified context of use (users, tasks, equipment & environments). - flexibility : product adaptability to different contexts of use, subdivided in context conformity, context extendibility and accessibility. - safety : the degree of expected impact of harm to people, business, data, software, property or the environment in the intended contexts of use. . From a user perspective, quality in use can be assessed measuring: - effectiveness by rating task performances of users - efficiency by rating efforts and time spent in task completion by users - user satisfaction by enquiring users on pragmatic and hedonic satisfaction - flexibility by assessing the degree of individualization - safety by assessing risk to users |
| TCH2 | Hardware ergonomics | It refers to anatomical and physiological characteristics of operators interacting with a workstation in their environment (bridges and operating rooms). Among the physiological characteristics the following should be taken into account when designing bridge or operating rooms layouts: - human vision: It refers to human visual capacity from the center of the visual field. Color depth perception and visual attention varies with the angle from the the center of the visual field; - hearing: It refers to human capability to detect auditory stimuli from workstations and external environment. Audibility, background noise and intelligibility of sounds should be taken into account as well when considering human hearing capabilities. - tactile senses: it refers to the ability to feel the differences between knobs, dials, controls and buttons. These differences play an important role when visual senses are limited or in high stress/urgency conditions (Grech 2005). Notwithstanding the human capabilities and limitations of humans, the bridge and operating room layouts should reflect the operational procedures of teams. |

| Code | Element | Table “R” - REG codes descriptions |
| --- | --- | --- |
| REG1 | Prescriptive | It specifically refers to carriage requirements, international conventions or any other obligation to comply. |
| REG2 | Performance based | It refers to minimum performance standards and a set of functional requirements for equipment. |
| REG3 | Deregulated | The deregulated approach refers to the removal or simplification of relevant organizations rules that constraint the operation of market forces. |
| REG4 | Industry regulated | The industry regulated approach is based on the principle that spontaneous association of enterprises control their collective action, and propose itself as an alternative to institutional regulations. |

| Code | Element | Table “R” - TRN codes descriptions |
| --- | --- | --- |
| TRN1 | Bridge Team Management training (BRM) | BRM training is concerned not so much with the technical knowledge and skills required to navigate a ship but rather with the cognitive and interpersonal skills needed to manage the navigation within a bridge team. With reference to IMO STCW code table A-II/2, Annex 2 (“Controlling the operation of the ship and care for persons onboard at the operational and management level”) the competence should be expressed in terms of: Knowledge and ability to apply effective resource management: - allocation, assignment and prioritization of resources; - effective communication onboard and ashore; - decisions reflect consideration of team experiences; - assertiveness and leadership, including motivation; - obtaining and maintaining situational awareness. Knowledge and ability to apply decision-making techniques: - situation and risk assessment; - identify and consider generated options; - selecting course of action; - evaluation of outcome effectiveness. |
| TRN2 | VTS Resource Management training (VRM) | VTS Resource Management training would be intended to follow the same concepts of BRM training (see TRN01). Such kind of course would deliver to shore-based teams the same non-technical skills of bridge teams, enhancing the degree of integration between VTSs and ships. IALA has developed standards for training and certification of VTS personnel (IALA recommendation V-103, Ed.2). IALA recommends that VTS Manager should be able to initiate, lead and implement change and continuous improvements, and to demonstrate leadership skills and management experience. However, a specific VRM model course to train VTS operators and supervisors in resource management does not exist yet. |
| TRN3 | Maritime Resource Management training (MRM) | MRM training has a wider purpose than a BRM one. MRM targets are not only ships’ bridges, but also shore based personnel. The most advanced simulation facilities are already integrating these concepts into their BRM courses, yet no formal requirement has been expressed by relevant institutions. In September 2011, 89 training providers in 33 countries are authorized to deliver the Maritime Resources Management training |
| TRN4 | Skill-based training | It refers to a training aimed to enhance technical skills which will then develop in “automated” behavior with little conscious processing of information, problem identification, diagnosis or decision-making involved. It typically reflects the needs of novices approaching new workstations. |
| TRN5 | Knowledge-based training | It refers to a training aimed to enhance the understanding of system characteristics. This involves analyzing the information present at a conceptual level, in order to choose the correct action. This level of training will develop operators’ conscious control of navigation systems. |

A threat is an event that originates outside the influence of the bridge team but requires active management to maintain safety

An error is a failure, by the bridge team, of planned actions to achieve their desired outcome