e-NAV2 / 07 / 02

- Continuity requirements

1	Background

Continuity, or reliability, is the ability of a system to function within specified performance limits without interruption during a specified period (normally covering a particular manoeuvre, such as a turn).

It has been recognised that the requirements set for continuity in the relevant IMO Resolutions (A.953(23) - World Wide Radio Navigation System & A.915(22) -  Future Requirements for Maritime GNSS) are difficult, if not impossible to achieve with existing or planned systems.

The requirement is 99.97% over a period of 3 hours, this would demand a Mean Time Between Failures (MTBF) of 416 days, whereas GPS fails to meet the accuracy requirement (10 m) for significant periods about once a month, on average.

IALA considered that a more realistic continuity requirement would be 99.97% over a period of 15 minutes (MTBF = 35 days). Before proposing a change to existing requirements, feedback was sought from practising mariners to ensure that the levels set were not only achievable, but met practical requirements.

The Committee is invited to consider this paper as the basis for an input to IMO, proposing revision of the above Resolutions.

2	Feedback

At a technical meeting of the Nautical Institute in December 2006, feedback was invited on the following questions:

- What are the stages of a manoeuvre?

- Which are the critical stages?

- How long would each stage last?

- What would your requirements be for navigation systems in each stage?

The feedback can be summarised as follows:

Manoeuvres may merge into each other and the recovery stage is not necessarily distinguished from the turn. Accuracy requirements are becoming more stringent in pilotage waters and alternative navigation systems must satisfy these requirements. The 3 hour continuity time interval may be required for track keeping in some pilotage areas.

Note: It has been assumed in the following analysis that a lower level of continuity is required for track-keeping than for turning.

3	Analysis of Continuity Requirements

Annex 1 gives a full definition of Continuity. Annex 2 reproduces the relevant requirements from IMO Resolutions A.915(22) & A.953(23).

Navigational phases:

Typically, navigation has been divided into three phases: ocean navigation, coastal navigation and restricted waters navigation. More recently, some documents have introduced other phases of restricted waters, namely harbour approaches, port and inland waterways navigation. (ref. IALA Navguide, 5th Edition)

The performance of Global Navigation Satellite Systems (GNSS) is becoming more important in all restricted waters navigation.

The three stages of manoeuvring are turning, recovery and track keeping, the critical stages in restricted waters being turning and recovering. A lower level of continuity over a longer interval would be needed for track keeping.

The time interval for each stage would depend on factors such as vessel size, draught, rate of turn, speed, water depth, weather etc., however, experience suggests that maximum times of manoeuvring in restricted waters are:

  - Turning: 10 minutes

  - Recovering: 3 minutes

  - Track Keeping: 3 hours

4	Proposed Requirements

The proposed requirements for the navigation system when manoeuvring in restricted waters are:

Turning and Recovering – Continuity: 99.97%; CTI: 15 minutes; Accuracy: 1-10 m

Track Keeping - Continuity: 99.6%; CTI: 3 hours; Accuracy: 10 m

Annex 1	Definition of Continuity

Continuity, or reliability, is the ability of a system to function within specified performance limits without interruption during a specified period (normally short term).

The equation below refers to an availability type calculation, i.e. the average over the

continuity time interval (CTI) that the service is available or the probability that the service is available at any instant during the CTI. It is more appropriate to calculate the probability that the service is available throughout the CTI. The suggested approach is given below:

Assuming the service is functioning at the beginning of the operation, then the probability that it is still functioning at a time (t) later is:

C = exp (-t/MTBF)

This is the standard expression for reliability and excludes scheduled outages (i.e. uses MTBF) assuming that planned outages will be notified and the operation will not take place.

The probability that the service will be available after a time CTI, that is the continuity is then:

C = exp (-CTI/MTBF)

If MTBF is very much greater than CTI, this can be approximated to:

C = 1 – (CTI/MTBF)

Where:

MTBF = Mean time between failures; based on a 2 year averaging period

CTI = Continuity time interval; in the case of maritime continuity, equal to 3 hours

There is no need to include the availability at the beginning of the time period of the

operation because if there is no service, then the operation will not commence.

- Annex 2 - IMO Resolutions

IMO resolution A.953(23) (World Wide Radio Navigation System) provides for three standards on accuracy, availability and continuity from a satellite radionavigation system. They represent the situation when coverage is provided by a system comprising the satellite transmissions alone, when a single additional transmitting station augments the satellite transmissions and when two or more additional transmitting stations augment the satellite transmissions. The standards, in tabular form, are:

| Area | Horizontal Accuracy | Signal Availability | Continuity | Augmentation |
| --- | --- | --- | --- | --- |
| Ocean | 100 m | >99.8% | N/A | None |
| Coastal/harbour (low risk) | 10m | >99.5% | 99.85% over 3 hours | Single station |
| Coastal/harbour (high risk) | 10 m | >99.8% | 99.97% over 3 hours | 2 or more stations |

IMO Resolution A.915(22) (Requirements for Future GNSS) adopted in January 2002, includes the following requirements for maritime navigation:

| Phase of voyage | Horizontal Accuracy | Signal Availability | Continuity |
| --- | --- | --- | --- |
| Ocean | 10 m | 99.8% | N/A |
| Coastal | 10 m | 99.8% | N/A |
| Restricted waters | 10 m | 99.8% | 99.97% over 3 hrs |
| Port | 1 m | 99.8% | 99.97% over 3 hrs |