Introduction and Objectives

The AIS Technical Working Group (TWG) of the IALA e-NAV Committee held an intersessional meeting in Ottawa, Canada from 16 till 18 October 2006. The host for the meeting was the Canadian Coast Guard. The venue was at the Department of Fisheries and Oceans Offices.

The group was welcomed by Dr. Sam Ryan, Director of Service Delivery and AIS Project Manager of the Canadian Coast Guard.

The IMO COMSAR Sub-Committee at its 10th session developed draft performance standards for AIS-SART and forwarded these to IALA and ITU for comments and proposals (see document 01). They asked the requested action before the COMSAR 11 meeting in February 2007.

During the IALA e-NAV1 meeting in September 2006, the AIS TWG reviewed the liaison note and supported the idea; however there was not sufficient time to discuss the options to meet the requirements. It was decided to organise an inter-sessional meeting to investigate the possible solutions in discussion with representatives from the industry.

In September 2006, ITU WP8B submitted a liaison to IMO on this subject. This liaison was also addressed to IALA for information. The liaison from ITU was considered as input to the discussion (see document 02).

The objective of the meeting was to develop requirements and technical solutions to meet the required IMO performance standards on AIS-SART. The meeting was organised as a brainstorm and discussion session to investigate the feasibility of AIS-SART stations and to compare these with the existing Radar-SART.

Important criteria considered were:

- Power consumption related to the required operating time;

- Compatibility with existing AIS stations (backwards compatible);

- Price of an AIS-SART station compared with Radar-SART.

The last point is strongly related to the design of the AIS-SART station and the power consumption in particular. It plays a role in all discussions dealing with the system configuration (receivers or no receivers), the capacity of batteries, access to the VHF data link (VDL), transmitting power, etc.

Discussion

Terminology:

‘AIS-SART’ can be used to indicate the application of SART based on AIS technology as used in the Performance Standard from IMO. ITU and IALA refer to all AIS units as AIS Station. Therefore ITU, IALA and IEC will refer to an ‘AIS-SART station’ to indicate an AIS unit used as AIS search and rescue transmitter (SART). This in line with ITU-R M.1371 terminology.

During the discussion, it became clear that AIS-SART is not a ‘navigation’ device but a ‘homing’ device for search and rescue operations, comparable with Radar-SART. An AIS-SART station could meet the above criteria with a minimum amendment to the ITU-R M. 1371 latest edition.

- System configuration

AIS stations normally have two receiver functions, a processor, a GNSS receiver and a transmitter to be able to receive, process, and transmit AIS messages and to be synchronised with other AIS stations. The receiving function is necessary to receive slot reservations made by other AIS stations and avoid conflicts in the use of time slots. An AIS-SART station does not necessarily need to have an AIS receiving function from an operational requirement point of view. A disadvantage of AIS receiver(s) is the power consumption in relation to the requested time of operation (96 hours).

The following configurations were discussed

Dedicated AIS receiver(s)

If AIS receivers are included, the system will be polite with respect to the use of the VDL and time slots. Receiving of transmitted messages will be guaranteed within the existing AIS specifications. Standard access to the VDL is possible. The transmission of messages for test purposes (most likely done in a port environment) will not conflict with transmissions of other users. One of the benefits can be, that certain transmitted messages from the AIS-SART will be confirmed to the persons in distress by an acknowledgement leading to an indication (e.g. LED) on the AIS-SART station that the message has been received by another AIS station (feeling to be heard). Disadvantage is the power consumption and the cost of receivers and processing facilities.

Elementary AIS receive function

The function is to detect any activity on the VDL to avoid slot conflicts but it cannot be used to decode AIS messages from other AIS stations. This type of receivers will have less power consumption and are relatively inexpensive. To further reduce energy, the receiver can only be switched on just before a transmission is scheduled. The option to reduce the reporting rate of transmissions, if no activity is detected on the VDL, was discussed but it was concluded that transmission of AIS messages is not the most critical part of the power consumption. Transmissions might lead to slot conflicts with other AIS users.

No AIS receiver

No AIS receiver makes the design of the AIS-SART station with the required reporting rate and time of operation more realistic and more cost effective. If no receiver is incorporated, the transmitted messages may have conflicts in the use of time slots with other AIS stations leading to garbled time slots.

The consensus of the group was that the third configuration, no AIS receiver, offers the best solution. It was also considered to use world-wide agreed upon time slots for AIS-SART. These slots could be randomly [accessed] used by AIS-SART stations. These slots would only be protected during SAR incidents. At other times these slots would be available for AIS use. Therefore there would be no adverse effect on the VDL by introducing AIS-SART. If this proposal is accepted, Annex F of IALA A-124 must be revised as appropriate.

GNSS receiver

A GNSS receiver is required to synchronise time slots and for position determination of the AIS-SART station. From the position data, the speed and course over ground (SOG and COG) can be calculated. The minimum required position update of 5 minutes, as stated in the draft IMO performance standards, with intermediate position reports every minute is attainable.

- Messages

To be backwards compatible with existing AIS stations and to be sure that those stations can receive, process and display AIS-SART messages, only existing AIS messages should be used. Any position report messages may be used. Message 3 is a candidate but at the moment it is reserved for Class A stations only. If Message 3 is chosen, a revision of the ITU-R M.1371 is required (can be done via the IALA Technical Clarifications of ITU-R M.1371 for later revision)

In addition to a position report, a one slot Safety related broadcast message (Message 14) may be used. This message can broadcast a predefined standardised text (to be determined by IMO e.g. ’mayday’) with a maximum of 16 characters (see ITU-R M.1371).

Both described messages are displayed on the MKD of every Class A station and are available for external connected display facilities like Radar and ECDIS on board of ships equipped with Class A stations. The consensus of the group was that it must be displayed on the Radar.

- Identification

A unique identifier for every AIS-SART station is needed. Normally every AIS station has a unique MMSI number, administrated and registered by the Administration of the country where the AIS station is located or registered. It consists of a MID (giving the country identity) and a unique unit number. See ITU-R M.585.

For the identification of AIS-SART stations, two methods are discussed.

- Use of MMSI numbers. In September 2006, ITU-R WP8B accepted a new revision of ITU-R M.585, including the assignment of devices associated with a parent ship (see text in the box below). This needs a registration by the Administration of every AIS-SART. This registration must be changed once the AIS-SART station is moved from one ship to another.

- The MMSI number field in AIS messages can accommodate almost twice the number of possible MMSI numbers. It was discussed that AIS-SART stations can get a unique serial number from the manufacturer, fitting in the range of the MMSI number field of AIS messages but outside the range of real MMSI numbers as defined by ITU. The advantage is that no registration is required and once an AIS-SART station will move from one ship to another, the identification can be unchanged. However, it is not clear during the meeting what the effect on different types of AIS stations and applications connected to the AIS will be, if MMSI numbers are used outside the range of ITU defined numbers. This needs further consideration if applicable.

There was no final conclusion on the identification during the meeting.

Annex 5

Assignment of identification to craft associated with a parent ship

1Devices used on craft associated with a parent ship, need unique identification. These devices which participate in the maritime mobile service should be assigned a nine digit unique number in the format:

9182M3I4D5X6X7X8X9

where the digits 3, 4 and 5 represent the Maritime Identification digit (MID) and X is any figure from 0 to 9. The MID represents only the territory or geographical area of the administration assigning the call identity for the craft associated with a parent ship.

2This numbering format is only valid for devices on board crafts associated with a parent ship. A craft may carry multiple devices for which an MMSI is required. These devices may be located in lifeboats, life-rafts, MOB-boats or other craft belonging to a parent ship.

3A unique MMSI should be assigned for each device onboard a craft associated with a parent ship and will have to be separately registered and linked to the MMSI of the parent ship.

4The format scheme shown above will accommodate 10 000 devices of this type per MID. If the administration concerned has more than 10 000 they may use an additional country code (MID) if it is already assigned by the ITU giving a further 10 000 identities.

5The assigned MMSI to these devices should also be available from the ITU MARS database, Ref. No. 20.16.

- Channel Management

Channel Management was discussed. The conclusion was that AIS-SART stations should not be subject to Channel Management. AIS-SART stations will only operate (transmit) on the international designated AIS channels (AIS 1 and AIS 2) which are applicable on all international waters. If in any coastal area Channel Management is used, it should only be done under control of AIS Base stations and it is assumed that AIS Base stations can receive at least AIS 1 and AIS 2.

- Procedure

The following procedure is foreseen to embed the specifications of AIS-SART stations in international agreed standards. After approval of the performance standards in IMO (COMSAR – MSC), an IEC standard, describing AIS-SART stations in detail (including environmental requirements) with test procedures is needed for certification of AIS-SART stations. This should be drafted in liaison with the AIS TWG of the IALA e-NAV Committee to ensure compatibility with the Clarifications for ITU-R M.1371. The AIS TWG, in co-operation with ITU-R WP8B, is responsible to maintain and complete the ITU-R M.1371 for new developments. This can be done in the ‘IALA Technical Clarifications to ITU-R M.1371’ or in a revision of ITU-R M.1371.

# Other business

Northern Lighthouse Board, Scotland, will conduct sea trials with AIS-SART stations, based on the preliminary performance standards for AIS-SART on 9th and 10th of November 2006 at Oban Bay, Scotland. Manufacturers are invited to take part.

# The Final Plenary

This report was considered, amended and accepted as representing the outcome of the discussion on AIS-SART stations.

The Chair of the AIS TWG thanked all of the participants for their attendance and contributions to the discussion.

Wim van der Heijden

Chair, AIS Technical Working Group of the e-NAV Committee

Recommendations from the Ottawa meeting on AIS-SART

The following recommendation is decided from the meeting:

- The Chair of AIS TWG is asked to put the output documents as per liaison via the IALA secretariat to Chair and Vice-Chair of the e-NAV Committee for forwarding to e-NAV Committee members for silent approval within a two week period. After approval by the e-NAV Committee the secretariat is asked to forward the revised liaison statement to IMO COMSAR (e-NAV1/output/02rev1) to Council and then to IMO COMSAR before the February 2007 meeting (COMSAR11) and to ITU for information.

Annex A: Participation

| Name | Organization | Country |
| --- | --- | --- |
| - BERTORELLI, John | CCG | Canada |
| - BOBER, Stefan | Federal waterways | Germany |
| - BROWNING, Peggy | L3 Communications | USA |
| - CARD, Michael | Zeni Lite | Japan |
| - CARSON-JACKSON, Jillian convenor IEC TC80 WG14 | AMSA | Australia |
| - DOLAN, Pete | Automatic Power | USA |
| - FURUYAMA, Kenji | Japan Radio Company | Japan |
| - HOLM, Otto | JOTRON | Norway |
| - JOHNSON, Mark | Shine Micro | USA |
| - KELLOWAY, Bruce vice-chair AIS TWG | CCG | Canada |
| - LINDLEY, Hakan | SMA | Sweden |
| - MCKECHNIE, David | Pol-E-Mar Inc (Automatic Power) | Canada |
| - NICHOLS, Bob | Automatic Power | USA |
| - NYBERG, Magnus | True Heading | Sweden |
| - PIETRAZEWSKI, David | USCG R&D | USA |
| - STEWART, Alan chair IEC TC80 WG14, rapporteur ITU-R WP8B | NLB | UK |
| - VAN DER HEIJDEN, Wim chair AIS TWG | Ship Monitoring Consultancy | Netherlands |
| - YAUCHI, Takamasa | Oki Consulting Solutions | Japan |

# Regrets

| Name | Organization | Country |
| --- | --- | --- |
| - BAZAROV, Yuri | Transas | Russia |
| - INGALLS, Earle | Oerlikon | Canada |
| - LOVE, Derek | Applied Tech | UK |
| - PAGE, Kevin | ICS Electronic | UK |
| - PETTERSSON, Mikael | Saab Transponder | Sweden |
| - WILLIAMS, Alwyn | THLS | UK |

Annex B: List of documents

| 01 | Liaison statement from IMO to IALA (e-NAV1/10/1 and e-NAV1/10/2) |
| --- | --- |
| 02 | Liaison from ITU to IMO and IALA (e-NAV1/10/3) |
| 03 | Liaison from IALA to IMO (e-NAV1/output/02) |
| 04 | Report of e-NAV1 Committee meeting (e-NAV1/output/01) |
| 05 | Draft revision ITU-R M.585-3 (WP8B-TEMP-0152-E) |
| 06 | Report of the e-NAV AIS TWG on AIS-SART (Ottawa-Output01) |
| 07 | Proposed revision on the ‘Draft performance standards for survival craft AIS search and rescue transmitter (AIS-SART)’ (Ottawa-Output02) |
| 08 | Liaison to IMO as revised from the e-NAV1 meeting (Ottawa-Output03) |