MARITIME NAVIGATION AND RADIOCOMMUNICATION
EQUIPMENT AND SYSTEMS - AUTOMATIC IDENTIFICATION SYSTEM -

AIS AtoN stations - Minimum operational and performance requirements - methods of test and required test results

- FOREWORD

1)	The IEC (International Electrotechnical Commission) is a worldwide organization for standardization comprising all national electrotechnical committees (IEC National Committees). The object of the IEC is to promote international co-operation on all questions concerning standardization in the electrical and electronic fields. To this end and in addition to other activities, the IEC publishes International Standards. Their preparation is entrusted to technical committees; any IEC National Committee interested in the subject dealt with may participate in this preparatory work. International, governmental and non-governmental organizations liaising with the IEC also participate in this preparation. The IEC collaborates closely with the International Organization for Standardization (ISO) in accordance with conditions determined by agreement between the two organizations.

2)	The formal decisions or agreements of the IEC on technical matters express, as nearly as possible, an international consensus of opinion on the relevant subjects since each technical committee has representation from all interested National Committees.

3)	The documents produced have the form of recommendations for international use and are published in the form of standards, technical specifications, technical reports or guides and they are accepted by the National Committees in that sense.

- In order to promote international unification, IEC National Committees undertake to apply IEC International Standards transparently to the maximum extent possible in their national and regional standards. Any divergence between the IEC Standard and the corresponding national or regional standard shall be clearly indicated in the latter.

- The IEC provides no marking procedure to indicate its approval and cannot be rendered responsible for any equipment declared to be in conformity with one of its standards.

- Attention is drawn to the possibility that some of the elements of this International Standard may be the subject of patent rights. The IEC shall not be held responsible for identifying any or all such patent rights.

International Standard IEC 62320-2 has been prepared by subcommittee 14, of IEC technical committee 80:

The text of this standard is based on the following documents:

| FDIS | Report on voting |
| --- | --- |
| XX/XX/FDIS | XX/XX/RVD |

Full information on the voting for the approval of this standard can be found in the report on voting indicated in the above table.

This publication has been drafted in accordance with the ISO/IEC Directives, Part 2.

The committee has decided that the contents of this publication will remain unchanged until ______. At this date, the publication will be

•	reconfirmed;

•	withdrawn;

•	replaced by a revised edition, or

•	amended.

CONTENTS

1	Scope	5

2	Normative references	5

3	Abbreviations	6

4	Description	7

4.1	Type 1 AIS AtoN Station	8

4.1.1	Type 1 AIS AtoN Station Characteristics	8

4.1.2	Capability	9

4.1.3	Type 1 AIS AtoN Station - Alternatives	9

4.2	Type 2 AIS AtoN Station	10

4.2.1	Type 2 AIS AtoN Station Characteristics	10

4.2.2	Capability	11

4.2.3	Type 2 AIS AtoN Station – Alternatives	11

4.3	Type 3 AIS AtoN Station	11

4.3.1	Type 3 AIS AtoN Station Characteristics	11

4.3.2	Type 3 AIS AtoN Station Capability	12

4.3.3	Type 3 AIS AtoN Station – Alternatives	12

4.4	Chaining of AIS AtoN Stations (Type 2 and 3)	13

5	Requirements for AIS AtoN Stations	14

5.1	Physical Layer Requirement	14

5.1.1	Transmitter Requirements	14

5.1.2	Receiver Requirements	15

5.1.3	Electro-magnetic Compatibility (EMC) Requirements	16

5.1.4	Power Consumption	16

5.1.5	Environmental requirements	16

5.2	Link Layer Requirements	16

5.2.1	AIS Messages	16

5.2.2	Synchronisation	17

5.2.3	VDL Access Schemes	17

5.2.4	Autonomous mode	19

5.2.5	Electronic Position Fix System	21

5.2.6	Built-in Integrity Test	21

5.3	Requirements for Configuration Method	22

5.3.1	Alternative for Type 1, 2 and 3	22

5.4	Other Requirements	24

5.4.1	Additional features	24

5.4.2	Manufacturer’s Information	24

5.4.3	Marking and identification	24

6	Tests of AIS AtoN stations - method of measurement and required results	24

6.1	General	24

6.2	Test conditions	24

6.2.1	Normal test conditions	24

6.2.2	Extreme test conditions	25

6.2.3	Standard Test environment	25

6.2.4	Test Signals	26

6.2.5	Arrangements for test signals applied to the receiver input	27

6.2.6	Encoder for receiver measurements	27

6.2.7	Waiver for receivers	27

6.2.8	Impedance	27

6.2.9	Artificial antenna (dummy load)	27

6.2.10	Facilities for access	28

6.2.11	Modes of operation of the transmitter	28

6.2.12	Measurement uncertainties	28

6.3	EMC tests	29

7	AIS AtoN Station Tests	29

7.1	RF Tests (Transmitter and Receiver)	29

7.1.1	TDMA Transmitter	29

7.1.2	TDMA Receivers	35

7.1.3	Conducted Spurious Emissions at the antenna	43

8	Functional Tests	44

8.1	Tests for Configuration Method	44

8.1.1	Configure Test Message 21 (Type 1, 2, 3)	44

8.1.2	Schedule Mode A FATDMA Message 21 (Types 1, 2, 3) (single report, alternating channel operation)	45

8.1.3	Schedule Mode B FATDMA Message 21 (Types 1, 2, 3) (dual report, dual channel operation)	46

8.1.4	Schedule Mode C FATDMA Message 21 (Types 1 and 2) (Single report, single channel operation)	46

8.1.5	Schedule Mode A RATDMA Message 21 (single report, alternating channel operation)	47

8.1.6	Schedule Mode B RATDMA Message 21 (dual report, dual channel operation)	48

8.1.7	Schedule Mode C RATDMA Message 21 (single channel operation)	49

8.1.8	Addressed Binary Data Message 6	49

8.1.9	Unscheduled Transmission	50

8.1.10	Test Message 8	51

8.1.11	AIS AtoN Configuration Messages 12	51

8.1.12	AIS AtoN Configuration Messages 14	51

8.2	Tests for Synchronisation accuracy	52

8.2.1	Synchronisation Error	52

8.2.2	Synchronisation test without UTC Direct	52

8.3	Tests for EPFS	53

8.3.1	Position Source	53

8.3.2	Invalid position	53

8.3.3	Off-position monitor	54

8.4	Additional Messages	54

8.4.1	Receive addressed message	54

8.5	Additional Functionality	55

8.5.1	Test for Configuration of the receiver turn-on times	55

8.5.2	Test for Configure Proprietary AtoN Control	56

8.5.3	Test for Configuration of Payload Re-broadcast	56

Purpose	56

8.5.4	Test for Forced Broadcast	57

8.5.5	Test for Version Information	58

8.5.6	Test for AFC – AtoN Function ID capability	58

Purpose	58

Method of Measurement	58

8.5.7	Test for VDL configuration using Chaining	60

Purpose	60

Method of Measurement	60

8.6	Test for BIIT	60

8.6.1	Transmitter shutdown procedure	61

8.7	Tests for Power Supply	61

8.7.1	Average Power Consumption	61

8.8	Environmental tests	62

8.9	Other tests	62

8.9.1	Quality assurance	62

8.9.2	Additional features	62

8.9.3	Manual	62

8.9.4	Marking and identification	62

Annex A - Bits for Regional Use	63

A.1	Message 21 – Regional Bits	63

Annex B – Presentation Interface Sentences Informative	64

B.1	[note – this annex is provided as a separate document at this time for formatting reasons]	64

MARITIME NAVIGATION AND RADIOCOMMUNICATION
EQUIPMENT AND SYSTEMS -

AUTOMATIC IDENTIFICATION SYSTEM -

AIS AtoN stations - Minimum operational and performance requirements - methods of test and required test results

- Part A – The AIS AtoN Station

# Scope

This International Standard specifies the minimum operational & performance requirements, methods of testing and required test results conforming to the performance standards adopted by IMO Res. MSC.74 (69), annex 3, Universal AIS.  It incorporates the technical characteristics of non-shipborne, fixed station AIS equipment, included in recommendation ITU-R M.1371and IALA Recommendation A-124.  Where applicable, it also takes into account the ITU radio regulations.  This Standard takes into account other associated IEC International Standards and existing National Standards, as applicable.

This standard is applicable for AIS installations on aids to navigation.

NOTE All text of this standard, that is identical to that in IMO resolution MSC.74(69), Annex 3 and IMO resolution A.694(17) or to that in ITU-R M.1371is printed in italics

# Normative references

The following referenced documents are indispensable for the application of this document. For dated references, only the edition cited applies. For undated references, the latest edition of the referenced document (including any amendments) applies.

IEC 60945, Maritime navigation and radiocommunication equipment and systems – General  requirements – Methods of testing and required test results

IEC 61108-1, Maritime navigation and radiocommunication equipment and systems – Global  navigation satellite systems (GNSS) – Part 1: Global positioning system (GPS) – Receiver equipment – Performance standards, methods of testing and required test results.

IEC 61108-2, Maritime navigation and radiocommunication equipment and systems – Global navigation satellite systems (GNSS) – Part 2: Global navigation satellite system (GLONASS) – Receiver equipment – Performance standards, methods of testing and required test results

IEC 61108-41, Maritime navigation and radiocommunication equipment and systems – Global navigation satellite systems (GNSS) – Part 4: Shipborne DGPS and DGLONASS maritime radio beacon receiver equipment

IEC 61162-1, Maritime navigation and radiocommunication equipment and systems – Digital interfaces – Part 1: Single talker and multiple listeners

IEC 62287-1, Maritime navigation and radiocommunication equipment and systems – Class B shipborne equipment of the automatic identification system (AIS)- Par 1: Carrier-sense time division multiple access (CSTDMA) techniques.

ISO/IEC 3309, Information technology – Telecommunications and information exchange between systems – High-level data link control (HDLC) procedures – Frame structure IMO Resolution A.694(17):1991, General requirements for shipborne radio equipment forming part of the Global Maritime Distress and Safety System (GMDSS) and for electronic navigational aids

IALA Recommendation A.126 – The Use of Automatic Identification System (AIS) in Marine Aids to Navigation

IMO Resolution MSC.74(69), Annex 3, Recommendation on performance standards for AIS

IMO Guidelines on the operational use of AIS (provisional)

IMO Resolution A.815(19):1995, Worldwide radionavigation system

IMO Resolution MSC.140(76)   Recommendation for the protection of the AIS VHF data link

ITU-R Recommendation M.489-2, Technical characteristics of VHF radiotelephone equipment operating in the maritime mobile service in channels spaced by 25 kHz

ITU Radio Regulations and ITU-R M.585

ITU-R Recommendation M.1371 (latest edition) Technical characteristics for a universal shipborne automatic identification system using time division multiple access in the VHF maritime mobile band.

# Abbreviations

AIS	Universal ship-borne Automatic Identification System

AtoN	Aid(s) to Navigation

BIIT 	Built-in Integrity Tests

BT	 	Bandwidth Time product

CPU	Central Processing Unit

ECDIS	Electronic chart display and information system

EPFS	Electronic position fixing system

ETA	Estimated Time of Arrival

EUT	Equipment under test

FATDMA	Fixed Access Time Division Multiple Access

GBS 	see IEC 61162-1, table 5

GGA 	see IEC 61162-1, table 5

GLL 	see IEC 61162-1, table 5

HDG	Heading

HDTWPL 	see IEC 61162-1, table 5

HSC	High speed craft

IHO	International Hydrographic Office

IMO	International Maritime Organisation

LR		Long Range

MAC	Medium access control

MKD	minimum keyboard and display

MMSI	maritime mobile service identity

NUC	Not under command

OSD 	see IEC 61162-1, table 5

PER 	packet error rate

PI		Presentation Interface

RAIM	Receiver autonomous integrity monitoring

RATDMA	Random Access Time Division Multiple Access

RMC 	see IEC 61162-1, table 5

Rx		Receive

SOG	Speed over ground

SOTDMA	Self Organizing Time Division Multiple Access

TDMA	Time Division Multiple Access

Tx		Transmit

UTC	Universal Time Co-ordinated

VBW 	see IEC 61162-1, table 5

VDL	VHF Data Link

VDM 	Serial output message containing VDL information (IEC 61162-1)

VSWR	Voltage Standing Wave Ratio

VTG 	see IEC 61162-1, table 5

Note: Abbreviations related to IEC 61162 series are not included in the above list.  For their meaning refer to that International Standard.

# Description

There are three types of AIS AtoN Stations as defined in Table 1.  The AIS AtoN Stations may have additional capabilities as defined in the Alternatives column.

Table 1 – Description of AIS AtoN Stations

| Requirements | Type 1 AIS AtoN Station | Type 2 AIS AtoN Station | Type 3 AIS AtoN Station | Alternatives |
| --- | --- | --- | --- | --- |
| VDL Receiver | No Receiver | Receiver for control functions only | 2 Receiving processes for autonomous mode |  |
| Transmitted Messages | 21 |  |  | 21 plus one or more of 6, 8, 12, 14 and other appropriate messages (Types 1, 2 and 3) plus 7, 13 (Type 3 only) |
| Access Mode for Message 21 | FATDMA |  |  | FATDMA and RATDMA (Type 3 only) |
| Access Mode for Messages other than 21, if implemented |  |  |  | FATDMA (Type 1 and 2) One or more of FATDMA, RATDMA or CSTDMA (Type 3) |
| Configuration / Communication method | Defined by manufacturer |  |  | Defined by manufacturer with Standard Sentences (Type 1, 2 and 3) |
| Physical Communication Interface | None |  |  | The electrical and physical characteristics shall be defined by manufacturer. (Type 1, 2 and 3) |
| Transmit Power | 12,5 W |  |  | As defined by manufacturer (Type 1, 2 and 3) |
| Transmitter capability | Dual channel |  |  | Single channel (Type 1 and 2) |
| Synthetic and Virtual AtoN | No |  |  | Yes (Type 1, 2 and 3) |
| Positioning Device | EPFS and Surveyed Position |  |  | Surveyed Position Only (no EPFS) (Type 1, 2 and 3) |
| UTC synchronisation | Direct Only |  |  | Direct, Indirect or semaphore (Types 3) |
| Assignment | Shall not respond to assignment Messages 16 and 23 |  |  |  |
| Interrogation | Shall not respond to interrogation Message 15 |  |  |  |

## Type 1 AIS AtoN Station

Type 1 AIS AtoN Station has no receiver. It transmits on FATDMA slots given in its configuration. Figure 1 shows the functional block diagram of a Type 1 AIS AtoN Station.

Controller

Power Supply

BIIT

AIS TX

Type 1 AIS AtoN Station

External Interface

Timing and Sync Device

Configuration

EPFS

Figure 1 – Functional block diagram of a Type 1 AIS AtoN Station

### Type 1 AIS AtoN Station Characteristics

The characteristics of the Type 1 AIS AtoN Station are:

- Transmits using FATDMA,

- No receive capability, therefore:

- Cannot be configured via the VDL,

- Cannot synchronise to other stations.

- Configuration interface as defined by the manufacturer,

- 12,5 W transmitter power,

- Dual channel transmission.

#### Controller

The controller composes Message 21 and ensures the correct operation of the AIS AtoN Station on the VDL.

#### Timing and Synchronisation Device

This device provides the time and synchronisation for the controller.

#### Power Supply

The power supply generates the internal voltages.

#### BIIT

The Built-in Integrity Tests (BIIT) shall provide integrity monitoring.

#### EPFS

Electronic Position Fixing System (EPFS) provides the current position of the AtoN.

#### Configuration

The interface used to configure the AIS AtoN Station.

### Capability

Type 1 AIS is capable of transmitting Message 21 using FATDMA.

### Type 1 AIS AtoN Station - Alternatives

#### Additional Controller Capability

In addition to Message 21, the controller shall compose optional output messages to the VDL as described in Table 2.

Table 2 – Summary of optional Type 1 AIS AtoN Station messages

| Msg ID | Message Name | Message Description | Application examples |
| --- | --- | --- | --- |
| 6 | Binary Addressed Message | Binary data for addressed communication | Monitoring of AtoN lantern, power supply, etc. |
| 8 | Binary Broadcast Message | Binary data for broadcast communication | Meteorological and hydrological data |
| 12 | Addressed Safety Related Message | Safety related data for addressed communication | Warn AtoN malfunctioning |
| 14 | Broadcast Safety Related Message | Safety related data for broadcast communication | Warn AtoN malfunctioning |

#### Configuration Method

The Type 1 AIS AtoN Station may be configured using Standard Configuration Sentences.  (IEC 61162 and as defined in Annex A).

#### No EPFS

When a surveyed position is used, the EPFS may not be required.

#### TDMA Transmitter (AIS TX)

The TDMA transmitter may operate on a single channel.

#### External Interface

The external interface(s) may be used for communication with external devices (e.g. AtoN lantern, AtoN power supply, hydrological and meteorological instruments, etc.).

#### Synthetic and Virtual AtoN

The AIS AtoN Station may be capable of transmitting message 21 for synthetic and virtual AIS AtoN.

## Type 2 AIS AtoN Station

Type 2 AIS AtoN Station transmits on FATDMA slots.

Type 2 AIS AtoN Station has a Control Receiver for messages containing configuration sentences. Figure 2 shows the functional block diagram of a Type 2 AIS AtoN Station.

Controller

Power Supply

BIIT

Control RX

AIS TX

Type 2 AIS AtoN Station

External Interface

Timing and Sync Device

Configuration

EPFS

Figure 2- Functional block diagram of a Type 2 AIS AtoN Station.

### Type 2 AIS AtoN Station Characteristics

The characteristics of the Type 2 AIS AtoN Station are:

- Transmits using FATDMA,

- Limited receiver capability, therefore cannot maintain a slot map and cannot use RATDMA access scheme,

- Configuration interface as defined by the manufacturer,

- 12,5 W transmitter power,

- Dual channel transmission.

### Capability

A Type 2 AIS AtoN Station has the capabilities of a Type 1 AIS AtoN Station, with the addition of a control receiver.

#### Control Receiver

The Type 2 AIS AtoN Station shall have a receiver operating on an AIS channel for control functions only.

### Type 2 AIS AtoN Station – Alternatives

The Type 2 AIS AtoN Station alternatives include all the Type 1 AIS AtoN Station alternatives.

## Type 3 AIS AtoN Station

Type 3 AIS AtoN Station has AIS receive and transmit capabilities in accordance with ITU-R M.1371.   Figure 3 shows the functional block diagram of a Type 3 AIS AtoN Station.

Controller

Power Supply

BIIT

AIS RX

AIS RX

AIS TX

Type 3 AIS AtoN Station

External Interface

Timing and Sync Device

Configuration

EPFS

Figure 3 – Functional block diagram of a Type 3 AIS AtoN Station

### Type 3 AIS AtoN Station Characteristics

The characteristics of the Type 3 AIS AtoN Station are:

- Receiving capability on both AIS channels,

- Transmits using FATDMA.

### Type 3 AIS AtoN Station Capability

A Type 3 AIS AtoN Station has the capability of a Type 1 AIS AtoN Station, with the following addition.

#### AIS Receiver (AIS Rx)

The Type 3 AIS AtoN Station shall have two AIS receiving processes to produce and maintain a slot map for autonomous interaction with the VDL.

### Type 3 AIS AtoN Station – Alternatives

The Type 3 AIS AtoN Station alternatives include all the Type 1 and Type 2 AIS AtoN Station alternatives, except section 4.1.3.4, with the following additions.

#### Additional Controller Capability

In addition to Message 21, the controller composes optional output messages to the VDL as described in Table 3.

Table 3 – Summary of optional Type 3 AIS AtoN Station messages

| Msg ID | Message Name | Message Description | Application examples |
| --- | --- | --- | --- |
| 6 | Binary Addressed Message | Binary data for addressed communication | monitoring of AtoN equipment. |
| 7 | Binary Acknowledge Message | Acknowledge of addressed binary message |  |
| 8 | Binary Broadcast Message | Binary data for broadcast communication | Meteorological and hydrological data |
| 12 | Addressed Safety Related Message | Safety related data for addressed communication | Warn AtoN malfunctioning |
| 13 | Safety Related Acknowledge Message | Acknowledge of addressed safety related message |  |
| 14 | Broadcast Safety Related Message | Safety related data for broadcast communication | Warn AtoN malfunctioning |

#### Access Mode

##### Message 21

Transmits using FATDMA or RATDMA.

##### Messages other than Message 21

For each message other than Message 21, the Type 3 AIS AtoN Station uses FATDMA, CSTDMA or RATDMA.

#### Indirect and Semaphore Synchronisation

A Type 3 AIS AtoN Station shall be able to synchronise to other AIS stations using UTC indirect synchronisation or acting as semaphore.

## Chaining of AIS AtoN Stations (Type 2 and 3)

A chain of AIS AtoN Stations allows for communication from an AIS Base Station to AIS AtoN Stations that are remotely located and unable to communicate directly with the base station. Messages are passed from station to station until the intended recipient is reached.

The concept requires an AIS AtoN Station to have knowledge of its neighbouring AIS AtoN Stations. A “parent station” is a station that is in the direction of the base station. A “child station” is a station that is directed away from a base station. Each AIS AtoN Station in the chain must know the entire chain to prevent unnecessary retransmission of the messages.

Message 6 and 25 are used for the transfer of the encrypted binary field. It is assumed that the whole chain has the same encryption key. The Source ID field of Message 6 and 25 is used to determine whether the received message is from a parent or child station. If not, then the received message is ignored.

The encrypted binary data field is decrypted to obtain the Function ID and MMSI of the AIS AtoN Station. If the Source ID of the message is set to the parent station ID and the Function ID is a Configuration, Query Request or Function, and the MMSI of the AIS AtoN Station is in the chain, then the message shall be retransmitted, with the Source ID set to own MMSI. If the Source ID of the message is set to the child station ID and the Function ID is a Query Response, then the message shall be retransmitted, with the Source ID set to own MMSI. Any other combination of known or unknown MMSI is ignored. (Table 4 refers)

The chaining algorithm may be ignored by setting the Source ID field to “@@@@@”. The AIS AtoN Station shall attempt to decrypt the binary data field, and check that it is the intended recipient of the message before processing the message any further.

Table 4 – Chaining of AIS AtoN Stations

| Source ID | Application Identifier | MMSI from Encrypted Binary Field | Action by Own Station |
| --- | --- | --- | --- |
| Parent | Function ID = Query Response | Responding AIS AtoN Station | In all cases, if Function ID = Query Response, then ignore |
| Parent | Function ID = Configuration or Query Request or Functional | Intended recipient | If Function ID = Configuration or Query Request or Functional, then verify intended recipient is in the chain and re-transmit message with Source ID set to own station MMSI |
| Parent | Function ID = Configuration or Query Request or Functional | Own Station | If Function ID = Configuration or Query Request or Functional, then verify intended recipient is Own Station and then Process |
| Child | Function ID = Query Response | Responding AIS AtoN Station | In all cases, if Function ID = Query Response, then re-transmit message with Source ID set to own station MMSI |
| Child | Function ID = Configuration or Query Request or Functional | Intended Recipient | In all cases, if Function ID = Configuration or Query Request or Functional, then ignore |
| “@@@@@” |  | Own Station | Process |
| “@@@@@” |  | Other | Ignore |
| Other |  | Ignore | Ignore |

# Requirements for AIS AtoN Stations

## Physical Layer Requirement

### Transmitter Requirements

#### Channel

The AIS AtoN Station shall operate on dual channels, Channel 1 and Channel 2, in the VHF Maritime Mobile Service band, using 25 kHz bandwidth, according to the ITU Radio Regulation, Appendix 18.

##### Alternative for Type 1 and Type 2 AIS AtoN Station

The Type 1 and Type 2 AIS AtoN Stations shall transmit on a single channel only, either Channel 1 or Channel 2.

#### Parameter settings

Table 5 and Table 7 are derived from Recommendation ITU-R M.1371 and give the parameters required for an AIS AtoN Station. For the meaning of the symbols and additional information (footnotes) refer to the appropriate section of Recommendation ITU-R M.1371.

Table 5 - Required parameter settings for an AIS AtoN Station

| Symbol | Parameter Name | Setting (FATDMA, RATDMA) | Setting (CSTDMA) |
| --- | --- | --- | --- |
| PH.RFR | Regional frequencies | Two channels between 156,025 MHz and 162,025 MHz |  |
| PH.AIS1 | Channel 1 (default channel 1) | 161.975 MHz |  |
| PH.AIS2 | Channel 2 (default channel 2) | 162.025 MHz |  |
| PH.BR | Bit rate | 9 600 bps |  |
| PH.TS | Training sequence | 24 bits |  |
| PH.TST | Transmitter settling time (Transmit power within 20% of final value. Frequency stable to within  1.0 kHz of final value). Tested at manufacturers declared transmit power. |  1.0 ms | ≤313 µs |
|  | Ramp down time |  832 µs | ≤313 µs |
|  | Transmission duration | ≤ 80 ms | ≤23 333 µs |
|  | Transmission delay | no delay | 2 083 µs |
|  | Transmitter output power | 12,5 W or as defined by manufacturer |  |

In addition, the constants of the physical layer of the AIS AtoN Station shall comply with the values given in Table 6 and Table 7.

Table 6 - Required Settings of Physical Layer Constants

| Symbol | Parameter name | Value |
| --- | --- | --- |
| PH.DE | Data encoding | NRZI |
| PH.FEC | Forward error correction | Not used |
| PH.IL | Interleaving | Not used |
| PH.BS | Bit scrambling | Not used |
| PH.MOD | Modulation | Bandwidth adapted GMSK |

Table 7 - Modulation parameters of the Physical Layer of the AIS AtoN Station

| Symbol | Name | Value |
| --- | --- | --- |
| PH.TXBT | Transmit BT-product | 0.4 |
| PH.RXBT | Receive BT-product | 0.5 |
| PH.MI | Modulation Index | 0.5 |

#### Transmitter Shutdown

An automatic transmitter shutdown shall be provided to ensure that transmission does not continue for more than 2 seconds.  This shutdown shall be independent of the operating system software.

#### Transmitter requirements

The technical characteristics as specified in Table 8 should apply to the TDMA transmitter.

Table 8 - Minimum required TDMA transmitter characteristics

| Transmitter parameters | Requirements |
| --- | --- |
| Carrier Power Error |  1,5 dB |
| Carrier Frequency Error |  500 Hz |
| Slotted Modulation Mask | –25 dBc Δfc< ±10 kHz –60 dBc ±25 kHz<Δfc< ±62.5 kHz |
| Transmitter Test Sequence & Modulation Accuracy | <3400 Hz for Bit 0, 1 2400 ± 480 Hz for Bit 2, 3 2400 ± 240 Hz for Bit 4 ... 31 For Bits Bit 32 …199 1740 ± 175 Hz for a bit pattern of 0101 2400 ± 240 Hz for a bit pattern of 00001111 |
| Transmitter Output Power versus Time | Power within mask shown in – Power versus time mask and timings given in Table 11. |
| Spurious emissions | –36 dBm 9 kHz ... 1 GHz –30 dBm 1GHz ... 4 GHz |

### Receiver Requirements

The technical characteristics as specified in Table 9 should apply to the TDMA receivers.

Table 9 - Required receiver characteristics

| Receiver Parameters | AIS Receiver | Control Receiver |
| --- | --- | --- |
| Sensitivity | 20% PER@ –107 dBm | 20% PER @ –97 dBm |
| High Input Levels | 2% PER @ -77 dBm 10% PER @ -7 dBm | 2% PER @ -77 dBm 10% PER @ -7 dBm |
| Co-channel rejection | 20% PER @ -10 dB | 20% PER @ -16 dB |
| Adjacent channel selectivity | 20% PER @ 70 dB | 20% PER @ 60 dB |
| Spurious response rejection | 20% PER @ 70 dB | 20% PER @ 60 dB |
| Intermodulation response rejection | 20% PER @ 74 dB | 20% PER @ 64 dB |
| Blocking and Desensitisation | 20% PER @ 78 dB inband & 86 dB outband | 20% PER @ 68 dB inband & 76 dB outband |

### Electro-magnetic Compatibility (EMC) Requirements

The AIS AtoN Station shall meet the EMC requirements as declared by the manufacturer. These environmental conditions shall be one of:

- IEC 60945,

- As defined by manufacturer.

### Power Consumption

The manufacturer shall state the average power consumed by the AIS AtoN Station under defined test conditions.

### Environmental requirements

The AIS AtoN Station shall meet the environmental conditions requirements as declared by the manufacturer. These environmental conditions shall be one of:

- IEC 60945 “Protected”,

- IEC 60945 “Exposed”, or

- As defined by manufacturer.

## Link Layer Requirements

The Link layer specifies how data shall be formatted and transmitted on the VDL.

The link layer requirements are referenced to ITU-R M.1371.

### AIS Messages

#### Message 21 Format and Content

The AIS AtoN Station shall broadcast Message 21, as defined in ITU-R M.1371.

##### Regional Bits

In Message 21, the status bits (7 6 5 4 3 2 1 0) are numbered such that bit 7 is the most significant bit, and bit 0 is the least significant bit. The first three bits (i.e. 7, 6 and 5) shall be used to define a Page ID. The Page ID can range from 0 to 7, allowing 8 pages. Page ID 0 shall not be used for the Regional/International application. Page IDs 1 to 6 are reserved for future use.

Page ID 7 (binary 111) shall be as defined in this standard (see Annex A).

If the regional bits are not used, they shall be set to ‘0’.

##### Virtual and Synthetic AIS AtoN Message

An AIS AtoN Station, when broadcasting Message 21 for Virtual and Synthetic AtoN, shall use the MMSIs allocated to the Virtual and Synthetic AtoN as issued under the same series for Real AIS AtoN Stations.

#### Alternative Messages

In addition to Message 21, the AIS AtoN Station may transmit other messages, in accordance with ITU-R M.1371. These are summarised in Table 1.

##### Type 1 and 2

Allowed alternative messages shall be message 21 and one or more of 6, 8, 12, 14 and other appropriate messages.

##### Type 3

Allowed alternative messages shall be message 21 and one or more of 6, 7, 8, 12, 13, 14 and other appropriate messages.

### Synchronisation

Synchronisation is used to determine the TDMA frames and individual slots so that the transmission of the AIS Message is performed within the desired slot.  The synchronisation for the AtoN AIS shall be UTC direct or UTC indirect mode.

When UTC direct synchronisation has failed, the AIS AtoN Station shall behave as configured.

The Type 3 AIS AtoN Station shall use UTC indirect synchronisation (if implemented).  When UTC synchronisation has failed, the Type 3 AIS AtoN Station may use semaphore synchronisation.

#### Synchronisation Accuracy

The transmission timing error, including jitter, of the AtoN AIS shall be within the limits as defined in Table 10, referring to an ideal transmission as defined by ITU-R M.1371.

Table 10 – Maximum allowed time error

| Synchronisation Mode | Maximum allowed Time Error |
| --- | --- |
| UTC direct synchronisation | ±1bit (±104 s) |
| UTC indirect synchronisation | ±3bits (±312 s) |
| Semaphore synchronisation | ±3bits (±312 s) |

### VDL Access Schemes

The AIS AtoN Station shall use FATDMA for the transmission of Message 21.

The AIS AtoN Station may optionally transmit Message 6, 7, 8, 12, 13, 14 and 25. The maximum length of Messages 6, 8, 12, and 14 is three slots per Message when using FATDMA or RATDMA (if implemented).  CSTDMA may be used for one slot Messages only.

#### Type 3 AIS AtoN Station

The Type 3 AIS AtoN Station shall use FATDMA or RATDMA (if implemented) for Message 21. The Type 3 AIS AtoN Station shall use the VDL access scheme defined by its configuration.

Single slot binary and safety related messages shall be transmitted using FATDMA or RATDMA or CSTDMA, if implemented.

Single slot acknowledgement messages 7 and 13 shall be transmitted within 4 seconds of receiving messages 6 and 12 using FATDMA, CSTDMA or RATDMA.

#### FATDMA and RATDMA VDL Access

The transmitter shall begin transmission by turning on the RF power after slot start (T0). The unit shall and reach -3dB before TB1 (see Figure 4).

The transmitter shall be turned off after the last bit of the transmission packet has left the transmitting unit; nominal transmission end is Te.

RATDMA shall use slots according to ITU-R M.1371.

Figure 4 – Power versus time mask

The access to the medium is performed as shown in Figure 4 and Table 11.

Table 11 – Definitions of timing for Figure 4

| Reference |  | bits | Time in milliseconds | Definition |
| --- | --- | --- | --- | --- |
| T0 |  | 0 | 0 | Start of transmission slot. Power shall NOT exceed –50 dB of Pss before To |
| TA |  | 0-6 | 0-0.624 | Power exceeds –50 dB of Pss |
| TB | TB1 | 6 | 0.624 | Power shall be within +1,5 or -3 dB of Pss |
|  | TB2 | 8 | 0.8324 | Power shall be within +1,5 or -1 dB of Pss |
| TE |  | 104 - 748 | 10.833 - 77.917 | Power shall remain within +1,5 or -1 dB of Pss during the period TB2 to TE The TE can vary depending on message type, data content and bit stuffing bits from minimum 104 bits for the shortest possible message (message 14 and no text content) to maximum length of 740 bits for a three slot message. TE shall not exceed; - 236 bits for a one slot message - 492 bits for a two slot message - 748 bits for a three slot message A station may occupy at maximum three consecutive slots for one (1) continuous transmission. Only a single application of the overhead (ramp up, training sequence, flags, FCS, buffering) is required for a long transmission packet. The length of a long transmission packet should not be longer than necessary to transfer the data; i.e. the AIS should not add filler. |
| TF |  | 112 - 756 | 11.667 – 78.787 | Power shall be -50 dB of Pss and stay below this |
| TG |  | 256, 512 or 768 | 26.667 one slot TX 53.333 two slot TX 80.000 three slot TX | Start of next transmission time period |

##### Link sub-layer 1: Medium Access Control (MAC)

Refer to ITU-R M.1371 and Section 5.2.1 for synchronisation.

##### Link sub-layer 2: Data Link Service (DLS)

Refer to ITU-R M.1371.

##### Link sub-layer 3: Link Management Entity (LME)

Refer to ITU-R M.1371.

#### CSTDMA VDL Access Mode

The operation of CSTDMA in the AIS AtoN Station shall be in accordance with ITU-R M.1371 and tested according to IEC 62287-1, however the ATON AIS is allowed to use the same transmit power setting for CSTDMA as for RATDMA and FATDMA.

All CSTDMA transmissions shall be limited to one slot.

### Autonomous mode

The AIS AtoN Station shall always operate autonomously and determine its own schedule for transmission of its messages based on its configuration. The station shall automatically resolve scheduling conflicts with other stations when using CSTDMA and RATDMA.

#### Message 21 Reporting Intervals

The reporting interval for Message 21 shall be 3 minutes by default. This shall be configurable to other reporting intervals.

The AIS AtoN Station shall be configurable to decrease the reporting interval for Message 21 when the AtoN is off-position.

#### Channel Operation

The AIS AtoN Station shall use channels as identified in this section.

##### Reporting Modes for Message 21

The AIS AtoN Station shall transmit Message 21 at the configured reporting interval.  As indicated in Figure 5, transmissions shall be:

- Mode A operation – Message 21 transmission alternates between Channel 1 and Channel 2 in a subsequent frame that is nominally one reporting interval later.  Message 21 content is updated for each message, or

- Mode B operation – The same Message 21 transmitted on Channel 1 and Channel 2 in quick (nominally 4 seconds) succession. The first transmission of each Message 21 may be on either Channel 1 or Channel 2.  The second transmission shall be on the other channel), or

- Mode C operation – Message 21 transmitted on a single channel, either Channel 1 or Channel 2. Message 21 content updated at each reporting interval.

Figure 5 – Reporting Modes for Message 21

##### Single Channel Operation for Message 21

The Type 1 and Type 2 AIS AtoN Station shall transmit on the designated channel using FATDMA slots of the selected frames in the UTC hour.  (as per Mode C, Figure 5)

### Electronic Position Fix System

#### Position Source

An EPFS shall be used as the source for AtoN position reporting unless a surveyed position is used.

If the internal EPFS is a GNSS receiver it shall meet the following requirements of IEC 61108 series: position accuracy, acquisition, re-acquisition, receiver sensitivity, RF dynamic range, interference susceptibility, position update, failure warnings, status indications and integrity flag, provide a resolution of one ten-thousandth of a minute of arc and uses WGS 84 datum.

If another type of EPFS is used, then it shall meet the requirements of the applicable standard and use WGS 84 datum.

##### Augmentation Systems

The EPFS may be capable of being corrected using any suitable augmentation system (e.g. SBAS, radiobeacon DGNSS, evaluation of message 17, etc.). The manufacturer shall declare which augmentation systems can be used, and that the augmentation system does not adversely affect Message 21 transmissions.

The manufacturer shall declare if the EPFS is not capable of being corrected.

##### Invalid position

If the EPFS device is unable to provide a valid position fix, then the reported position shall be longitude = 181° = not available = default and latitude = 91° = not available = default and the time stamp field shall be set to a value of 63.

##### Off-position monitoring

If the floating AtoN is within its on-position limits, the Off-Position Indicator shall be set to “0” in the transmitted message 21.

If a floating AtoN is off-position, the AIS AtoN Station shall identify this condition and the Off-Position Indicator shall be set to “1” in the transmitted message 21.  The reporting interval when the AIS AtoN Stations is off position shall be determined by its configuration.  (5.2.4.1 refers)

#### Alternatives for Types 1, 2 and 3

##### Position Source

If a surveyed position is used, an EPFS is not required.

When a surveyed position is used, the latitude and longitude fields of the transmitted Message 21 shall contain the surveyed position, the “type of electronic position fixing device” is set to “7” (surveyed), “RAIM-Flag” field is set to “0”, Off-position Indicator field is set to “0” and the “position accuracy” field is set in accordance to the accuracy of the surveyed position (i.e. “1” if better then 10 m, otherwise “0”).

### Built-in Integrity Test

The AIS AtoN Station shall have a Built in Integrity Test (BIIT) process which tests for conditions as described Table 12.  If standard configuration sentences are used, the warning / notification conditions shall be sent via sentence ADS.

Table 12 – AIS AtoN Station reaction to BIIT conditions

| Condition | Reaction of the AIS AtoN Station |
| --- | --- |
| AIS: Tx malfunction | Stop transmission |
| AIS: Antenna VSWR exceeds limit (only if external interface used) | Continue operation |
| AIS: Rx Channel 1 malfunction | Stop RATDMA and CSTDMA transmissions on affected channel |
| AIS: Rx AIS B malfunction | Stop RATDMA and CSTDMA transmissions on affected channel |
| AIS: General failure | Stop transmission |
| AIS: EPFS failure | Continue operation |
| AIS: Direct Synchronisation Failure | Configurable action |
| AIS: Synchronisation Lost | Configurable action |
| AIS: DGNSS input failed | Continue operation |

## Requirements for Configuration Method

The configuration method shall be as defined by manufacturer and held in non-volatile memory.

Configuration may use standard configuration sentences (see Error: Reference source not found) either directly or via the VDL.

The configuration method shall:

- Configure the content for message 21,

- Configure transmission parameters for message 21 and any other messages supported by the manufacturer,

- Configure the behaviour of the AIS AtoN Station when synchronisation is lost,

- Configure the behaviour of the AIS AtoN Station when off position.

The manufacturer shall provide a means to verify configuration and version information of the AIS AtoN Station.

### Alternative for Type 1, 2 and 3

#### Standard IEC 61162 Sentences

The Standard Configuration Sentences shall be as defined in IEC 61162 series.

Table 13 provides an overview of the sentences that shall be used for configuration of AIS AtoN applications. It includes existing sentences from IEC 61162 with additional AtoN sentences as provided in Annex B.

Table 13 – Standard Sentences

| Sentence Formatter | Input |  |  | Output |  |  | Description / Comments |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Type 1 AIS AtoN Station | Type 2 AIS AtoN Station | Type 3 AIS AtoN Station | Type 1 AIS AtoN Station | Type 2 AIS AtoN Station | Type 3 AIS AtoN Station |  |
| AAR | X | X | X | Q | Q | Q | Configure Broadcast Rates for AIS AtoN Station Messages. |
| ACE | X | X | X | Q | Q | Q | Extended General AIS AtoN Station Configuration |
| ACF | X | X | X | Q | Q | Q | General AIS AtoN Station Configuration |
| AFB | X | X | X |  |  |  | Force Broadcast |
| AFC |  |  |  | Q | Q | Q | AtoN Station Function Capability |
| AID | X | X | X | Q | Q | Q | Configure or change MMSI |
| AKE |  | X | X |  | Q | Q | Define Encryption Key |
| ARW |  | X | X |  | Q | Q | Receiver Turn on Times |
| TSP | X | X | X | Q | Q | Q | Configure Prohibited slots |
| MCR | X | X | X | Q | Q | Q | Proprietary AtoN Control |
| MPR | X | X | X |  |  |  | Message Payload Rebroadcast |
| VER |  |  |  | Q | Q | Q | Version |
| ABM | X | X | X |  |  |  | Addressed Binary Message |
| BBM | X | X | X |  |  |  | Broadcast Binary Message |
| ABK |  |  |  | X | X | X | Acknowledgement Message |
| VDM | X | X | X |  | X | X | VHF Data link Message |
| VDO |  |  |  | X | X | X | VHF Data-link Own-vessel message |
| NAQ |  |  |  | X | X | X | Output when a Command Fails to Execute |

X – Indicates input to or output from the AIS AtoN Station.

Q – Indicates that the sentence may be externally requested using the IEC61162 standard query sentences.

#### Chaining of AIS AtoN Stations

The AIS AtoN Station may support chaining to communicate messages to other AIS AtoN Stations. (section 4.4 refers)

No additional IEC 61162 sentences are required to support this functionality.

## Other Requirements

### Additional features

Additional features shall not adversely affect the transmission of Message 21.

### Manufacturer’s Information

The information shall describe:

- Factory default MMSI,

- External interfaces,

- Configuration of the AIS AtoN Station,

- Hardware and electrical specifications,

- Average power consumption,

- Implementation method for firmware upgrades.

### Marking and identification

The AIS AtoN Station shall be marked with the following information:

- Identification of the manufacturer,

- Model identification,

- Serial number of the unit, and

- Operating voltage.

The title and version of each software element included in the installed software system shall be either marked on the equipment or displayed on command.

# Tests of AIS AtoN stations - method of measurement and required results

## General

Physical test parameters and testing subject to national requirements and may override parameters stated below.

## Test conditions

### Normal test conditions

#### Temperature and humidity

Temperature and humidity shall be within following range:

Temperature		+15° C  to  +35° C

Humidity		20 %  to   75 %

#### Power supply

The normal supply power used for the tests shall be in accordance with the nominal power of the EUT declared by the manufacturer and taking into account the variations set by local safety regulations concerning power supplies e.g. IEC 60950 as applicable in many countries.

### Extreme test conditions

Extreme test conditions are as specified in IEC 60945. Where required, tests under extreme test conditions shall be a combination of:

- dry heat and upper limit of supply voltage applied simultaneously, and

- low temperature and lower limit of supply voltage applied simultaneously.

### Standard Test environment

The EUT is tested in an environment using test equipment to measure the transmitted messages. The EUT will be configured via the configuration interface prior to the tests. Operation is checked on channels in the maritime mobile band. Refer to Figure 6.

Data input &

Display

external evaluation

(Laptop)

AIS - Equipment

under test

(EUT)

Test installation

Target

Simulator

TX

RX

RX

Message Input, configuration

Message output,  documentation

GNSS

Radioprocessor

Configuration Interface or PI

Figure 6 – Block diagram of AIS AtoN test setup

### Test Signals

#### Standard Test Signal Number 1

A series of 010101 as the data within an AIS message frame, with header, start flag, end flag and CRC. NRZI is not applied to the 010101 bit stream or CRC (i.e. unaltered "On Air" data).  The RF should be ramped up and down on either end of the AIS message frame.

#### Standard Test Signal Number 2

A series of 00001111 as the data within an AIS message frame, with header, start flag, end flag and CRC. NRZI is not applied to the 00001111 bit stream or CRC. The RF should be ramped up and down on either end of the AIS message frame.

NOTE: Transmitters may have limitations concerning their maximum continuous transmit time and/or their transmission duty cycle. It is intended that such limitations are respected during testing.

#### Standard Test Signal Number 3

A Pseudo Random Sequence (PRS) as specified in ITU/T Recommendation 0.153 as the data within an AIS message frame with header, start flag, end flag and CRC. NRZI is not applied to the PRS stream or CRC. The RF should be ramped up and down on either end of the AIS message frame.

#### Standard Test Signal Number 4

This test signal consists of 200 packets grouped into clusters of 4 as described in Figure 8. Each cluster consists of 2 consecutive transmissions of the packets described in Table 19.

NRZI shall be applied to every packet. After sending packet 1 and 2 the notional initial state of the NRZI process shall be inverted and then packet 1 and 2 repeated.

Packet 1

Packet 2

Packet 1

Packet 2

Invert initial NRZI state here

Between every transmitted packet there shall be at least 2 free time periods. The RF carrier shall be switched off between packets to simulate normal operation.

Figure 7 – Format for repeating four-packet cluster

Table 14 – Content of first two packets

| Packet | Parameter | Bits | Contents | Comment |
| --- | --- | --- | --- | --- |
| 1 | Training | 22 | 0101....0101 | Preamble reduced by 2 bits because of ramp-up overlap |
|  | Start flag | 8 | 01111110 |  |
|  | Data | 168 | Pseudo Random | As per Table XX[fixed PRS data derived from … table] |
|  | CRC | 16 | Calculated |  |
|  | End flag | 8 | 01111110 |  |
| 2 | Training | 22 | 1010....1010 | Preamble reduced by 2 bits because of ramp-up overlap |
|  | Start flag | 8 | 01111110 |  |
|  | Data | 168 | Pseudo Random | As per Table 20 |
|  | CRC | 16 | Calculated |  |
|  | End flag | 8 | 01111110 |  |

Table 15 – Fixed PRS data derived from ITU-T.0.153

| Address | Contents (HEX) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0-7 | 0x04 | 0xF6 | 0xD5 | 0x8E | 0xFB | 0x01 | 0x4C | 0xC7 |
|  | 0000.0100 | 1111.0110 | 1101.0101 | 1000.1110 | 1111.1011 | 0000.0001 | 0100.1100 | 1100.0111 |
| 8-15 | 0x76 | 0x1E | 0xBC | 0x5B | 0xE5 | 0x92 | 0xA6 | 0x2F |
|  | 0111.0110 | 0001.1110 | 1011.1100 | 0101.1011 | 1110.0101 | 1001.0010 | 1010.0110 | 0010.1111 |
| 16-20 | 0x53 | 0xF9 | 0xD6 | 0xE7 | 0xE0 | 21 Byte's = 168 bits (+ 4 stuffed bits) , CRC = 0x3B85 |  |  |
|  | 0101.0011 | 1111.1001 | 1101.0110 | 1110.0111 | 1110.0000 |  |  |  |

### Arrangements for test signals applied to the receiver input

Sources of test signals for application to the receiver input shall be connected in such a way that the source impedance presented to the receiver input is 50 .

This requirement shall be met irrespective of whether one or more signals using a combining network are applied to the receiver simultaneously.

The levels of the test signals at the receiver input terminals (RF socket) shall be expressed in terms of dBm.

The effects of any intermodulation products and noise produced in the test signal sources shall be negligible.

### Encoder for receiver measurements

Whenever needed and in order to facilitate measurements on the receiver, an encoder for the data system shall accompany the EUT, together with details of the normal modulation process. The encoder is used to modulate a signal generator for use as a test signal source.

Complete details of all codes and code format(s) used shall be given.

### Waiver for receivers

If the EUT has two TDMA receivers, and the manufacturer declares that both TDMA receivers are identical, the test shall be limited to one receiver and the test for the second receiver shall be waived. The test report shall mention this.

### Impedance

In this standard the term "50 " is used for a 50  non-reactive impedance.

### Artificial antenna (dummy load)

Tests shall be carried out using an artificial antenna, which shall be a non-reactive non-radiating load of 50  connected to the antenna connector.

NOTE: 	Some of the methods of measurement described in this standard for the transmitter(s), allow for two or more different test set ups in order to perform those measurements. The corresponding figures illustrate therefore one particular test set up, and are given as examples. In many of those figures, power attenuators (providing a non-reactive non-radiating load of 50  to the antenna connector) have been shown. These attenuators are not "artificial antennas" as defined in 6.2.9.  The method of measurement used shall be stated in the test report.

### Facilities for access

All tests shall be performed using the standard port(s) of the EUT, where provided. Where access facilities are required to enable any specific test, these shall be provided by the manufacturer.

### Modes of operation of the transmitter

For the purpose of the measurements according to this standard, there shall be a facility to operate the transmitter unmodulated.

Alternatively, the method of obtaining an unmodulated carrier or special types of modulation patterns may also be decided by agreement between the manufacturer and the test laboratory. It shall be described in the test report. It may involve suitable temporary internal modifications of the equipment under test.

### Measurement uncertainties

Maximum values of absolute measurement uncertainties shall be as indicated in Table 16.

Table 16 – Maximum values

| Parameter | Maximum Value |
| --- | --- |
| RF frequency | ± 1 x 10-7 |
| RF power | ± 0.75 dB |
| Adjacent channel power | ± 5 dB |
| Conducted spurious emission of transmitter | ± 4 dB |
| Conducted spurious emission of receiver | ± 3 dB |
| Two-signal measurement | ± 4 dB |
| Three-signal measurement | ± 3 dB |
| Radiated emission of transmitter | ± 6 dB |
| Radiated emission of receiver | ± 6 dB |
| Transmitter attack time | ± 20 % |
| Transmitter release time | ± 20 % |
| Transmitter transient frequency (frequency difference) | ± 250 Hz |

For the test methods according to this standard, these uncertainty figures are valid to a confidence level of 95 %.

The interpretation of the results recorded in a test report for the measurements described in this standard shall be as follows:

- the measured value related to the corresponding limit shall be used to decide whether an equipment meets the requirements of this standard,

- the actual measurement uncertainty of the test laboratory carrying out the measurements, for each particular measurement, shall be included in the test report,

- the values of the actual measurement uncertainty shall be, for each measurement, equal to or lower than the figures given in this clause (absolute measurement uncertainties).

## EMC tests

Refer to National Standards for EMC testing.

# AIS AtoN Station Tests

## RF Tests (Transmitter and Receiver)

### TDMA Transmitter

Unless otherwise stated, all transmitter tests shall be performed at the highest power setting.

#### Frequency Error

Purpose

The frequency error of the transmitter is the difference between the measured carrier frequency in the absence of modulation and its required frequency.

Method of measurement

Transmitter under

test

Power attenuator

Frequency

meter

Figure 8 – Measurement arrangement

- The equipment shall be connected as illustrated in Figure 8.

- The carrier frequency shall be measured in the absence of modulation.

- The measurement shall be made under normal test conditions and extreme test conditions.

- The test shall be performed on the lowest operating frequency and the highest operating frequency as declared by the manufacturer.

Required results

The frequency error shall not exceed ± 0,5 kHz, under normal test conditions and ± 1 kHz under extreme test conditions.

#### Carrier Power

Purpose

The transmitter carrier power conducted (Pc) is the mean power delivered to a nominal 50 Ohm load during a radio frequency cycle. The rated power shall be nominally 12,5 W or as declared by the manufacturer.  The carrier power accuracy shall be tested at the nominal level of 12,5W or the level declared by the manufacturer.

Method of measurement

- The equipment shall be connected as illustrated in Figure 9.

- The carrier power shall be measured in the absence of modulation.

- The measurement shall be made under normal test conditions and extreme test conditions.

- The test shall be performed at the lowest and highest operating frequencies as declared by the manufacturer

- If the manufacturer optionally declare multiple power settings then the carrier power test shall be repeated at those settings at both the lowest and highest operating frequency of the EUT.

Transmitter under

test

Power attenuator

Power

meter

Figure 9 – Measurement arrangement

Required results

Pc shall be within  1,5 dB of the rated nominal power under normal conditions and within  3dB of the rated nominal power under extreme conditions.

#### Modulation Spectrum Slotted Transmission

Purpose

This test is to ensure that the modulation and transient sidebands produced by the transmitter under normal operating conditions fall within the allowable mask.

Method of measurement

- The test shall use test signal number 3.

- The EUT shall be connected to a spectrum analyser. A resolution bandwidth of 1 kHz, video bandwidth of 3 kHz or greater and positive peak detection (maximum hold) shall be used for this measurement. A sufficient number of sweeps shall be used and sufficient transmission packets measured to ensure that the emission profile is developed.

- Tests shall be performed on the lowest operating frequency on which the EUT can transmit according the manufacturers specification and Channel 2 (162,025 MHz).

Required result

The spectrum for slotted transmission shall be within the emission mask as follows:

- in the region between the carrier and  10 kHz removed from the carrier, the modulation and transient sidebands shall be below 0 dBc;

- at 10 kHz removed from the carrier, the modulation and transient sidebands shall be below  –25 dBc;

- at 25 kHz to  62,5 kHz removed from the carrier, the modulation and transient sidebands shall be below the lower value of –60 dBc or –30 dBm;

- in the region between  10 kHz and  25 kHz removed from the carrier, the modulation and transient sidebands shall be below a line specified between these two points.

The reference level for the measurement shall be the carrier power (conducted) recorded for the appropriate test frequency in 7.1.1.2.

For information the emission mask specified above is shown in Figure 10.

Figure 10 – Emission mask

#### Transmitter Test Sequence and Modulation Accuracy

Purpose

The test is to verify that the training sequence starts with a 0 and is a 0101 pattern of 24 bits.   The peak frequency deviation is derived from the baseband signal to verify modulation accuracy.

#### Method of measurement

Figure 11 – Measurement arrangement for modulation accuracy

The measurement procedure shall be as follows:

- The equipment shall be connected in either configuration A or configuration B as shown.   The trigger device is optional if the equipment is capable of synchronising to the transmitted bursts,

- The transmitter shall be tuned to Channel 2 (162,025 MHz),

- The transmitter shall be modulated with test signal number 1,

- The deviation from the carrier frequency shall be measured as a function of time,

- The transmitter shall be modulated with test signal number 2,

- The deviation from the carrier frequency shall be measured as a function of time,

- Measurements shall be repeated at the lowest frequency on which the EUT can transmit, according to the manufacturer's specification,

- Testing shall be repeated under extreme test conditions.

Required results

In each case, verify that the training sequence begins with ‘0’

Peak frequency deviation at various points within the data frame shall comply with Table 22. These limits apply to both the positive and negative modulation peaks. Bit 0 is defined as the first bit of the training sequence.

Table 17 – Peak frequency deviation vs. time

| Measurement period from centre to centre of each bit | Test signal 1 |  | Test signal 2 |  |
| --- | --- | --- | --- | --- |
|  | Normal | Extreme | Normal | Extreme |
| Bit 0 to bit 1 | <3 400 Hz |  |  |  |
| Bit 2 to bit 3 | 2 400 ±480 Hz |  |  |  |
| Bit 4 to bit 31 | 2 400 ±240 Hz | 2 400 ±480 Hz | 2 400 ±240 Hz | 2 400 ±480 Hz |
| Bit 32 to bit 199 | 1 740 ±175 Hz | 1 740 ±350 Hz | 2 400 ±240 Hz | 2 400 ±480 Hz |

#### Transmitter output power versus time function

Definition

Transmitter output power versus time function is a combination of the transmitter delay, attack time, release time and transmission duration as defined in Table 14 where:

- Transmitter delay time (TA -To) is the time between the start of the slot and the moment when the transmit power may exceed –50 dB of the steady-state power (Pss).

- Transmitter attack time (TB2 - TA) is the time between the transmit power exceeding –50 dBc and the moment when the transmit power maintains a level within + 1,5 / - 1dB from Pss.

- Transmitter release time (TF - TE) is the time between the end flag being transmitted and the moment when the transmitter output power has reduced to a level 50 dB below Pss and remains below this level thereafter.

- Transmission duration (TF - TA) is the time from when power exceeds – 50 dBc to when the power returns to and stays below – 50 dBc.

Figure 12 – Power versus time mask

Table 18 - Definition of timings for Figure

| Reference |  | Bits | Time | Definition |
| --- | --- | --- | --- | --- |
| T0 |  | 0 | 0 ms | Start of transmission slot. Power shall NOT exceed –50 dB of Pss before To |
| T0 - TA |  | 0-6 | 0-0,624 ms | Power may exceed –50 dB of Pss* |
| TB | TB1 | 6 | 0,624 ms | Power shall be within +1,5 or -3 dB of Pss* |
|  | TB2 | 8 | 0,8324 ms | Power shall be within +1,5 or -1 dB of Pss* |
| TE (includes 1 stuffing bit) |  | 231 | 24,024 ms | Power shall remain within +1,5 or -1 dB of Pss during the period TB2 to TE* |
| TF (includes 1 stuffing bit) |  | 239 | 26,146 ms | Power shall be -50 dB of Pss and stay below this |
| TG |  | 256 | 26,624 ms | Start of next transmission time period |

* There shall be no modulation of the RF after the termination of transmission (TE) until the power has reached zero and next slot begins (TG).

Method of measurement

- The measurement shall be carried out by transmitting test signal number 1 (note that this test signal generates one additional stuffing bit within its CRC portion).

- Tests shall be performed on 2 channels (Lowest declared frequency and 162,025 MHz).

- The EUT shall be connected to a spectrum analyser.

- A resolution bandwidth of 1 MHz, video bandwidth of 1 MHz and a sample detector shall be used for this measurement.

The analyser shall be in zero-span mode for this measurement. The spectrum analyser shall be synchronised to the nominal start time of the slot (T0), which may be provided externally, or from the EUT.

Required result

The transmitter power shall remain within the mask shown in Figure 12 and associated timings given in Table 18.

### TDMA Receivers

#### Sensitivity

Purpose

The maximum usable sensitivity (data or messages, conducted) is the minimum signal level at the receiver input, produced by a carrier at the specified frequency of the receiver, modulated with the specified test signal, which will, without interference, produce a data signal with a specified packet error rate (PER) after demodulation.

Method of measurement

Message generator

Signal generator

Receiver under test

Message measuring test set

Figure 13 – Measurement arrangement

The measurement procedure shall be as follows:

- The signal generator shall be at the lowest frequency of the receiver as declared by the manufacturer and shall be modulated to generate test signal number 4.

- The signal level at the input of the receiver shall be set to –107 dBm for a Type 3 device and -97 dBm for a Type 2 device.

- The message measuring test set shall be monitored and the packet error rate observed. The PER shall be derived by the following formula:

PER = (PTX – PRX)/PTX 100 ( %)

where

PRX is the number of packets received without errors,

PTX is the number of transmitted packets.

- The test shall be at the highest frequency declared by the manufacturer.

- Repeat under extreme conditions, at either the lowest or the highest declared frequency. The signal generator shall be adjusted so the level at the input to the receiver is –101 dBm for a Type 3 device and -91dBm for a Type 2 device.

Required results

Maximum PER of 20 %

#### Error Behaviour at High Input Levels

Purpose

The error behaviour (performance) at high input levels is defined in the same manner as for the measurement of the maximum usable sensitivity when the level of the wanted signal is 100 dB above the maximum wanted sensitivity.

Method of measurement

Message generator

Signal generator

Receiver under test

Message measuring test set

Figure 14 – Measurement arrangement

The measurement procedure shall be as follows:

- The measurement configuration for receiver sensitivity (7.1.2.1) shall be used.

- The signal generator shall be modulated to generate test signal number 4.  The test shall be carried out at the lowest and the highest  TDMA frequency declared by the manufacturer.  The message measuring test set shall be monitored and the packet error rate observed.

- The level of the input signal shall be adjusted to a level of –77 dBm;

- The level of the input signal shall be adjusted to a level of –7 dBm;

- 200 packets shall be transmitted and the PER shall be calculated.

Required results

The PER shall not exceed 2 % under c) and 10 % under d).

#### Co-Channel Rejection

Purpose

The co-channel rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due the presence of an unwanted modulated signal, both signals being at the specified frequency of the receiver.

Method of measurement

Signal generator

A

Combiner

Receiver under

test

Message measuring test set

Signal generator

B

Figure 15 – Measurement arrangement

The measurement procedure shall be as follows:

- Two generators A and B, shall be connected to the receiver via a combining network,

- The wanted signal, provided by signal generator A, shall be at the lowest declared frequency of the receiver and shall be modulated to generate test signal number 4,

- The unwanted signal, provided by generator B, shall also be at the lowest declared frequency of the receiver. Generator B shall be modulated to generate test signal number 3, either continuously or in the same time period as that used by generator A for test signal number 4. The content of the wanted and unwanted signals shall not be synchronised,

- The level of the wanted signal from generator A shall be adjusted to –101 dBm for a Type 3 device and to –91 dBm for a Type 2 device,

- The level of the unwanted signal from generator B shall be adjusted to –111 dBm for a Type 3 device and –107 dBm for a Type 2 device,

- The message measuring test set shall be monitored and the packet error rate (PER) observed,

- The test shall be carried out at the highest declared frequency of the receiver.

Required result

The PER shall not exceed 20 %.

#### Adjacent Channel selectivity

Purpose

The adjacent channel selectivity is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted signal which differs in frequency from the wanted signal by an amount equal to the adjacent channel separation for which the equipment is intended.

Method of measurement

Signal generator

A

Combiner

Receiver under

test

Message measuring test set

Signal generator

B

Figure 16 – Measurement arrangement with messages

The measurement procedure shall be as follows:

- Two generators A and B, shall be connected to the receiver via a combining network,

- The wanted signal, provided by signal generator A, shall be at the lowest declared frequency of the receiver and shall be modulated to generate test signal number 4,

- The unwanted signal, provided by generator B, shall be frequency modulated with a 400 Hz sine wave with a deviation of ±3 kHz. Generator B shall be at a frequency 50 kHz above that of the wanted signal,

- The level of the wanted signal from generator A shall be adjusted to a level of –101 dBm for a Type 3 device and to -91dBm for a Type 2 device.,

- The level of the unwanted signal from generator B shall be adjusted to –31 dBm for both type 2 and 3,

- The message measuring test set shall be monitored and the packet error rate observed,

- Repeat the above measurement with the unwanted signal 50 kHz below the wanted signal,

- The test shall be repeated [steps b) through g)] at the highest TDMA frequency declared by the manufacturer.

Required results

The PER shall not exceed 20 %.

#### Spurious Response Rejection

Purpose

The spurious response rejection is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted modulated signal at any other frequency, at which a response is obtained.

Manufacturers’ declarations

The manufacturer shall declare the following in order to calculate the “Limited Frequency Range” over which the initial part of the test will be performed:

- List of intermediate frequencies:	IF1, IF2, ...IFN in Hz

- Switching range of the receiver:	sr

- Frequency of the local oscillator at Channel 2 and at the lowest TDMA channel: fLOH, fLOL

Introduction to the Method of measurement

The initial evaluation of the unit shall be performed over the “Limited Frequency Range” and shall then be performed at the frequencies identified from this test and at “Specific Frequencies of Interest” (as defined below).

To determine the frequencies at which spurious responses can occur the following calculations shall be made:

- Calculation of the "Limited Frequency Range":

- The limits of the limited frequency range (LFRHI LFRLO ) are determined from the following calculations:

LFRHI = fLOH + (IF1 + IF2 +…+IFN + sr/2)

LFRLO = fLOL - (IF1 + IF2 +…+IFN + sr/2)

- Calculation of Specific Frequencies of Interest (SFI) outside the limited frequency range:

These are determined by the following calculations:

SFI1 = (K * fLOH) + IF1

SFI2 = (K * fLOL) - IF1

Where K is an integer from 2 to 4

Method of measurement over the Limited Frequency Range

Two methods are available for the measurements over the Limited Frequency Range, one based on SINAD measurements and the other based on PER measurements. Either method may be used, but in each case shall be followed by the Method of measurement at Identified Frequencies.

Signal generator

A

Combiner

Receiver under

test

Message measuring test set

Signal generator

B

Figure 17 – PER/BER or SINAD Measuring Equipment

Method of search over the "Limited Frequency Range" using SINAD measurement

- Two generators A and B shall be connected to the receiver via a combining network.

- The wanted signal, provided by generator A, shall be at 161,975 MHz* and shall be modulated with a 1 kHz sine wave at  2,4 kHz deviation.

- The unwanted signal, provided by generator B, shall be frequency modulated with a 400 Hz sine wave giving a deviation of  3 kHz.

- Initially, generator B (unwanted) shall be switched off (maintaining the output impedance).

- The signal level from generator A (wanted) shall be adjusted to – 101 dBm at the receiver.

- The SINAD value shall be noted (and shall be greater than 14 dB)

- Signal generator B shall be switched on and adjusted to – 27 dBm (- 37 dBm for the non-RATDMA receiver) at the receiver.

- The frequency of the unwanted signal shall be varied in steps of 5 kHz over the Limited Frequency Range (from LFRLO  to LFRHI).

- The frequency of any spurious response detected (by an decrease in SINAD of 3 dB or more) during the search shall be recorded for use in the next measurement.

Method of search over the "Limited Frequency Range" using PER or BER measurement

- Two generators A and B, shall be connected to the receiver via a combining network.

- The wanted signal, provided by generator A, shall be at 161,975 MHz* and shall be modulated to generate test signal number 3.

- The unwanted signal, provided by generator B, shall be frequency modulated with a 400 Hz sine wave giving a deviation of  3 kHz.

- Initially, generator B (unwanted) shall be switched off (maintaining the output impedance).

- The signal level from generator A (wanted) shall be adjusted to – 101 dBm at the receiver.

- The PER or BER shall be noted.

- Signal generator B shall be switched on and adjusted to – 27 dBm (- 37 dBm for the non-RATDMA receiver) at the receiver.

- The frequency of the unwanted signal shall be varied in steps of 5 kHz over the Limited Frequency Range (from LFRLO to LFRHI)

- The frequency of any spurious response detected (by an increase in either PER or BER) during the search shall be recorded for use in the next measurements.

- In the case where operation using a continuous packet stream is not possible a similar method may be used.

* If the manufacturer’s specified receiver frequencies do not include 161.975 MHz, one of the manufacturer’s specified receiver frequencies may be used as an alternative.

Method of measurement  (at Identified Frequencies)

- Two generators A and B, shall be connected to the receiver via a combining network.

- The wanted signal, provided by generator A, shall be at 161,975 MHz* and shall be modulated to generate test signal number 3.

- The unwanted signal, provided by generator B, shall be frequency modulated with a 400 Hz sine wave giving a deviation of  3 kHz. Generator B shall be at the frequency of that spurious response being considered.

- Initially, generator B (unwanted) shall be switched off (maintaining the output impedance).

- The signal level from generator A (wanted) shall be adjusted – 101 dBm at the receiver.

- Generator B shall be switched on, and the level of the unwanted signal set to  - 31 dBm ( - 41 dBm for the non-RATDMA receiver).

- For each frequency noted during the tests over the Limited Frequency Range and the Specific Frequencies of Interest (SFI1 and SFI2), transmit 200 packets to the EUT and note the PER.

* If the manufacturer’s specified receiver frequencies do not include 161.975 MHz, one of the manufacturer’s specified receiver frequencies may be used as an alternative.

Required results

At any frequency separated from the specified frequency of the receiver by two channels or more, the PER shall not exceed 20 %.

#### Inter-modulation response rejection

Purpose

The inter-modulation response rejection is the capability of the receiver to receive a wanted modulated signal, without exceeding a given degradation due to the presence of two close-spaced unwanted signals with a specific frequency relationship to the wanted signal frequency.

Method of test

Signal Generator A

A

Message

measuring

test set

Combiner

Receiver

under

test

Signal Generator B

B

Signal Generator C

Figure 18 – Measurement arrangement for inter-modulation

The measurement procedure shall be as follows:

- Three signal generators shall be connected to the receiver via a combining network.

- The wanted signal, provided by signal generator A, shall be at the specified frequency of the receiver and shall be modulated to generate test signal number 3.

- The unwanted signal from generator B shall be unmodulated.

- The unwanted signal from generator C shall be frequency modulated with a 400 Hz sine wave at a deviation of ± 3 kHz.

- The signal level from generator A (wanted) shall be set for – 101 dBm at the receiver input.

- The signal level from generators B and C shall be set for – 27 dBm (- 37 dBm for non-RATDMA) at the receiver input.

- The frequencies of generators A, B, C shall be set as per test number 1 of Table 19.

- The message measuring test set shall be monitored and the PER observed over 200 transmissions.

- Repeat the measurement with frequencies set as per test number 2 of Table 19.

Table 19 – Frequencies for inter-modulation test

| Test Number | Generator A Wanted AIS Signal | Generator B Unmodulated (+/- 500 kHz) | Generator C Modulated (+/- 1000 kHz) |
| --- | --- | --- | --- |
| 1 ( RATDMA Receiver) | 162,025 MHz | 161,525 MHz | 161,025 MHz |
| 1 (Non-RATDMA Receiver) | Highest operating frequency on which the EUT can operate | Highest operating frequency on which the EUT can operate – 500 kHz | Highest operating frequency on which the EUT can operate - 1000 kHz |
| 2 (both RATDMA and non-RATDMA Receiver) | Lowest operating frequency on which the EUT can operate | Lowest operating frequency on which the EUT can operate + 500 kHz | Lowest operating frequency on which the EUT can operate + 1000 kHz |

Required results

The PER shall not exceed 20 %.

#### Blocking or Desensitization

Purpose

Message

measuring

test set

Receiver

under

test

Combiner

Signal generator

A

Signal generator

B

Blocking is a measure of the capability of the receiver to receive a wanted modulated signal without exceeding a given degradation due to the presence of an unwanted input signal at any frequency other than those of the spurious responses or the adjacent channels.

Figure 19 – Measurement arrangement for blocking or desensitisation

Method of measurement

The measurement procedure shall be as follows:

- Two generators A and B, shall be connected to the receiver via a combining network as shown in Figure 19.

- The wanted signal, provided by signal generator A, shall be at the lowest operating frequency on which the EUT can transmit (or receive for a non-RATDMA receiver) according the manufacturers specification and be modulated with test signal number 3.

- The unwanted signal from generator B shall be unmodulated and tuned to the frequency of signal generator A plus 5 Mhz. Initially, signal generator B (unwanted signal) shall be switched off (maintaining the output impedance). The level of the wanted signal from generator A shall be adjusted to – 101 dBm at the receiver input.

- Generator B shall then be switched on, and the level of the unwanted signal set to – 15 dBm for the RATDMA receiver or – 25 dBm for the non-RATDMA receiver.

- 200 packets shall be transmitted and the PER recorded.

- Repeat the test [Steps a) to e)] with signal generator A tuned to the highest operating frequency on which the EUT can transmit (or receive for a non-RATDMA receiver) and signal generator B tuned to the frequency of signal generator A minus 5 MHz

Required results

The PER shall not exceed 20 %.

### Conducted Spurious Emissions at the antenna

#### Spurious Emissions from the Receiver

Purpose

Conducted spurious emissions to the antenna are any RF emissions generated in the receiver and conveyed to the antenna terminal.

Method of measurement

Conducted spurious emissions shall be measured as the power level of any frequency component to the antenna terminals of the receiver. The receiver antenna terminals are connected to a spectrum analyser or selective voltmeter having an input impedance of 50  and the receiver is switched on.

The measurement shall extend over the frequency range 9 kHz to 4 GHz.

Required results

The power of any spurious emission in the specified range at the antenna terminal shall not exceed – 57 dBm in the frequency range 9 kHz to 1 GHz and – 47 dBm in the frequency range 1 GHz to 4 GHz.

#### Spurious Emissions from the Transmitter

Purpose

Spurious emissions are emissions at frequencies other than those of the carrier and sidebands associated with normal modulation.

Method of measurement

Conducted spurious emissions shall be measured with the unmodulated transmitter connected to the artificial antenna.  The measurement shall be made over a frequency range from 9 kHz to 4 GHz, excluding the frequencies within +/- 62,5 kHz of the transmitting frequency.

Required results

The power of any spurious emission outside +/- 62,5 kHz of the transmitting freqeuncy shall not exceed – 36 dBm in the frequency range 9 kHz to 1 GHz and – 30 dBm in the frequency range 1 GHz to 4 GHz.

# Functional Tests

## Tests for Configuration Method

For all of the Functional Tests the setup for the Method of Measurement shall be as defined by manufacturer:

- Using standard configuration sentences via direct connection to an interface, or

- Using standard configuration sentences via VDL, or

- Using the manufacturer’s proprietary method.

### Configure Test Message 21 (Type 1, 2, 3)

Purpose

Purpose of this test is to ensure that Message 21 parameters can be entered into the EUT and are retained after power cycle.

Method of Measurement

Set-up the standard test environment

Configure the EUT with the following parameters for transmission of Message 21:

- MMSI number: 991234567,

- Type of AtoN: “20” – Cardinal Mark North,

- Name of AtoN: “TEST FLOATING ATON”,

- Position accuracy: to accuracy of EPFS,

- Assigned position (longitude and latitude): “Within Off-position threshold of current EPFS position”,

- Dimension/Reference for position: “A=B=C=D=5”,

- Type of EPFS: Enter EUT’s EPFS type (e.g. “1” for GPS),

- Off Position Threshold: 200m,

- Power Level set to High,

- Channel 1 set to channel 2087; if receiver supported, set Channel 1 receiver to same,

- Channel 2 set to channel 2088; if receiver supported, set Channel 2 receiver to same,

- Virtual AtoN Flag set to 0 = default = real AtoN at indicated position,

- Set AtoN status default (00000000),

- Off Position Behaviour set to “Maintain Current”,

- Set UTC Lost Behaviour as per manufacturers declaration.

Standard Configuration Sentences Via configuration port:

The Message 21 content is configured using the AID, ACF and ACE sentence combination.

Standard Configuration Sentences Via VDL:

The Message 21 content is configured via VDL using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier and binary data.

- Read configuration from EUT.

- Remove power from the EUT for five minutes.

Switch on the EUT.

Read configuration from EUT.

Required Results

Verify that configuration is:

- Accepted by EUT and that the parameters have been correctly set.

- Retained.

### Schedule Mode A FATDMA Message 21 (Types 1, 2, 3) (single report, alternating channel operation)

Purpose

Test that the AIS AtoN Station operates in accordance with configured reporting schedule (5.2.4.1refers).

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1.

Configure reporting of Message 21 to have the following parameters:

- Start on channel B,

- Start slot: 512,

- Reporting interval: 3 minutes,

- Frame for the first transmission in every UTC hour: UTC minute: 1,

- Start the EUT two minutes ahead of a schedule transmission,

- Run the test over the hour and day boundary.

Standard Configuration Sentences Via configuration port:

The Schedule for Mode A FATDMA transmission is configured using the AAR sentence.

Standard Configuration Sentences Via VDL:

The schedule for Mode A FATDMA transmissions via VDL is configured using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data.

Required results

Verify that the:

- EUT transmits Test Message 21 in the configured slots on both channels;

- EUT starts transmission in the correct UTC frames and alternates channels at the reporting interval within one reporting interval (3 minutes in this case), and should not wait until UTC minute 1. (The channel A transmissions shall occur in minutes 4, 10, 16, 22, 28, 34, 40, 46, 52 or 58 with an increment of 6 minutes, The channel B transmissions shall occur in minutes 1, 7, 13, etc with an increment of 6 minutes.)

- Reporting behaviour is consistent through the hour and day boundaries.

- Transmitted data is correct.

### Schedule Mode B FATDMA Message 21 (Types 1, 2, 3) (dual report, dual channel operation)

Purpose

Test that the AIS AtoN Station operates in accordance with configured reporting schedule Section 5.2.4.2.1 and transmits correct data.

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1.

Configure reporting of Message 21 to have the following parameters:

- Start Channel 1: Start slot 512,

- Channel 2: Start slot: 612,

- Reporting interval: 3 minutes,

- Frame for the first transmission in every UTC hour: UTC minute 2,

- Start the EUT two minutes ahead of a schedule transmission,

- Run the test over the hour and day boundary.

Standard Configuration Sentences Via configuration port:

The Schedule for Mode B FATDMA transmission is configured using the AAR sentence.

Standard Configuration Sentences Via VDL:

The schedule for Mode B FATDMA transmissions via VDL is configured using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data.

Required results

Verify that the:

- EUT transmits Test Message 21 in the configured slots on both channels,

- EUT starts transmission in the correct UTC frame and continues with the correct increment within one reporting interval and should not wait until UTC minute 2,

- Reporting behaviour is consistent through the hour and day boundaries,

- Transmitted data is correct.

### Schedule Mode C FATDMA Message 21 (Types 1 and 2) (Single report, single channel operation)

Purpose

Test that the AIS AtoN Station operate in accordance with configured reporting.

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1.

Configure reporting of Message 21 to have the following parameters:

- Transmit channel: A or B,

- Start slot: 512,

- Reporting interval: 3 minutes,

- Frame for the first transmission in every UTC hour: UTC minute: 1,

- Start the EUT two minutes ahead of a schedule transmission,

- Run the test over the hour and day boundary.

Standard Configuration Sentences Via configuration port:

The Schedule for Mode C FATDMA transmission is configured using the AAR sentence.

Standard Configuration Sentences Via VDL:

The schedule for Mode C FATDMA transmissions via VDL is configured using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data.

Required results

Verify that the:

- EUT transmits Test Message 21in the configured slots on the designated transmit channel,

- EUT starts transmission in the correct UTC frame on the designated transmit channel at the reporting interval within one reporting interval and should not wait until UTC minute 1,

- Reporting behaviour is consistent through the hour and day boundaries,

- Transmitted data is correct.

### Schedule Mode A RATDMA Message 21 (single report, alternating channel operation)

Purpose

The purpose of this test is to ensure that the EUT can be configured to operate in accordance with Section 5.2.4.2, ensuring the slot selection is random within the one minute interval and that the slot reuse algorithm is properly implemented.

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 with a VDL loading of 10 %.

Configure reporting of Message 21 to have the following parameters:

- Reporting interval: 3 minutes,

- Frame for the first transmission in every UTC hour: UTC minute 2.

Repeat Test

- Apply an 80% load to the VDL.

Repeat the Test

Standard Configuration Sentences Via configuration port:

The Schedule for Mode A RATDMA transmission is configured using the AAR sentence.

Standard Configuration Sentences Via VDL:

The  schedule for Mode A RATDMA transmissions via VDL is configured using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data.

Required results

Verify that the EUT transmits Test Message 21:

- Using RATDMA so that the slot selection is random within the one minute interval, and alternates the transmission channel between successive reports,

- With the correct reporting intervals,

- With the correct data.

Verify that the EUT selects its slots randomly.

Verify that the EUT applies the slot reuse algorithm as defined in ITU-R M.1371.

### Schedule Mode B RATDMA Message 21 (dual report, dual channel operation)

Purpose

Purpose of this test is to ensure that the AIS AtoN Station can be configured to operate in accordance with Section 5.2.4.2.

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 with a VDL loading of 10 %.

Configure reporting of Message 21 to have the following parameters:

- Reporting interval: 3 minutes,

- Frame for the first transmission in every UTC hour: UTC minute 2.

Standard Configuration Sentences Via configuration port:

The Schedule for Mode B RATDMA transmission is configured using the AAR sentence.

Standard Configuration Sentences Via VDL:

The schedule for Mode B RATDMA transmissions via VDL is configured using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data.

Required results

Verify that the EUT transmits Test Message 21:

- Using RATDMA so that the slot selection is random within the one minute interval, with dual reports on both channels,

- Sending in correct intervals,

- With correct transmitted data.

### Schedule Mode C RATDMA Message 21 (single channel operation)

Purpose

Purpose of this test is to ensure that the AIS AtoN Station can be configured to operate in accordance with Section 5.2.4.2.

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 with a VDL loading of 10 %.

Configure reporting of Message 21 to have the following parameters:

- Reporting interval: 3 minutes,

- Frame for the first transmission in every UTC hour: UTC minute 2.

Standard Configuration Sentences Via configuration port:

The Schedule for Mode C RATDMA transmission is configured using the AAR sentence.

Standard Configuration Sentences Via VDL:

The schedule for Mode C RATDMA transmissions via VDL is configured using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data.

Required results

Verify that the EUT transmits Test Message 21:

- Using RATDMA so that the slot selection is random within the one minute interval with single reports on a single channel,

- Sending in correct intervals,

- With correct transmitted data.

### Addressed Binary Data Message 6

Purpose

Test of Message 6 operation of the EUT using the implemented Access methods.

Method of Measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 and transmission schedule for Message 21 as defined in 8.1.2 with an “intended recipient” MMSI.

- Configure the EUT as defined by manufacturer’s documentation for transmission of a scheduled addressed binary data Message 6 with test binary data consisting of the bit pattern Hex “7E 3B 3C 3E 7E” forming a message by setting the parameters for the following operation modes, where implemented.

- FATDMA (See 8.1.2 for Mode A setup; 8.1.3 for Mode B setup; 8.1.4 for Mode C setup)

- RATDMA (See 8.1.5 for Mode A setup; 8.1.6 for Mode B setup; 8.1.7 for Mode C setup),

- CSTDMA:

- Time (hour, minute),

- Channel(s),

- Reporting Interval.

- Repeat the test for FATDMA, RATDMA, and CSTDMA but without an acknowledgement from the intended recipient.

- If possible to use externally generated data, repeat the test exceeding the maximum length of Message 6.

- Repeat the test for the maximum length of Message 6 by repeating the bit pattern Hex “7E 3B 3C 3E 7E” sequence in the binary data field.

Required Results

Verify that the:

- Message sent by the EUT conforms to message content, access method, channel, slot number and reporting interval.

- EUT does not retransmit for FATDMA but does retransmit for RATDMA and CSTDMA according to the configuration.

- Message is not sent.

- Message is sent with the correct content.

- EUT continues transmitting Message 21 in all cases.

CSTDMA access of message 6 shall comply with IEC 62287-1 with regard of VDL access and message length.

### Unscheduled Transmission

Purpose

Unscheduled transmissions are those transmissions that are not planned, and the competent authority wishes the AtoN Station to broadcast autonomously such as an unexpected ALARM condition.  The VDL access method for these message types is as defined by manufacturer.   This test will verify the AtoN operation when such a message is input.

Method of Measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 and transmission schedule for Message 21 as defined in 8.1.2 with an “intended recipient” MMSI.

Introduce a transmission of an unscheduled Binary Message as defined by manufacturer’s documentation using the access modes declared by the manufacturer.

For an Addressed Message repeat the test, but without an acknowledgment from the intended recipient.

Standard IEC 61162 Sentences:

An unscheduled message using standard format would be ABM, ABK or BBM.

Required Results

Check that the EUT continues to transmit Message 21 in all cases.

Check the message transmitted by the EUT conforms to message content, access method.

Check that the EUT retransmits as configured.

### Test Message 8

Purpose

The purpose of this test is to verify that Message 8 can be entered into the EUT.

Method of Measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 and transmission schedule for Message 21 as defined in 8.1.2.

Configure the EUT as defined by manufacturer’s documentation for transmission of a scheduled addressed binary data Message 8 with test binary data consisting of the bit pattern Hex “7E 3B 3C 3E 7E” forming a message by setting the parameters for the following operation modes, where implemented.

- FATDMA (See 8.1.2 for Mode A setup; 8.1.3 for Mode B setup; 8.1.4 for Mode C setup).

- RATDMA (See 8.1.5 for Mode A setup; 8.1.6 for Mode B setup; 8.1.7 for Mode C setup).

- CSTDMA:

- Time (hour, minute),

- Channel(s),

- Reporting Interval.

- If possible to use externally generated data, repeat the test exceeding the maximum length of Message 8.

- Repeat the test for the maximum length of Message 8 by repeating the bit pattern Hex “7E 3B 3C 3E 7E” sequence in the binary data field.

Required Results

Verify that the:

- Message sent by the EUT conforms to message content, access method, channel, slot number and reporting interval

- Message is not sent

- Message is sent with the correct content

- EUT continues transmitting Message 21 in all cases.

- CSTDMA access of message 6 shall comply with IEC 62287-1 with regard to VDL access and message length.

### AIS AtoN Configuration Messages 12

Repeat tests 8.1.8,and 8.1.9 for Message 12.

### AIS AtoN Configuration Messages 14

Repeat tests 8.1.10 for Message 14.

## Tests for Synchronisation accuracy

### Synchronisation Error

Purpose

To measure the synchronisation error of the EUT.

Method of measurement

Set-up standard test environment and operate EUT in normal mode.

Set the EUT reporting interval to 1 minute for message 21 and all other implemented messages.

Operate the EUT in all implemented synchronisation modes:

- EUT using UTC direct synchronisation.

- EUT using UTC indirect synchronisation.

- EUT using Semaphore synchronisation.

Record VDL messages and measure the time between the transmission patterns as defined by ITU-R 1371 and the actual transmission made by the EUT. The transmission timing shall be measured and referenced to the beginning of the start of a transmission packet (start flag) according to ITU-R 1371.

Required results

The synchronisation error with its additive jitter shall not exceed

- ± 104 s using UTC direct synchronisation.

- ± 312 s using UTC indirect synchronisation.

- ± 312 s referenced to the Semaphore synchronisation.

### Synchronisation test without UTC Direct

Purpose

The purpose of this test is to verify that the EUT can synchronise indirectly.

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1. Choose test conditions in a way that EUT receives messages from:

- Base station direct (semaphore) synchronisation and no stations with direct synchronisation. Disable internal synchronisation source.

- Mobile direct (semaphore) synchronisation and no stations with direct synchronisation. Disable internal synchronisation source.

- Mobile indirect (UTC) synchronisation and no stations with direct synchronisation. Disable internal synchronisation source.

- Enable internal synchronisation source.

Record transmitted messages.

Required results

Verify that the EUT transmits according to its configuration in each case.

## Tests for EPFS

### Position Source

Purpose

The purpose of this test is to verify that the position source correctly populates the fields in Message 21.

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 and transmission schedule for Message 21 as defined in 8.1.2.

Record the EUT transmissions.

Repeat the test with a surveyed position.

Required result

Verify that the position and time stamp fields are valid.

Verify that the EUT has the correct parameter settings for “Type of electronic position fixing device” and “RAIM-flag”.

### Invalid position

Purpose

The purpose of this test is to verify that the EUT responds correctly when the EPFS outputs an invalid position.

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 and transmission schedule for Message 21 as defined in 8.1.2.

Perform the following tests where implemented by the manufacturer:

- Configure the EUT to Transmit with LAT/LON not known (91/181) and Time Stamp Invalid (63).

Prevent the EPFS receiver from generating position fixes.

- Configure the EUT to Transmit with last known position and Time Stamp Invalid (63).

- Configure the EUT to Stop Transmitting.

Required result

- Verify that when the EPFS position fixes are inhibited, the EUT transmits message 21 with the parameters latitude and longitude set to “not available” and the time stamp is set to “63”.

- Verify that when the EPFS position fixes are inhibited, the EUT transmits message 21 with the parameters latitude and longitude set to last known position and the time stamp is set to “63”.

- Verify that when the EPFS position fixes are inhibited, the EUT stops transmitting.

### Off-position monitor

Purpose

The purpose of this test is to verify that the EUT responds correctly when it is off position.

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 and transmission schedule for Message 21 as defined in 8.1.2.

- Configure the EUT to be on-position with off position behaviour set to maintain current broadcast schedule.

- After verification of on-position indicator in message 21, the EUT shall be moved off-position.

- Move the EUT to be on-position.

- If implemented, configure the EUT with off position behaviour set to a new reporting interval.

- After verification of on-position indicator in message 21, the EUT shall be moved off-position.

Required result

- Verify that message 21 has the Off-position Indicator field set to “0”.

- Verify that message 21 has the Off-position Indicator field set to “1” in its next message. And that the original reporting schedule has not changed.

- Verify that message 21 has the Off-position Indicator field set to “0” in its next message.

- Verify that message 21 has the Off-position Indicator field set to “1” in its next message. And that the original reporting schedule has changed to the new reporting interval.

- Verify that message 21 has the Off-position Indicator field set to “0” in its next message. And the reporting interval returns to the original reporting schedule.

## Additional Messages

### Receive addressed message

Purpose

The purpose of this test is to verify that the EUT correctly receives and, if so configured, processes an addressed message.

Method of measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 and transmission schedule for Message 21 as defined in 8.1.2.

- Apply an addressed binary message (message 6; EUT as destination) to the VDL.

- Apply an addressed binary message (message 6; other station as destination) to the VDL.

Record received messages and frame structure.

Required results

- EUT receives and processes the message in accordance with the manufacturer’s specification.

- EUT does not process the received message.

## Additional Functionality

Tests for additional functionality as implemented by the manufacturer.

### Test for Configuration of the receiver turn-on times

Purpose

Purpose of this test is to ensure that the operational time period for the receivers can be configured using the configuration port of the EUT or the appropriate VDL message.

Method of Measurement

Set-up standard test environment and operate EUT in normal mode.

Configure the receiver turn-on times of the EUT with the following parameters:

- MMSI of the AtoN Station,

- Receiver On or Interval,

- Time of first turn on period,

- Duration of Receiver wake up,

- Interval between receiver activation.

Standard Configuration Sentences Via configuration port:

The Receiver Turn-On times are configured using the ARW sentence.

Standard Configuration Sentences Via VDL:

The Receiver Turn-On times are configured via VDL using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data

Using the implemented methods (one or both):

- Enter the appropriate data with the parameter “Receiver On or Interval” of on.

- Enter the appropriate data with a definition of a turn on interval.

- Query the ARW Configuration of the receiver turn-on times via configuration port using the QARW sentence or other means provided by the manufacturer.

- Query the ARW Configuration of the receiver turn-on times via VDL and define a FATDMA slot for the VDL replay.

Required Results

- Verify that the EUT receiver is turned on all the time.

- Verify that the EUT receiver is turned on during the defined time period and interval.

- Verify that the EUT returns on a query with the appropriate message content via PI using ARW sentence.

- Verify that the EUT returns on a query via VDL with the appropriate VDL message on the assigned slot and channel using the appropriate Application Identifier and binary data.

### Test for Configure Proprietary AtoN Control

Purpose

Purpose of this test is to ensure that the payload of this sentence is used to control the AtoN. The payload can be entered into the EUT using the configuration port of the EUT or the appropriate VDL message.

Method of Measurement

Set-up standard test environment and operate EUT in normal mode.

Configure the Proprietary AtoN Control function of the EUT with the following parameters:

- MMSI of the AtoN Station,

- Payload for Proprietary AtoN Control.

Standard Configuration Sentences Via configuration port:

The Proprietary AtoN Control data is configured using the MCR sentence.

Standard Configuration Sentences Via VDL:

The Proprietary AtoN Control data is configured via VDL using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data.

Using the implemented methods (one or both):

- Enter the appropriate Proprietary AtoN Control data.

- Query the Proprietary AtoN Control data via configuration port using the QMRC sentence or other means provided by the manufacturer.

- Query the Proprietary AtoN Control data via VDL and define a FATDMA slot for the VDL replay.

Required Results

- Verify that the EUT acts upon the received Proprietary AtoN Control data.

- Verify that the EUT returns on a query with the appropriate message content via PI using MRC PI sentence.

- Verify that the EUT returns on a query via VDL with the appropriate VDL message on the assigned slot and channel using the appropriate Application Identifier and binary data.

### Test for Configuration of Payload Re-broadcast

### Purpose

Purpose of this test is to ensure that the EUT can be commanded to rebroadcast the payload or to define a new message for autonomous, continuous transmission. The payload or new message type can be entered into the EUT using the configuration port of the EUT or the appropriate VDL message.

If standard sentences are used, the AAR configuration with message type/id for a specific MPR must precede the MPR to identify it as autonomous continuous transmission. If it is a single transmission, this payload will be broadcast using the slots reserved by the AAR with message id/type = 0, otherwise it will use the schedule defined by the AAR for this message id/type.

Method of Measurement

Set-up standard test environment and operate EUT in normal mode.

Configure the Payload Re-broadcast function of the EUT with the following parameters:

- Message Type,

- Message Identifier,

- Total Number of Sentences,

- Sequence Number,

- Payload (encapsulated data, as defined by IEC 61162).

Standard Configuration Sentences Via configuration port:

The Payload Re-broadcast data is configured using the MPR sentence.

Standard Configuration Sentences Via VDL:

The Payload Re-broadcast data is configured via VDL using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data.

Using the implemented methods (one or both):

- Enter the appropriate AAR data to configure broadcast rates for AtoN Station Messages for the following Payload Re-broadcast.

- Enter the appropriate Payload Re-broadcast data.

Required Results

Verify that the EUT re-broadcasts the appropriate VDL message with the correct data content.

### Test for Forced Broadcast

Purpose

Purpose of this test is to ensure that the EUT can be forced to broadcast a specified VDL message via PI or VDL.

Method of Measurement

Set-up standard test environment and operate EUT in normal mode.

Enter the Forced Broadcast data to the EUT with the following parameters:

- Message Type,

- Message Identifier,

- VDL channel for message transmission,

- Time and slot message transmission,

- Number of consecutive slots for message transmission.

Standard Configuration Sentences Via configuration port:

The Forced Broadcast data is configured using the MPR sentence.

Standard Configuration Sentences Via VDL:

The  Forced Broadcast data is configured via VDL using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data.

Using the implemented methods (one or both) enter the appropriate Forced Broadcast data to the EUT.

Required Results

Verify that the EUT transmits the requested VDL message at the defined time and slot.

### Test for Version Information

Purpose

Purpose of this test is to ensure that the EUT can provide version information.

Method of Measurement

Set-up standard test environment and operate EUT in normal mode.

- Enter the query for Version Information to the EUT using the manufacturer implemented methods.

Standard Configuration Sentences Via configuration port:

The Version Information is queried using the QVER sentence and the response is provided using VER.

Standard Configuration Sentences Via VDL:

The Version Information is queried via VDL using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data

Required Results

- Verify that the EUT provides with the requested Version Information.

### Test for AFC – AtoN Function ID capability

### Purpose

Purpose of this test is to ensure that the EUT can provide a list of supported functionality.

### Method of Measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 and transmission schedule for Message 21 as defined in 8.1.2.

Enter the query for the function supported.

Standard Configuration Sentences Via configuration port:

The list of supported functions is queried using the QAFC sentence and the response is provided using AFC.

Standard Configuration Sentences Via VDL:

The list of supported functions is queried via VDL using Message 25 or Message 6 with the appropriate Application Identifier/Function Identifier, and binary data.

Required Results

Verify that the EUT provides the list of functions that are supported by the AtoN Station.

#### Test for assigning an encryption key for VDL configuration

Purpose

Purpose of this test is to ensure that the encryption key for VDL configuration can be entered into the EUT using the configuration port or the appropriate VDL message.

In order to reset this key via the PI the user must know the current encryption key.  The initial encryption key, when shipped from the mfg, will be all 0’s.

Method of Measurement

Set-up standard test environment and operate EUT in normal mode.

Configure encryption key of the EUT with the following parameters:

- MMSI of the AtoN Station,

- Current AES encryption key,

- New AES encryption key.

Via configuration port:

Enter encryption key via configuration port using AKE PI sentence or any other means provided by the manufacturer.  The sentence used on the configuration port allows for the entire 128 bit encryption key to be entered.

Via VDL:

Enter encryption key via VDL using Message 25 or Message 6 with the appropriate Application Identifier and binary data. The VDL message only allows the least significant 56 bits to be modified.

Using the implemented methods (one or both):

- Enter the appropriate data with the correct MMSI and correct current ASE encryption key.

- Enter the appropriate data with the correct MMSI and false current ASE encryption key.

- Query the ASE encryption key via configuration port using the QASE sentence or other means provided by the manufacturer.

Query via the VDL for the encryption key is not allowed.

Required Results

- Verify that the new encryption key is not accepted by changing the transmission behaviour of the EUT using an encrypted VDL configuration message.

- Verify that the new encryption key is accepted by changing the transmission behaviour of the EUT using an encrypted VDL configuration message.

Verify that the EUT returns on a query with the appropriate message content via PI using ASE sentence.

### Test for VDL configuration using Chaining

### Purpose

Purpose of this test is to verify that, if chaining is implemented, the AtoN Station supports receiving information from a base station via intermediate AtoN Stations and then transmits the response back through the intermediate AtoN Stations to the base station.

### Method of Measurement

Set-up the standard test environment and use the configuration as defined in 8.1.1 and transmission schedule for Message 21 as defined in 8.1.2.  Introduce a base station that will be issuing the configuration VDL sentences and at least two other AtoN Stations transmitting Message 21, one of which is the EUT.

Standard Configuration Sentences Via configuration port:

Using the AID sentence a chain is established by defining the parent and all known children within each AtoN station in the link.

Standard Configuration Sentences Via VDL:

A chain is established by defining the parent and all known children within each AtoN station in the link via VDL using Message 25 or Message 6 with the appropriate Application/Function Identifier and binary data.

Using the implemented method(s):

Establish a chain by configuring each AtoN Station with the MMSI numbers for the parent and all child AtoN Stations. (e.g. the first AtoN in the chain would have the Base Station as its “parent” and all other AtoN stations in the chain would be “child” AtoN stations, the next AtoN in the chain would have that first AtoN station as the parent and all other AtoN stations in the chain would be “child” AtoN stations, and so on).

- Query the AtoN stations to verify the chain has been correctly established.

- Introduce via the Base Station a Message 25 with configuration information addressed to the last AtoN Station in the chain.

Query the last AtoN Station to verify configuration.

Required Results

- Verify that the chain is established.

- Verify that the EUT functions correctly within the chain and at the end of the chain, for transferring, receiving and initiating messages.

## Test for BIIT

Purpose

Purpose of this test is to prove the correct response by the EUT to its BIIT.

Method of measurement

Set-up standard test environment and operate EUT in normal mode.

- Disconnect the antenna from the EUT.

- Apply fault to Channel 1 receiver.

- Apply fault to Channel 2 receiver.

- Disable augmentation system, if fitted.

Required results

- The EUT shall cease transmissions.

- RATDMA and CSTDMA transmissions shall cease on Channel 1.

- RATDMA and CSTDMA transmissions shall cease on Channel 2.

- EUT shall continue to operate.

### Transmitter shutdown procedure

Purpose

The purpose of this test is to verify that the transmitter has an automatic shutdown.

Method of measurement

Review the manufacturer’s declaration.

Required results

The manufacturer shall provide a declaration in the documentation that states the EUT will function as required.

## Tests for Power Supply

### Average Power Consumption

Purpose

Purpose of this test is to ensure that the power consumption of the AIS AtoN Station is as stated in the manufacturer’s documentation.

Method of measurement

Set-up standard test environment and operate EUT in normal mode.

- Configure reporting of Message 21 to have the following parameters:

- Transmit power level: 12,5W, or the manufacturer’s declared level,

- Channel 1 slots: 512 and 513,

- Channel 2 slots: 612 and 613,

- Reporting interval: 3 minutes,

- The test shall be run for 30 minutes with 10 full duty cycles to measure the average power consumption.

- Optionally, repeat the test for RATDMA for the same transmit power and reporting interval.

Required results

Verify that for 10 full duty cycles, the average power consumption of the EUT does not exceed 110 % of the value stated in the manufacturer’s documentation.

## Environmental tests

Tests shall be done in accordance with IEC60945, ‘Durability and resistance to environmental conditions’; Protected or Exposed, or as defined by manufacturer.

## Other tests

### Quality assurance

The manufacturer shall declare the quality assurance standard to which the EUT is manufactured.

### Additional features

The manufacturer shall declare any additional features of the EUT.  These features are not tested in accordance with this document. The manufacturer declaration shall confirm that additional features, including position accuracy augmentation, do not adversely affect Message 21 transmissions.

### Manual

The manual shall include information concerning:

- External connectors, if applicable,

- Correct installation of the unit and antennae,

- Configuration,

- Power consumption,

- Firmware upgrades, if applicable

- Configuration interface, including hardware and electrical details.

### Marking and identification

Verify label complies with 5.4.3 Marking and Identification.

- - Bits for Regional Use

Informative

  - Message 21 – Regional Bits

As indicated in IALA Recommendation A-126 in Message 21, 8 bits are reserved for Regional Use.  The diagram below represents the recommended use of these bits.

These bits shall be employed as follows:

- The first three bits shall be use to define a Page ID. The Page ID can range from 0 to 7, allowing 8 pages. The first page is not used for the Regional/International application and is defined as the default “not used” condition in Recommendation ITU-R M.1371.  Page 7 (binary 111) is defined in this liaison note. Pages 0 to 6 are reserved for future use. The future use is envisaged as being for monitoring of AtoN parameters such as voltages, currents, temperatures, etc.

- Page 7 shall be implemented in all types of AIS AtoN Stations. The final 5 data bits are defined as in the diagram above.

Explanatory notes are:

- Manufacturer’s default setting for the Regional Bits of Message 21 shall be all zeros.

- Health flag alarm shall be set to 1 to indicate a fault in or failure of the AtoN system or AIS AtoN station, at this location. Further indication of the fault or failure detail can be achieved by use of additional pages within the Regional Bits, or addressed binary Message 6.

- By using only page 7 there is no need to toggle through the messages, only message id 7 has to be read thus allowing an immediate filtering.

One bit is used for alerting the competent authority that there is a problem at the AIS AtoN Station. This allows a competent authority to avoid using Message 6, if there is pressure on VDL slots, while still receiving some monitoring information every time Message 21 is sent by the AIS AtoN Station.

For further detail on the use of the remaining pages refer to IALA documentation.

- – Presentation Interface Sentences
Informative

  - [note – this annex is provided as a separate document at this time for formatting reasons]

Switching range corresponds to the frequency range over which the receiver can be tuned.

This may be a VCO, crystal, sampling clock, BFO, Numerically Controlled Oscillator depending on the design of the equipment.

These 8 bits are called “AtoN status” in ITU-R M.1371-3