Input Paper to IALA e-Nav 02 (March 2007)

Submitted by The Nautical Institute

# S-Mode of operation for e-navigation systems

Summary:

The Nautical Institute has proposed a standardised and simplified default interface for integrated navigation displays. Benefits of such a system, if it were adopted, would be a common default display that all mariners and pilots could be trained in the use of, while still allowing industry to benefit from developing through innovation. Key challenges for such a concept would be to develop such a display, test it for effectiveness, and have it accepted internationally.

The IALA e-Nav 02 Committee are invited to consider the proposal and to assess its role in the concept of e-navigation.

Standardisation vs. Innovation

It is recognised that there is a vital need to embrace new technology and for manufacturers to be able to innovate with the expectation that if they get it right, they will be rewarded with sales of their products. Recent innovations include the Chart Radar; New Technology (NT) non-magnetron radar; and ECDIS; just to name a few. Significant innovations from the past have included electronic position fixing systems, the gyrocompass and even the chronometer in its day, to name but a few. Standardisation of navigation equipment on the bridge however would simplify training on a world-wide basis and ensure that pilots and mariners could instantly be familiar with the operation of such equipment when joining a vessel and therefore be better placed to concentrate on making good decisions (note IMO MSC circ. 1091). The question therefore is how to balance these two objectives.

## Background

The Nautical Institute has held a series of international conferences on ‘IBS and the Human Element’ and we were urged, often and passionately, by sailing mariners to keep things simple, make best use of standardisation and to put a high priority on training. One outcome from these conferences was the desire for an IMO approved default setting that could be triggered by a single button. Although the mariner/display interface is only one aspect of e-navigation, it is an important one. This general concept has now evolved into what we are arbitrarily calling ‘S-Mode’ of operation and we are currently researching the validity of such a initiative.

The concept of S-Mode builds on the concept of a ‘default setting’ by being a ‘default mode’. This mode is made possible by the increasing use of Multi-Function Displays (MFDs) where Radar, charts, electronic position systems etc are inputs that can be arranged or re-arranged in any form on a display.

## S-Mode

S-Mode would require all navigation displays, regardless of manufacture, to have a clearly identified button, that when pressed brings the display into a standard format with a standard menu/control system, standard interface (i.e. keyboard/joystick etc..) and basic features. For example there may have to be tactical display, for near time decisions (collision, and hazard avoidance) and a complementary display for voyage and passage planning. At the press of a button the tactical display might revert to a 12 mile range Radar view with targets showing relative vectors and perhaps hazardous depth contours shown (from vector chart data). This view would be standardised and familiar to all mariners and pilots, and then could be manipulated through a standard menu system for a limited, although adequate, functionality. The advantage to this would be that:

- Training for S-Mode could be standardised throughout the world

- Any mariner or pilot would be comfortable to revert to S-Mode and be competent in using the systems layout and functionality, regardless of manufacturer.

- Masters or companies could impose S-Mode only uses by crews until such time that they have proven they are competent to use further functionality that may have been provided by individual manufactures.

- S-Mode could also be used at times when the bridge team comprised multiple persons who needed to share a common display for decision making.

With the performance of S-Mode secured and strictly governed by the IMO, manufacturers would be able to develop further functionality that they could market to shipowners as ‘value added feature’. If, in time, these innovative features proved to be popular and effective, they could then be brought into ‘S-Mode’ in a controlled way by the IMO.

At a basic level, some ships might opt to only have S-Mode functionality installed, but there may be other vessels, which by the nature of their trade or quality of their training can take advantage of new and innovative features that would be developed by the industry.

## Development

In order to develop such a mode The Nautical Institute considers it will be necessary for international mariners to define the user needs, then to work with industry to create mock-up variations, and then to test these variations using simulation with realistic bridge teams. After though testing and evaluation, the system would be put forward to IMO for adoption.

This is a fledgling idea and the Nautical Institute is currently continuing its research into the concept validity. IALA e-nav 02 is asked to consider any possible role that S-Mode might have with e-navigation and provide feedback as appropriate.