e-NAV 2 / 06 / 07

Evaluation of integrated GNSS and INS for marine navigation

## Introduction

This paper is a summary of a study carried out for the General Lighthouse Authorities of the UK & Ireland by the IESSG, University of Nottingham.

The objective of the project was to provide an assessment of the capabilities of GNSS-INS to assist with future rationalisation of marine aids to navigation, taking into consideration the e-navigation concept. It examined the price and performance of Inertial Measurement Units (IMU) and considered whether a proliferation of ship-borne inertial systems could change the demands placed on GLA services.

The Committee is invited to note the paper for information and consider whether the findings have any consequences for IALA Policy.

# INS for Commercial Marine Applications

Inertial navigation systems are not currently used in most commercial marine applications. This has been because of their historically high cost and the lack of any convincing cost benefit analyses for these applications. Additionally, there has not been a high profile accident where a subsequent investigation showed that it would have been prevented solely by the mandatory use of INS; therefore there has been no pressure for the introduction of INS.

A fundamental advantage of an inertial sensor is that it is ship based and independent of external systems. In this respect it is similar to radar - they both give relative positional data.

Integration of an inertial sensor into a commercial marine navigation system has the potential to produce the following benefits:

- Improved navigational solution, with positional and velocity estimates available at a rapid update rate

- Automatic warnings that other sensors, including GNSS, have failed or are giving reduced navigational accuracy

- Maintain an acceptable navigation solution in the event of a failure or degradation of other sensors

This study concentrated on quantifying the accuracy of navigation in the event of a GNSS failure, using different grades of IMU (‘Navigation’, ‘Tactical’ and ‘Low Cost’) and various (simulated) GNSS failure periods.

## Results

It was clear from trials that none of the options considered was suitable to maintain navigation accuracy during a major outage of GNSS. However, the following observations could be made:

- For ocean areas, both navigation and tactical-grade IMUs will give protection for appreciable outages (over 1 hour and 15 minutes respectively).

- For coastal areas, 15 minutes of good accuracy would be maintained using a navigation-grade IMU, giving good time for reversion to traditional navigational techniques, in particular, using radar for absolute position fixing and increasing the frequency of positioning using visual sights.

- For the harbour approach phase, accuracy to 10 metres could be obtained for 3.5 minutes with a navigation-grade IMU-based system.  This would give time for an experienced user to reference the radar to conspicuous ground fixed targets, provided these were available.

- For docking and where sub-10 metre accuracy is required, INS coupled with GNSS does not appear to offer any benefit in the event of a GNSS failure.

Unfortunately, the price/performance ratios of the available IMUs make it unlikely that they could become an acceptable solution. Future MEMS-based INS with similar performance and lower price could prove to be a useful addition to a ship’s navigation system.

## Conclusions

- Although INS may offer tangible advantages to improve a ship’s navigation it is unlikely that its adoption alone would lead to a reduction in the need for conventional AtoNs.

- Because the duration of a GNSS outage is indefinable, INS cannot be considered as a primary backup to GNSS

- Its main safety advantage is in allowing accurate navigation for a defined time after GNSS failure to enable the navigation of the ship to revert to alternative techniques in an orderly manner.

- When GNSS is available it is speculated that the extra motion information obtainable from INS could prove useful to enhance the performance of autopilots, thereby reducing fuel consumption.

- The INS units considered in the study do not appear to provide any useful reversionary capability when used in applications needing accuracies of much better than 10 metres.

- An INS coupled with GNSS and other existing shipborne sensors, such as a gyrocompass, may give superior performance. This is discussed further below.

## Recommendations for Further Work

- Trials should be carried out on a vessel with lower dynamics, representative of larger, commercial craft, such as a GLA buoy tender,.

- Any further trials should consider the possible benefits to the integrated solution of combining the outputs of a ship-borne heading reference system.

- This study has not attempted to draw any conclusions about the improved integrity that may derive from the integration of GNSS and INS.  A rigorous study of the performance of integrity monitoring algorithms for an integrated GNSS/INS solution should be carried out, possibly using data from the above proposed vessel trial.