AISM Association of Internationale de Signalisation Maritime  IALA

International Association of Marine Aids to Navigation and Lighthouse Authorities

IALA Recommendation A-126

On

The Use of the Automatic Identification System (AIS) in Marine Aids to Navigation

Edition 1.2

2007-01-19

20ter, rue Schnapper     -     78100     Saint Germain en Laye        -       France

Telephone +33 1 34 51 70 0 Telefax +33 1 34 51 82 05 Telex 695499 ialaism f

E-mail          :         iala-aism@wanadoo.fr      Internet      :      hjttp://iala-aism.org

Document Revisions

Revisions to the IALA Document are to be noted in the table prior to the issue of a revised document.

| Date | Page / Section Revised | Requirement for Revision |
| --- | --- | --- |
| 2007-xx-xx | All of document | Release of IEC 62320-2 and of ITU-R m.1371-3 |

IALA Recommendation on the use of the Automatic Identification Systems (AIS) in Marine Aids to Navigation Services.

(Recommendation A-126)

- THE COUNCIL

RECALLING that one of the aims of the Association is to foster safe, economic and efficient movement of vessels and the protection of the environment through the improvement and harmonisation of aids to navigation, vessel traffic services and other means world-wide.

NOTING Regulation V/19.2.4 of the 1974 SOLAS Convention, as amended, on the carriage of AIS equipment on board ships;

NOTING ALSO the studies carried out by IALA on shipborne identification systems have assisted in the development and adoption of:

- ITU Recommendation ITU-R M. 825 on the characteristics of a transponder system using DSC techniques for use with VTS and ship-to-ship identification;

- ITU Recommendation ITU-R M. 1371 on the Technical Characteristics for a Ship-borne Automatic Identification System (AIS) Using Time Division Multiple Access in the Maritime Mobile Band;

- IALA Technical Clarifications on ITU-R M.1371;

- IEC Standard 61993-2 Ed2: Class A Shipborne equipment of the Universal Automatic Identification System (AIS) - Operational and Performance requirements, methods of testing and required test results; and,

- IMO Recommendation on Performance Standards for a ship-borne Automatic Identification System (AIS), (MSC 74(69) Annex 3)

- IMO SN/Circular 217 on Interim Guidelines for the Presentation and Display of AIS Targets

- IEC 62320-1 AIS Base Stations – Minimum operational and performance requirements – methods of test and required test results

- IEC 62320-2 AIS AtoN stations - Minimum operational and performance requirements - methods of test and required test results

- IMO SN Circ.235 Guidance on the application of AIS binary messages

- IEC 62287 Maritime radionavigation and communication equipment and systems – Class B shipborne equipment of the Automatic Identification System (AIS) using CSTDMA techniques – Operation and performance requirements, method of test and required test results

NOTING FURTHER that IALA has adopted:

- Recommendation A-123 on the Provision of Shore Based Automatic Identification Systems (AIS);

- Recommendation A-124 on AIS Shore Stations and Networking Aspects Related to the AIS Service; and,

- Guidelines on the Universal Automatic Identification System (Volumes 1 and 2).

- The IALA Navguide, which includes a section on the use of AIS as an Aid to Navigation

RECOGNISING that the use of AIS in VTS operations will assist in the development and maintenance of a traffic image, particularly with respect to the:

- Identification of vessels;

- Tracking of vessels;

- Simplification of information exchange; and,

- Provision of additional information to assist in vessel traffic management.

RECOGNISING ALSO that an AIS transponder could provide information and data that could:

- Be used as an aid to navigation;

- Complement existing aids to navigation;

- Monitor the performance of aids to navigation;

- Monitor the “on station” position of floating aids to navigation;

- Provide identity, state of “health” and other navigational information such as meteorological and hydrographical data, if available, to ships and shore authorities; and

- Be used to assess traffic type and pattern to assist in providing the appropriate level of service and mix of aids to navigation.

HAVING CONSIDERED the various applications of AIS that have been identified by IMO, ITU, IEC and IALA;

HAVING DECIDED that, in addition to the transfer of data from ship to ship, ship-to shore and shore-to ship, as identified by IMO, the Automatic Identification System is defined as a system for use as:

- A marine Aid to Navigation;

- A tool to assist in VTS operations; and

- A tool to assist aids to navigation service providers.

RECOMMENDS that:

- National Members, and other appropriate Authorities providing marine aids to navigation services, use appropriate AIS units as part of their Marine Aid to Navigation Services for

  - the provision information and data to shipping, and

  - monitoring purposes.

- National Members, and other appropriate Authorities providing marine aids to navigation services, take into account the use of ‘AIS for Aids to Navigation’ set out in the annex to this recommendation.

*    *    *

Annex

AIS for Aids to Navigation

Table of Contents

THE COUNCIL	3

1	Background	2

2	Introduction	3

2.1	Synthetic and Virtual AIS AtoN	4

3	Technical standard for AIS AtoN units	4

4	Application of AIS to Aids to Navigation	5

5	Implementation 	7

5.1	Real AIS AtoN	8

5.2	Synthetic AIS	8

5.3	Virtual AIS AtoN	8

5.4	MMSI numbers for AIS AtoN	9

5.5	Reporting intervals for AIS AtoN messages	9

5.6	Transmission of Message 12 or Message 14, when the AIS AtoN station is repeating an AIS SART message.	10

5.7	AIS VDL channels for AIS AtoN messages – Reporting Modes	10

5.8	Configuration of Message 21, Aids to Navigation Report	11

6	References	23

- Background

Automatic Identification System (AIS) is an autonomous and continuous broadcast system, operating in the VHF maritime mobile band.  It exchanges information such as vessel identification, position, course, speed, etc. between mobile and fixed stations.  It handles multiple reports, using Time Division Multiple Access (TDMA) technology ensuring reliable and robust operation.

Chapter V of the 1974 SOLAS Convention (as amended) requires mandatory carriage of Automatic Identification System (AIS) equipment on all vessels constructed on or after 01 July 2002.  Implementation for other types and sizes of SOLAS Convention vessels was required to be completed not later than 31 December 2004.

AIS as applied to Aids to Navigation (AtoN) improves and enhance services provided to mariners.  The purpose of this document is to provide recommendations and guidance for the use of AIS in this field.

- Introduction

The International Association of Marine Aids to Navigation and Lighthouse Authorities (IALA) define an Aid to Navigation (AtoN) as:

“a device or system external to vessels that is designed and operated to enhance the safe and efficient navigation of vessels and/or vessel traffic”

An AIS AtoN station provides  positive identification and position of the AtoN.  In addition,the following enhanced services can be provided by an AIS AtoN station:

- Meteorological information

- Hydrological information

- Provides an accurate position referenced to WGS84 for fixed and floating AtoN

- For floating AtoN, provides an indication of on or off station

- Provide real-time information for performance monitoring of  AtoN systems, including light, racon, and power supply

ITU-R M.1371 defines the “Aids to Navigation Report” Message 21. AtoN AIS enables AtoN providers to broadcast information on the:

- Type of AtoN,

- Name of the AtoN,

- Position of the AtoN,

- Position accuracy indicator,

- Type of position fixing device,

- Time stamp,

- Dimension of the AtoN and reference positions,

- Status of the AtoN systems,

Synthetic and Virtual AIS AtoN

In some cases, for practical or economic reasons it may not be appropriate to fit an AIS AtoN station to an AtoN and the ‘Synthetic AIS AtoN’ approach may be taken.  A ‘Monitored’ Synthetic- AtoN AIS consists of a GNSS(Global Navigation Satellite System) receiver, processor, and a non-AIS data-link, for example packet radio or GSM (Global System for Mobile Communications).  The data is received ashore where it is converted into AIS format and re-broadcast by an AIS shore station or by an AIS AtoN station, as if it came from an AIS unit on the AtoN.  Where a Synthetic AIS AtoN approach is taken, sensors for meteorological and hydrological data may be provided, if required.  If the Synthetic AIS AtoN approach is applied to fixed AtoN, then the position monitoring may be omitted and the AIS AtoN would then be called a ‘Predicted’ Synthetic AIS AtoN.

It may be appropriate to create a ‘Virtual AIS AtoN’. In this case the target would appear at the Virtual AIS AtoN location, even though there is no physical AtoN.  Message 21 would clearly identify this as a Virtual AIS AtoN.  An example of where a Virtual AIS AtoN would be useful is the emergency marking of a new wreck, providing virtual indication until a physical buoy can be established.

Technical standard for AIS AtoN units

Technical standards for AIS AtoN are defined in IEC document IEC62320-2, AIS AtoN stations - Minimum operational and performance requirements - methods of test and required test results.

There are three main classifications of an AIS AtoN station, with different functionality. These are called Type 1, Type 2, and Type 3 and are fully described in IEC 62320-2.

## Type 1 AIS AtoN station

The Type 1 AIA AtoN station is a transmit only device, operating in FATDMA mode.

This is the simplest type of AIS AtoN station, and although it can be used at any AtoN station, its limited functionality and likely lower cost make it suitable for basic applications. It could also be suitable for temporary marking of hazards, such as floating wrecks and icebergs, or on an AtoN marking a sunken wreck.

The Type 1 station is dependent on an AIS Base Station in the vicinity for its slot reservations.

## Type 2 AIS AtoN station

The Type 2 AIS AtoN is similar to a Type 1, but has, in addition, an AIS receiver of limited capability operating on a single AIS channel. This receiver allows the Type 2 unit to be remotely configured and controlled via the AIS VDL.

The Type 2 station is dependent on an AIS Base Station in the vicinity for its slot reservations.

## Type 3 AIS AtoN station

The Type 3 AIS AtoN is a more complex device than the Type 1 and Type 2, and contains two AIS receiving processes that allow it to participate fully on the AIS VDL. This means that the Type 3 station can function in RATDMA mode and FATDMA mode.

The Type 3 station is therefore capable of:

- Autonomous operation independent of a Base Station

- Receiving and relaying AIS messages, including control and configuration messages for itself or for other AIS AtoN stations in a chain – refer to IEC 62320-2

- Relaying AIS messages related to GMDSS search and rescue

- Unlike Type 1 and Type 2, the Type 3 AIS AtoN station is capable of indirect synchronisation to UTC, using its receiving processes

- Application of AIS to Aids to Navigation

The application of AIS to AtoN is a significant use of the AIS, and ITU has defined AIS Message 21 for exclusive use with AtoN.  Other messages may be used with AtoN and are referenced in ITU-R M.1371 - Technical Characteristics for a Universal Automatic Identification System Using Time Division Multiple Access in the VHF Maritime Mobile Band (Reference 1).

In addition to Aids to Navigation Report, Message 21, an AIS AtoN may also transmit Messages 6, 8, 12 and 14.

Message 6

Message 6, Addressed Binary Message, can be employed by an AIS AtoN for sending AtoN status reports to the competent authority responsible for the AtoN. Examples of useful data are battery voltage trend data, lantern status data, and solar power system charging current. The benefits for the competent authority are knowledge of equipment status, opportunity for preventative maintenance, early notification of faults, and ultimately increased availability.  Such performance information can be fed back into the design process for AtoN systems.

Message 8

Message 8 is a broadcast binary message. IMO has published a limited list of Message 8 formats for international use. Competent authorities are free to use other Message 8 formats on a regional basis.

As an example, among the list of IMO Message 8 formats is Application 1 Meteorological and Hydrological  Data. Sensors on the AtoN could be providing this data to the AIS AtoN station, which in turn broadcasts this Message 8.

Message 12

An example of the use of a Message 12, Addressed Safety Related Message, by an AIS AtoN could be where a Type 3 AIS AtoN receives an AIS SART message, and relays this, as a Message 12, to the SAR authority.

Message 14

Message 14 is a Broadcast Safety Related Message. As with the Message 12 example, an AIS SART message could be re-broadcast by an AIS AtoN

A Type 3 AIS AtoN station may also transmit Messages 7 and 13.

The primary purpose of an AIS AtoN is to promote and enhance safety and efficiency of navigation by one or more of the following:

- Providing a positive and all-weather means of identifying an

- Complementing existing signals from AtoN;

- Transmitting accurate positions of floating AtoN (possibly corrected by DGNSS);

- Indicating if a floating AtoN is off station;

- Providing a complementary service to Racons;

- Marking or delineating tracks, routes, areas, and limits (for example areas to be avoided and Traffic Separation Schemes (TSS))

- Marking offshore structures (for example wind turbines, oil and gas platforms)

- Providing weather, tidal, and sea state data.

A further set of benefits for the AtoN provider include the following:

- Monitoring the status of an AtoN;

- Tracking an AtoN that is off position;

- Identifying ships involved in collisions with AtoN;

- Gathering real-time information on the ‘state of health’ of an AtoN;

- Remotely controlling changes in AtoN parameters.

AIS may be applied to both floating and fixed AtoNs, and more than one AIS message format may be transmitted as noted above.  The service provider has an obligation to verify the broadcast information and the correct operation of the AIS for AtoN.

Offshore structures are seen as hazardous objects.  Fixed offshore structures include offshore wind turbines and fixed drilling platforms.  AIS Message 21, ‘Aids-to-Navigation Report’ may be an appropriate message for reporting position of an offshore structure.  Other messages, such as Message 8, indicating, for example, an area to be avoided, could also be broadcast.

Implementation

An AIS AtoN can be implemented in three ways, Real, Synthetic, and Virtual.

  - Real AIS AtoN

A “Real AIS AtoN” is transmitted as a Message 21 from an AIS AtoN that is physically located on the AtoN.

  - Synthetic AIS

For practical or economic reasons it may not be appropriate to fit an AIS to an AtoN.  In this case, the ‘Synthetic’ AIS approach may be taken.  There are 2 types of Synthetic AIS AtoN, “Monitored Synthetic AIS AtoN” and “Predicted Synthetic AIS AtoN”.

    - “Monitored Synthetic AIS AtoN”

A “Monitored Synthetic AIS AtoN” is transmitted as a message 21 from an AIS Station that is located remotely from the AtoN.  The AtoN physically exists and there is a communication link between the AIS Station and the AtoN.  The communication between the Aid and AIS shall confirm the location and status of the AtoN.

    - “Predicted Synthetic AIS AtoN”

A “Predicted Synthetic AIS AtoN” is transmitted as a Message 21 from an AIS Station that is located remotely from the AtoN.  The AtoN physically exists but the AtoN is not monitored to confirm its location or status.

Only a Monitored Synthetic AIS AtoN ensures the integrity of the floating AtoN, therefore the use of a Predicted Synthetic AIS AtoN is not recommended for use on floating AtoN.  Predicted Synthetic AIS AtoN broadcasts for fixed AtoN is acceptable as the location will not change, but the status of the AtoN is not verified.

  - Virtual AIS AtoN

A “Virtual AIS AtoN” is transmitted as a Message 21 for an AtoN that does not physically exist.

It may be appropriate to create a virtual AtoN on a display, where no physical AtoN exists.  In this case the symbol would appear on the display for a specified location, even though there is no physical AtoN.  A nearby base station or AtoN station could broadcast this message. The flag in Message 21 would clearly identify this as a Virtual AIS AtoN.  An example of where Virtual AIS AtoN could be useful is the marking hazards to navigation on a temporary basis, until more permanent AtoN can be established.

It should noted that only vessels equipped with AIS will receive a Virtual AIS AtoN message.

  - MMSI numbers for AIS AtoN

    - MMSI numbers for all AIS AtoN

Message 21 must include an Maritime Mobile Station Identification (MMSI) number. All AIS AtoN MMSI numbers, as defined in ITU XXXXXX , are of format 99 followed by a three-digit MID followed by a four-digit unique identifier

    - MMSI numbers for Synthetic and Virtual AIS AtoN

Each Synthetic and Virtual AIS AtoN must have a unique MMSI number. The Repeat Indicator in Message 21 is used to identify that the message is broadcast from another location.

Reporting intervals for AIS AtoN messages

    - Message 21

The reporting interval for Message 21 should be chosen so that a vessel receives an appropriate number of Messages 21 from coming into range of the AIS AtoN broadcast until reaching the AIS AtoN location. Three is considered the minimum.

Factors to take into account are:

- Vessel speed of approach

- Topology, for examples vessels approaching from around a headland

- Nominal transmission range

Ideally a short reporting interval would be used, but in many cases the power consumption of the AIS AtoN will also influence the choice of reporting interval.

An example is of a high speed vessel approaching at 40 knots, from the open sea, and with a reception range of 20 nautical miles. In this case a reporting interval of 10 minutes would be sufficient to receive three messages.

A second example is of a vessel approaching from around a headland which masks the AIS signal, with the vessel doing 10 knots, and where the AIS AtoN appears at 2 miles range when the vessel clears the headland. In this case a reporting interval of 4 minutes would be sufficient to receive three messages.

    - Reporting intervals for other messages

Reporting intervals for other messages should be based on operational requirements. Two examples follow.

Message 6 for AtoN monitoring.

This message need only be sent as often as the competent authority requires that data. However in practice power consumption by the AIS AtoN will be minimised if  this message is sent with the same reporting interval as Message 21.  This is because most AIS AtoN devices will power down parts of their operating system between transmissions (“sleep mode”), and so sending Message 6 during the Message 21 wake-up portion of the sleep mode cycle does not add an extra wake-up period. Sending additional messages during the wake portion of the cycle has only a minimal effect on AIS AtoN device power consumption.

Message 8 for Meteorological and Hydrological data.

Again this should be coordinated with the wake-sleep cycle for Message 21. However by its nature, this message is required less frequently, so that a multiple of the Message 21 reporting interval would be appropriate. In situations where the Message 8 for Meteorological and Hydrological data is repeated by an AIS Base Station, the reporting interval at the AIS AtoN station might be reduced to 30 or 60 minutes, for example.

  - Transmission of Message 12 or Message 14, when the AIS AtoN station is repeating an AIS SART message.

These messages should be repeated each time they are received, on both channels.

  - AIS VDL channels for AIS AtoN messages – Reporting Modes

    - Reporting modes for Message 21

The Type 1 and Type 2 AIS AtoN stations may transmit on a single AIS channel, either Channel A or Channel B, or on both channels. The Type 3 AIS AtoN station must transmit on both channels.

Refer to IEC 62320-2, section 5.2.4.2.1 Reporting Modes for Message 21.

IALA recommends the use of Mode B for dual channel devices. Single channel devices must use Mode C.

    - Reporting modes for other messages

Reporting modes for other messages should be based on operational requirements.  Two examples follow.

Message 6 for AtoN monitoring.

This application is essentially point to point transfer of monitoring data, and so a single channel may be sufficient.

Message 8 for Meteorological and Hydrological data.

This application is likely to be intended for the benefit of shipping and so to increase the likelihood of reception, the use of Mode B from IEC 62320-2, section 5.2.4.2.1 Reporting Modes for Message 21, is recommended.

Configuration of Message 21, Aids to Navigation Report

Configuration of an AIS AtoN station, and of the essential Message 21, is described in IEC 62320-2.

Position monitoring for floating aids

- The position derived from an Electronic Position Fixing System (EPFS) can be used in conjunction with the  reference, or charted, position and a ‘guard ring’ to monitor the position of floating AtoNs and to generate an ‘off station’ alarm which sets the off-station bit in Message 21.

- IEC 62320-2 does not prescribe any specific algorithm for computing off position for the purpose of setting the off-position flag in Message 21. This algorithm is left to the AIS AtoN manufacturer to decide.

- When selecting an off-position algorithm, consideration should be given to spurious position fixes from the EPFS. A single spurious position fix from the EPFS should not set the off-position flag in Message 21.

    - Name of AtoN

- When assigning the “Name of AtoN” field in Message 21, the following priority should be used:

- Charted name;

- National or international identification number;

- Description of special characteristics. (This content is up to the service provider, but examples are light flash character, range in nautical miles, elevation in metres.)

    - AtoN dimension data in Message 21

The data content of Messgae 21, desribed in ITU-R M.1371, includes data bits for specifying the dimensions of an AtoN and the location of the GNSS receiving antenna on the AtoN structure. The value of this data is questionable in most practical cases, as the dimensions of most AtoN structures will be small compared with the passing distance of a vessel, and so they may be considered as points.

    - Message 21, Data bits reserved for regional or local application

The eight data bits “Reserved for regional or local application” were originally intended only for local applications at the discretion of the competent authority. These bits can still be used on a regional basis, however there is now an internationally agreed use of these bits defined by IEC 62320-2, for carrying basic data on the status of the AtoN systems. These bits can be used in “page format”, meaning that additional data functionality can be added in the future, regionally and internationally.

Refer to IEC 62320-2, part 5.2.1.1.1.

Controller

Power Supply

BIIT

AIS RX

AIS RX

AIS TX

## Autonomous continuous

AtoN Interface

e.g. IEC 61162

Internal Sync Source

DSC

Configuration

EPFS

- References

Reference 1

ITU, Technical Characteristics for a Universal Automatic Identification System Using Time Division Multiple Access in the VHF Maritime Mobile Band, ITU-R M.1371. (The most recent edition should be used.)

Reference 2

IALA, Technical Clarifications on Recommendations ITU-R M.1371, Edition 1.5 or later. (The most recent edition should be used.)

Reference 3

IEC 62320-2 AIS AtoN stations - Minimum operational and performance requirements - methods of test and required test results

TABLE 34

IALA NAVGUIDE, Edition 4, Chapter 3.

Synthetic AIS AtoN – the AtoN physically exists, but the AIS Station is located remotely from the AtoN.  Synthetic AIS AtoN may be ‘Monitored’ or ‘Predicted’.  Section 4.2 refers.

Virtual AIS AtoN – used to transmit a message 21 for an AtoN that does not physically exist.  Section 4.3 refers.

Note:  Only a Monitored Synthetic AIS AtoN ensures the integrity of the floating AtoN, therefore the use of a Predicted Synthetic AIS AtoN is not recommended for use on floating AtoN.  Predicted Synthetic AIS AtoN broadcasts for fixed AtoN is acceptable as the location will not change, but the status of the AtoN is not verified.