# Working Towards Draft IALA Recommendation e-Nav 0221

# On UMDM Objects

--- introduce formal IALA recommendation heading later ---

# Introduction

The Universal Maritime Data Model (UMDM) is an abstract representation of the maritime domain. Specifically, it represents the entities and relationships among the entities that exist in the maritime domain. The purpose of this document is to introduce the model scheme, define vocabulary and to give instructive examples of how to build model elements.

Entities are represented in the UMDM by Objects. Each Object consists of (or contains) one or more Attributes that represent the Properties of the Object. An Attribute can be another Object, or it can represent a single Property. The Property describes the Attribute in terms of the values that it can contain. Other Attributes may be used to further describe the Object.

Figure 1: (show as tree)

Object

One or more Attributes

Attribute

Property or Object

Property

value; range of values; resolution, operations

Some simple examples are used to introduce these concepts. The first example is a gift object that can be decomposed as follows:

Figure 2:

Gift						Object

Something Wonderful		Attribute

Box					Attribute

Ribbon					Attribute

Box						Object

Material				Attribute

Color					Attribute

Ribbon						Object

Style					Attribute

Color					Attribute

Material				Attribute

Something Wonderful: 			 Object

one of Gold, Silver, Diamond, Chocolate, Roses	Attribute

Material: 					Attribute

one of Paper, Wood, Steel (default Paper)	Value

Color:						Attribute

one of Blue, Green, Red		Value

Please note the “one of” notation. The “one of” notation is used to indicate an enumeration of values.

The next example introduces the concept of objects as extensions of other objects (as opposed to containing other objects). An extended object has all of the Attributes of the original object and can be used any place an original object can be used. An extended object can contain additional Attributes. This is also known as inheritance.

Figure 3:

Decorated Box: is kind of Box

Paint

Gift Box: is kind of Decorated Box

Name Tag

A Decorated Box is a Box object extended by adding the Paint Attribute. A Decorated Box thus contains these Attributes: Material, Color and Paint. A Gift Box further extends Box (by extending Decorated Box) by adding the Name Tag Attribute. The Gift Box then contains these Attributes: Material, Color, Paint and Name Tag.

Returning to the Gift example, since Decorated Box and Gift Box are both extended from the Box object, a Gift can contain either, a Box, a Decorated Box or a Gift Box. Please note that Gift can contain any of these new boxes without changing the definition of Gift. This shows the power of the extension mechanism.

For the next example, we first set up a simple set of objects:

Figure 4:

Mailable

Address

Postage

Letter: is kind of Mailable

... (additional Data Elements)

Delivery

Mailable

.... (additional Attributes)

Now, we introduce the concept of multiple object extension. With multiple object extension, the new object is extended from more than one original object.

Figure 5:

Sendable Box: is kind of Box: is kind of Mailable

... (additional Attributes)

The new Sendable Box object can be used in the Delivery example as a Mailable object, and in the Gift example as a Box object. This technique allows an object to be used in multiple contexts and can be used to introduce relationships between objects.

Need “List Of” example?

Need “Set Of” example?

Advanced discussion ideas (make an annex? A new chapter?):

Overriding of Attributes in extended objects

The following, Section 2, defines the terms we have used so far.

The remainder of this document contain examples of fully specified Attributes and Objects.

# Definition of Terms

Terms used to describe models:

Information 	– “knowledge concerning objects, such as facts, events, things, processes or ideas, including concepts, that within a certain context has a particular meaning” (ISO/PAS 16917:2002(E))

Data 	– “representation of facts, concepts or instructions in a formalized manner, suitable or communication, interpretation or processing by humans or by automatic means” (ISO/PAS 16917:2002(E))

Data model 	– “description of the organization of data in a manner that reflects an information structure” (ISO/PAS 16917:2002(E))

Entity 	– “any concrete or abstract thing of interest, including associations among things” (ISO/PAS 16917:2002(E))

Terms used to describe objects:

Object	– representation of an entity

Attribute	– a descriptive element of an Object, which could be a property of an Object or another Object; sometimes also called “feature”

Property	– representation of the value; includes value, value range, resolution, operations); sometimes also called “type”

Instance	– the instance of an object is a specific, individual and unique entity modelled by the object.

List of	– collection of objects

Is Kind of	– object extension; sometimes also called “inheritance”

Terms used for describing values:

Numeric	– a value with some arithmetic significance

Character String	– a textual value

Boolean	– value True or False

Date/Time	–

Enumeration	– values with pre-assigned meaning used exclusively (one of…)

Set	– values with pre-assigned meaning used in combination (zero or more of…)

need some work on operations.....

# General Purpose Properties and Objects

These are simple types and objects that can be used to construct many different objects.

## Time Related Properties

## Time-of-Day Property

This property is used to indicate the time of an event, such as for a position report or an estimated time of arrival.

| Name | Value range | Description |
| --- | --- | --- |
| Time-of-Day | 01 January 2000 – 31 December 2199; 0.001 second resolution | UTC date and time |

### Elapsed Time Property

| Name | Value range | Description |
| --- | --- | --- |
| Elapsed Time | 0 – 16 000 000 000 seconds; 0.001 second resolution | Greater than 200 years |

## Location Related Properties

### Longitude Property

| Latitude Property Name | Value range | Description |
| --- | --- | --- |
| Longitude | -180 – 180 degrees; 0.000 000 01 degree resolution | WGS-84 Ellipsoid; Negative values are West |

| Name | Value range | Description |
| --- | --- | --- |
| Latitude | -90 – 90 degrees; 0.000 000 01 degree resolution | WGS-84 Ellipsoid; Negative values are South |

### Altitude Property

| Name | Value range | Description |
| --- | --- | --- |
| Altitude | -11 000 – 30 000 meters; 0.01 meter resolution |  |

### Distance Property

| Name | Value range | Description |
| --- | --- | --- |
| Distance | 0 – 200 000 meters; 0.01 meter resolution |  |

## Position Related Properties and Objects

This property is used for reporting position. Latitude and longitude resolution has been chosen to to be better than 0.01 meter resolution.

### Basic 2D Position Object

Latitude Attribute		Latitude Property

Longitude Attribute		Longitude Property

### Position Source Property

| Name | Value range | Description |
| --- | --- | --- |
| Source | Enumeration: GNSS, DGNSS, eLoran, Chayka, Radar, RTK, Inertial, Estimated, Reported |  |

### Position Integrity Property

| Position Accuracy Property Name | Value range | Description |
| --- | --- | --- |
| Integrity | Enumeration: None, Good, Bad, Unknown | None = integrity not monitored; Good = integrity good; Bad = integrity bad; Unknown = not known if monitored |

| Name | Value range | Description |
| --- | --- | --- |
| Position Accuracy | 0 – 1 000 meters; 0.01 meter resolution | Standard deviation of position error |

### General Position Object: Is Kind of Basic 2D Position Object

Altitude Attribute			Altitude Property

Time Stamp Attribute			Time-of-day Property

Position Source Attribute		Position Source Property

Integrity Attribute			Position Source Property

Accuracy Attribute			Position Accuracy Property

### Identifier Property

### COG Simple Property

### SOG Simple Property

### Track State Object

Identifier Attribute

Time Stamp Attribute			Time-of-day Simple Property

Position Attribute			General Position Object

COG Attribute			COG Property

SOG Attribute				SOG Property

# Properties and Objects for Aids to Navigation

## AtoN Object

### AtoN Object

Position Attribute			General Position Object

List of AtoN Devices Attribute	List of Basic AtoN Device Objects

### AtoN Type Property

### Basic AtoN Device Object

AtoN Type Attribute			AtoN Type Property

## Light Device Object

### Light Color Property

### Light Characteristic Property

### Light Object: is kind of Basic AtoN Device Object

Color Attribute			Light Color Property

Characteristic Attribute		Light Characteristic Property

Height Attribute			Altitude Property

Range Attribute			Distance Property

## Buoy Device Object

### Buoy Type Property

### Buoy Top Mark Property

### Normal Alarm Property

### Buoy Object: is kind of Basic AtoN Device Object

Type Attribute			Buoy Type Property

Height Attribute			Altitude Property

Top Mark Attribute			Buoy Top Mark Property

Swing Attribute			Distance Property

On-position Status Attribute		Normal Alarm Property

# Rules for maintaining data type definition

--- here should the rules be included which govern the amendment of the data types

--- note: particular care for legacy of “old data types”

--- refer to the Registry - UMOR