| From: e-NAV committee |  |
| --- | --- |
| To: PAP | 13 March 2009 |

Liaison Note

- AIS coverage in relation to user requirements of LRIT

- Introduction

In response to the following from the minutes of PAP 17:

“…

The Members of the European Union provided information on the development of a European Master Plan for the fixed-based AIS networks, and expressed their concerns as regards further work needed by IMO to agree procedures to define the coverage of fixed-based AIS networks on the basis of the real coverage and not related to the definition of sea area A1.  Several delegations supported that consideration from the European Union that in order to avoid costly installations on ships as well as unnecessary costs to States to buy LRIT information from communication services providers, the AIS system should be used as much as possible instead of the LRIT system.  Due to lack of time, decisions on the matter were reported to MSC 85.

…

Action Item:

The AIS TWG is requested to consider and then report on AIS coverage in relation to the user requirements of LRIT.”

No firm definition of AIS coverage exist, however the WG2 and WG3 of the IALA e-NAV committee have considered the matter and responds as follows.

- User requirements of LRIT

The LRIT performance standards require tracking data from SOLAS vessels, which in terms of content can be substituted with AIS data.

The nominal reporting rate of LRIT is one position report every 6 hours, however Administrations can poll vessels or request an increased update rate up to one position report every 15 minutes.

The minimum AIS reporting rate (when moored) is one positionreport every 3 minutes, resulting in a worst case of 5 position reports being transmitted within every 15 minute period.

Thus reception of 20% of all transmitted AIS position reports will correspond to the maximum reporting rate required by LRIT in the worst case.

Moving vessels will provide a much higher update rate, typically every 6-30 seconds resulting in 30-150 reports within a 15 minute period. As little as 3 % reception of expected AIS reports would thus be sufficient for moving vessels.

- AIS coverage

Coverage at VHF frequencies, and AIS in particular, will be affected by:

- Antenna system

- Propagation path

- Weather

- Sea state

- VHF datalink loading

It is possible to use a propagation modelling tool based on Recommendation ITU-R P.1546-3 to estimate the range performance of AIS, however this tool does not take into account all the factors listed, yet real life coverage may still not be predicted precisely, why validation is needed.

AIS transmission rates vary depending on the type and speed of the transmitting platform. It is not necessary to receive all transmissions to compile a valid record of vessel activity, and reception rates of perhaps 50% or 20% may suffice in some circumstances.

AIS may transmit different types of message e.g. addressed binary messages, depending on the application. The performance of different message types may vary e.g. if an acknowledgement is required.

IALA guidelines on AIS, Edition 1.1, Volume 1, Part II, section 16.1, refers to the basic principles of RF coverage based on the Sea Area A1 definition for VHF, reused to a rough estimate of AIS radio coverage, based on the following formula, assuming omnidirectional antennas:

The radius A is equal to the transmission distance between a ship’s VHF antenna at a height (h) above sea level and the height (H) of the antenna of the VHF coast station which lies at the centre of the circle.

The following formula should be used to calculate the range A in nautical miles

A=2.5(Square root of H (in metres) + Square root of h (in metres))

The A1 area definition assumes vessel antenna heights of 4 meters, however several administrations have argued that an antenna height of 10 meters is more relevant for SOLAS vessels, and several studies have indicated that actual AIS coverage in terms of position tracking range is far greater than the predictions of the formula above.

In a presentation at eNAV6 by AMSA, field trials indicate documented reception of 10-20% of AIS transmissions up to ranges of a factor of 1.5 times the coverage ranges expected by the formula above.

These figures are consistent with observations from DaMSA, who observed consistent reception at ranges between 1.3 and 2.0 times the expected ranges derived by the formula above.  (In one instance, observed range was only 0.7 times the expected value. This turned out to be the result of a previously undetected antenna cable fault.)

Results of AMSA study:

Observations clearly indicate, that real life coverage may be significantly different from the estimated coverage, and will change as a function of a lot of factors, including sea state and the current channel loading of the AIS frequencies in the area in question. Since positions reports from vessels within coverage are received continuously, from which the ranges to a particular base station can be derived, and since the expected reporting rate of vessels can be derived from a firm rule-base, AIS data itself can in principle be used to compute a validated coverage, compared to a defined criteria.

An example of a definition for validation is given:

‘The Helcom AIS service area is the area from which at least [50] % of the transmitted positions reports (measured during a [6] minute period) from a ship with an antenna height of [>4] m above sea level and an AIS installation in accordance with IMO guidelines, can be successfully received during [98] % of the time (taking weather, sea and VHF propagation conditions into consideration)’

- Proposed definitions of coverage

Based on the discussion above, the Technical working groups suggest 2 definitions of AIS coverage – one for planning purposes, and one for the purpose of validating coverage.

Validated coverage should be the basis for declaring a service area.

Roughly estimated (planned) coverage

Assuming omnidirectional AIS Shore Station antennas, the radius A is equal to the transmission distance between a ship’s AIS antenna at a height (h) above sea level and the height (H) of the antenna of the AIS shore station which lies at the centre of the circle. The following formula should be used to calculate the expected AIS coverage range A in nautical miles:

A = Ux * ( Square root of H (in metres) + Square root of h (in metres) )

Ux denotes a factor derived by experience, which is determined by the Usage criteria for which coverage is being estimated. The coverage criteria for VTS operations is somewhat different than the coverage criteria for LRIT applications.  (Different Ux values may be applicable if estimating coverage for low power devices like the AIS class B or SART devices.)

Further studies are needed to determine the Ux factor for various applications, however the observations above suggest the following:

| Application | Ux | Expected % position reports detected | H (meters) | h (meters) | Expected range (Nautical miles) |
| --- | --- | --- | --- | --- | --- |
| LRIT | 3.75 | >20% (class A) | 50 100 | 10 | 38 49 |
| VTS | 2.5 | >80% (class A) | 50 100 | 10 | 26 33 |

Validated coverage

Given the availability of data from an AIS Base Station, or an AIS network as a whole, data can be analyzed to generate documented coverage maps. (Should coverage for class B or AIS SART be considered, separate maps should be created.)

By continuously calculating the interval between position reports compared to expected reporting interval based on vessel speed and rate of turn, and mapping the observed reception percentages to the current position of the vessel into a map covering the area in question, will generate a documented coverage map.

Averaging the percentage of reception for all relevant vessels passing through a given area, can determine if AIS coverage for that area meets a given criteria or not.

Such criteria would again depend on the intended application – relevant criteria could be:

LRIT:  >20% reception (average)

VTS: >80% reception (average)

…

One obvious drawback to this last method would be, that areas without any ship traffic will turn out as having no (documented) coverage, however combining the rough estimation technique with documented coverage analysis will give a very clear picture of reliable coverage.

Further work is needed, for such a definition is to be operational, and should be tested.

- Related developments

The EU is supporting a regional project (the ‘EfficienSea’ project) amongst 17 partner institutions in the Baltic Sea region. This project does contain a related task regarding the development of AIS network planning and coordination tools for use within the EU.

The Baltic Sea area may constitute a suitable trial area for evaluating such coverage definitions, in relation to LRIT applications. IALA PAP may consider liaising with EMSA or the lead partner of the EfficienSea project (DaMSA), and suggest the evaluation of the proposed coverage definitions in this (or similar) project(s).