e-Navigation Committee

6th Session										9 March 2009-02-19

This note gives an update on work by the General Lighthouse Authorities, Research & Radio Navigation Directorate regarding Virtual Aids to Navigation and is provided for the information of the e-Navigation Committee.

Development of Virtual AtoN Systems

1.	Introduction

The purpose of the Virtual AtoN System is to provide a near-instantaneous warning to the mariner of a new danger, such as a wreck, obstruction or floating debris. This warning must be provided in a form that can be received, interpreted and displayed by any class of vessel in the required operational area and carrying appropriate equipment.

# 2.	Importance of VAtoNs

The ability to provide Virtual AtoNs could be one of the most significant technical developments for the GLA (and other maritime administrations) in the medium term. This development has become possible through the deployment of a network of AIS base stations around the coast and the installation of onboard equipment with the capability of displaying the Virtual AtoN symbols.

The main application of Virtual AtoNs in the short term would be warning of new hazards, such as wrecks, obstructions or floating debris. However, Virtual AtoNs could have other applications in the longer term, such as “cats-eyes” marking the centre of “motorways of the sea” .

# 3.	Informing the User

Informing the user about the deployment and removal of Virtual AtoNs will be important for them to be fully effective. However, informing the user if the Virtual AtoN has been discontinued before the removal of the wreck or obstacle, whether by intention or because of malfunction, will be crucial to safe navigation. The method of promulgating this information and the time taken to present it to the user should be the subject of a thorough risk analysis.

4.	Design Approach

A Systems Engineering approach is being used in preparing the outline design and architecture for a GLA Virtual AtoN System. This involves assessing available information; requirements analysis; functional analysis; defining measures of effectiveness; identifying the essential elements and generating a model. A risk analysis will be performed using FMECA techniques.

The requirements will be reviewed and refined in an iterative process, involving formal requirements capture, including a workshop and trials. Since it is recognised that the architecture may change as a result of the detailed requirements analysis and trials, a schematic model has been generated at this stage and used to produce a data flow diagram. In particular, the mode of displaying Virtual AtoNs (symbology, graphical imagery) will be a matter for further investigation. In this way the model will be optimised and a plan will then be finalised for realising the model.

# 5.	Criteria

The criteria for rapidly marking wrecks and other new dangers relate to an assessment of the danger posed to shipping in the area. The criteria for replacement of existing AtoN with virtual AtoN will involve an assessment of the navigational value of the physical characteristics. Typical locations would include deepwater AtoN intended only for Safety –Of –Life –At-Sea Convention (SOLAS) vessels. Possible future provision for new vessel types such as Wing In Ground (WIG) craft arise from the need to mark a channel without having the physical obstruction of an AtoN. However, it remains to be seen whether there would be a long-term requirement for virtual AtoNs in these applications, once the electronic navigation chart had been updated.

The GLA have classified their areas of responsibility according to times to respond to wrecks and other incidents. These times range from 6 hours in the busiest areas (e.g. Dover Straits) to 36 hours in some western parts. Assuming a close correlation between shipping density and probability of collisions and wrecks, the same classification areas might be applicable to the provision of supporting infrastructure for Virtual AtoNs.

## 6.	Technical Constraints

Display equipment installed onboard ships at present, including ECDIS and radar, does not generally have the capability of displaying virtual AtoNs. However, new IEC specifications (IEC 62288 & 62388) for Navigation Displays and Radar do include a symbol for AIS AtoNs, a diamond centred on the position, with a V inside it to indicate Virtual AtoNs.

Another technical matter to be resolved is the availability of a transmission network. In the UK the organisation responsible for AIS is the Maritime and Coastguard Agency (MCA) and in Ireland the Irish Republic Coast Guard (IRCG). Negotiations have been started with the MCA on a Service Level Agreement for the AIS Data feed and on the procedures for providing Virtual AtoNs on behalf of the GLA  If this proves too difficult, then alternatives, such as expanding the GLA network, or providing transportable base stations may be considered.

### 7.	Outline Design

## 8.	Optimisation

Full requirements analysis, with consultation of users and service providers, will be needed to refine and optimise the design. This will probably be carried out in the form of a workshop, during which opinions will be sought on how the design concept could be improved, giving particular attention to the mode of display. This might best be achieved using a simulator to present a number of alternatives. Following this optimisation process, trials will be carried out with the preferred option to ensure that it gives the expected performance.

## 9.	Risk Analysis

A risk analysis of the final proposed system will be performed using Failure Mode Effect and Criticality Analysis (FMECA), drawing on the previous FMEA/Fault Tree Analysis of AIS carried out for the GLA.

The analysis will consider risks involved in the essential elements of the system identified, both individually and as a complete system.

## 10.	Conclusions

The iterative, systems engineering approach should produce the optimum design for a GLA Virtual AtoN System.

The final design and plan for its implementation should be completed by the end of 2009, subject to the technical constraints mentioned above.